3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005864

Elbonia, August 19 – September 1, 2020

Source:	Orange

Title:	KI #6, New Sol:  Replacement of the UAV Controller of a UAS

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract of the contribution: This paper proposes a solution for the Replacement of the UAV Controller of a UAS.

# 1 Introduction

The solution gives the possibility to substitute the current UAV controller by another one. Such UAVC substitution may be of particular interest for future drone use cases, for examples:

- The UAV is under the control of an automated controller but some event (ex: anomaly detection, geofencing boundaries crossing) calls for manual control.

- The UAV is under the control of a given UAVC, but law enforcement, or any authorized third-party, wishes to take control of that drone e.g. to modify the drone trajectory.

In the proposed solution, the new UAVC is associated with the same USS.

# 2 Proposal

It is proposed to update TR 23.754 as follows.

* * * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 | X | X | X | X | X | X | X |
| 3 |  | X |  |  |  |  |  |
| 4 | X | X | X |  |  |  | X |
| 5 | X | X | X |  |  |  | X |
| 6 | X | X |  |  | X |  |  |
| 7 | X | X |  |  | X |  |  |
| 8 | X |  |  |  |  |  |  |
| 9 | X |  |  | X |  |  |  |
| 10 |  | X |  |  |  |  |  |
| 11 |  | X |  |  |  |  |  |
| 12 |  | X |  |  |  |  |  |
| 13 |  |  |  | X |  |  | X |
| 14 |  |  |  | X |  |  |  |
| 15 |  |  |  | X |  |  |  |
| 16 |  |  |  | X | X |  |  |
| 17 |  | X |  |  | X |  |  |
| 18 |  |  |  |  | X |  |  |
| 19 |  |  |  |  | X |  |  |
| 20 |  |  |  |  |  | X |  |
| 21 |  |  |  |  |  | X | X |
| X |  |  | X |  |  | X |  |

* * * * Next Change (all new text) * * * *

## 6.X	Solution #X: Replacement of the UAV Controller of a UAS

### 6.X.1	Introduction

This solution addresses the requirement "An UAV Controller can be removed from a UAS and replaced with another UAV Controller or a TPAE" (see clause 4.2), as part of Key Issues 3 and 6.

This solution assumes that the UAV is already registered and authorized within the 3GPP systems, as well as within the CAA/UTM system, and that this UAV is already communicating with UAV Controller 1, associated with a USS. UAV Controller 1 is replaced by UAV Controller 2, which is associated with the same USS.

As for Solution 5, a UAV Flight Enablement Subsystem (UFES) may be used in the 3GPP system, to provide a single interface to the USS, thus limiting the impacts to the 3GPP system.

### 6.X.2	Procedures

#### 6.X.2.1	5GS Procedure

Figure 6.X.2.1-1: Procedure for replacing the UAV Controller of a UAS

- The UAV is communicating with UAV Controller 1.

- The USS sends a UAV Operation Update to the SMF (via UFES if applicable), containing new authorized UAV and UAV Controller pairing information.

- The SMF configures user plane connectivity for UAV to UAV Controller 2 communication based on information provided by the USS (e.g. policies and/or traffic filters) to enable user plane connectivity between the UAV and the UAV Controller 2 and disable user plane connectivity between the UAV and the UAV Controller 1.

- The UAV communicates with UAV Controller 2.

#### 6.X.2.2	EPS Procedure

Figure 6.X.2.2-1: Procedure for replacing the UAV Controller of a UAS

- The UAV is communicating with UAV Controller 1.

- The USS sends a UAV Operation Update to the P-GW (via UFES if applicable), containing new authorized UAV and UAV Controller pairing information.

- The P-GW configures user plane connectivity for UAV to UAV Controller 2 communication based on information provided by the USS (e.g. policies and/or traffic filters) to enable user plane connectivity between the UAV and the UAV Controller 2 and disable user plane connectivity between the UAV and the UAV Controller 1.

- The UAV communicates with UAV Controller 2.

### 6.X.4	Impacts on services, entities and interfaces

For 5GS: an "update" operation is added to the SMF service that receives the authorisation data from USS.

For EPS: the reference point between the P-GW and the UFES or USS for authorisation data is extended to allow update of the authorisation data.

* * * * End of Changes * * * *