3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2006255

Elbonia, August 19 – September 1, 2020	(revision of S2-2005341)

Source:	Huawei, HiSilicon

Title:	KI#8, Sol#29: Update to add an Application Status for the application data from UE

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update the Solution#29 for KI#8 to add an Application Status for the application data from UE.

1. Introduction

Multiple applications, which are classified into foreground (FG) and background (BG), could be run in a UE at the same time. FG and BG applications can be managed differently to efﬁciently maintain the service experience in case of network congestion.

-	For FG application, a UE sends a request to the application server or other UEs and then, they immediately respond to the UE. When network congestion occurs on Foreground application, the user is likely to notice a delay since Foreground application is generated by the user interaction.

-	For BG application, a UE periodically communicates with application servers in order to exchange or update information. Even when the network congestion occurs, the user of the UE is unlikely to notice the delay of the communications.

However, it is difﬁcult for the mobile operators to distinguish those types of Application Status only by IP addresses/port numbers.

So it is proposed that the NWDAF could collect this Application Status from UE during the training phase, then a Application Status Classifier could be trained to help determine if a new flow of an application is in FG or BG status during the inference phase. Finally, the FG or BG status for the new flow could be exposure to the 5GC NF (e.g. PCF) to determine a new QoS parameters for the new flow.

During the SA2#139E meeting, the Soluion#29 for the KI#8 is agreed to support the Application data collection from NEF/AF to NWDAF, however, it does not indicate which status (foreground or background) of the Application data. So this contribution proposes to update the Solution#29 for KI#8 to add an Application Status for the application data from UE.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91 v0.4.0.

# * * * * First change * * * *

6.29	Solution #29: Support UE data as an input for Control Plane load balancing analytics

6.29.1	Description

This solution is proposed for Key Issue 8: UE data as an input for analytics generation.

UEs are natural data collection points to gather more localised input data to generate analytics. This is enhanced by logging sensor data, including UE orientation, altitude, location, speed and heading as also captured in RAN WG studies in Rel-16 (TR 37.816 [13]).

When it comes to social behaviour and social gatherings like sport matches, concerts, operas festivals etc., several UEs may adopt tightly correlated behaviours that we refer here to as "collective behaviour" without any explicit group designation. As an example, such UEs may adopt similar route, velocity, orientation, mobility pattern or relative proximity as a collective behaviour within a joint event (e.g. to reach a destination spot). As a result, collecting input parameters from UEs that reflect user intent or indicate likely actions that such UEs may take can be useful to generate analytics data within the 5GC at NWDAF.

As an example, based on such input data (indicating a collective user behaviour), NWDAF may generate analytics to identify expected changes in load per tracking area (TA) or AMF region via identifying correlation in collective behaviour of the indicated non-designated group. Accordingly, OAM (or other related consumer NFs) may utilise the load analytics (if applicable) to adjust the weight factor per AMF instance within an AMF set (under Operator policy) based on the above collective behaviour-derived analytics (within a given AMF region or TA as defined by TS 23.502 [3]) to avoid future overloads / congestions.

NOTE:	Whether and how OAM may use NWDAF analytics needs to be determined by SA5.

Another example, multiple applications, which are classified into foreground (FG) and background (BG), could be run in a UE at the same time. The FG application means the application status in active mode and the BG application means the application status in in active mode. The Applications status i.e. active or inactive can be managed differently to efﬁciently guarantee the service experience in case of network congestion.

-	For application status in active mode, a UE sends a request to the application server or other UEs and then, they immediately respond to the UE. When network congestion occurs on Foreground application, the user is likely to notice a delay since Foreground application is generated by the user interaction.

-	For application status in inactive mode, a UE periodically communicates with application servers in order to exchange or update information. Even when the network congestion occurs, the user of the UE is unlikely to notice the delay of the communications.

However, it is difﬁcult for the mobile operators to distinguish those types of Application Status only by IP addresses/port numbers.

Therefore, the NWDAF could collect this Application Status from UE during the training phase, and then an Application Status Classifier could be trained to help determine if a new flow of an application is in FG or BG status during the inference phase. Finally, the FG or BG status for the new flow could be exposure to the 5GC NF (e.g. PCF) to determine new QoS parameters for the new flow.

6.29.2	Input Data

If the application and/ or MDT does not have the requested parameters, AF may ask UE to collect (raw) data and finally signal the processed application related data parameters to NWDAF.

Table 6.29.2-1: UE input data (via AF after anonymization, statistical analysis or normalisation)

| Information | Source | Description |
| --- | --- | --- |
| List of UE IDs | NEF/AF | List of external UE IDs identified with a collective behaviour within area of interest |
| Collective Attribute, Time-Stamped | UE (via AF/MDT) (NOTE 1, 2) |  |
| > Application ID | NEF/AF | Identifying the application providing this information |
| > Timestamp | UE/AF | Identifying a timestamp for the application |
| > IP 5-tuple | UE/AF | Identify a flow of the application |
| > Application Status | UE/AF | Indicate the flow of the application is in a foreground status (i.e. active status) or background status (i.e. inactive status) |
| NOTE 1: UE input data collection for a specific UE (via MDT), can be done as captured in 6.2.3.1, TS 23.288 [5]. NOTE 2: UE input data collection from AF for known parameters can be done based on clause 6.2.2.3, TS 23.288 [5]. NOTE 3: The Application Status could help NWDAF to train an Application Status Classifier, which could be used to classify a new application data into foreground status or background status. |  |  |

Editor's note:	The full list of UE input data is FFS to be aligned with the method of data collection from UE as subject of other solutions for this KI (KI#8).

6.29.3	Output Analytics

Many network analytics could be derived by the NWDAF by using the input data from UE.

For example, Application Status statistics information is defined in Table 6.29.3-1.

Table 6.29.3-1: Application Status statistics

| Information | Description |
| --- | --- |
| UE ID | UE ID e.g. SUPI or IMEI for a UE which is indicated in the Target of Analytics Reporting |
| Application Status (1..max) | List of predicted application status |
| > Application ID | Application ID for an application of the UE |
| > Timestamp | The timestamp for the application |
| > Application Status | The application status (e.g. background i.e. inactive or foreground i.e. active) for the application |

Application Status predictions information is defined in Table 6.29.3-2.

Table 6.29.3-2: Application Status predictions

| Information | Description |
| --- | --- |
| UE ID | UE ID e.g. SUPI or IMEI for a UE which is indicated in the Target of Analytics Reporting |
| Application Status (1..max) | List of predicted application status |
| > Application ID | Application ID for an application of the UE |
| > Timestamp | The timestamp for the application |
| > Application Status | The application status (e.g. background i.e. inactive or foreground i.e. active) for the application |
| > Confidence | Confidence of this prediction |

6.29.4	Procedures

Figure 6.29.4-1: Procedure to collect UE data for Control Plane load balancing analytics

1.	The Consumer NF sends a request to the NWDAF for NF load analytics. The Analytics ID is set to "NF load information" with analytics filter information based on an area of interest (e.g. to cover specific TAs or AMF regions). The consumer NF may set a time window in future to get predictions on pattern of load changes within that window.

2.	NWDAF follows the UE Input Data Collection Procedure for all SUPIs retrieved (e.g. based on input data in clause 6.29.2) within the area of interest (via setting the analytics filter information to specific TA or AMF region as requested at step 1) and collects the UE input data accordingly.

3.	The NWDAF may optionally retrieve the NF load and NF status information from NRF for corresponding NF instances.

4.	NWDAF processes the collected input data (e.g. to identify UEs with collective behaviour) and (optionally) combines with current NF load analytics (e.g. per AMF instance) retrieved from NRF.

5.	NWDAF provides predicted pattern of load changes (e.g. across TAs or AMF regions) to the Consumer NF.

6-8.	If, at step 1, the Consumer NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them to the Consumer NF upon reception of notification of new UE input data.

Editor's note:	The method of UE Input Data Collection in Step 2 or UE Input Data notification in Step 6 is FFS and subject of other Solutions for this KI (KI#8).

6.29.5	Impacts on services, entities and interfaces

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *