3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2006298

Elbonia, August 19 – September 01, 2020	(revision of S2-2005397r01)

Source:	Huawei, HiSilicon

Title:	KI#1: Update of Sol. 3: resolving open issue(s) for UE leaves MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: There are is an EN in solution#3 related to UE leaves multicast service, this document focus on addressing this EN.

# 1. Introduction

In section 6.3.2.3, there is an ENs shown as below:

Editor's note1. Option A and B are only introduced temporarily, and a single solution should be selected, Solutions for joining and leaving should be aligned.

Same handling as the Session joining procedure, the leaving procedure can be updated accordingly. The detail change is in step 18~ 24 of clause 6.3.2.3.

Also to align the termninology defined in architecture paper, i.e. S2-2005411, the NF used in the flow are also updated.

# 2. Proposal

It is proposed to capture the following changes in TR 23.757.

# * * * * First change * * * *

#### 6.3.2.3	UE Leaves Multicast Service via PDU Session Modification Procedure

Figure 6.3.2.3-1: PDU session modification for multicast leave

1.	The UE joins one or multiple multicast services as specified in clause 6.3.2.1.

At any time the UE can determine to leave the multicast service.

Alternative 1: user plane signalling:

2.	The UE sends the user plane message (e.g., the IGMP Leave) when the UE want to leave one or multiple multicast services. The user plane message shall include the related information about the multicast service to be leaving, such as the multicast address.

3.	The reception of the leave message triggers the UPF to notify the SMF. The SMF initiates PDU session modification procedure upon the reception of the notification from the UPF.

Alternative 2: control plane signalling:

4.	The UE sends the PDU Session Modification Request when the UE wants to leave one or multiple multicast services. The PDU Session Modification Request shall include the related information about the multicast service to be leaving, such as the multicast address.

5.	The AMF invokes Nsmf_PDUSession_UpdateSMContext (SM Context ID, N1 SM container (PDU Session Modification Request with the associated multicast service information (e.g., leave indication, multicast service identity, etc.))).

If the UE is receiving multicast via unicast PDU session steps 6 to 14 apply:

If SMF and MB-SMF are different and the multicast data are not needed to be distributed via unicast distribution within a PDU session to other UEs served by UPF steps 6 to 8 apply, i.e. the shared tunnel between the UPF and MB-UPF is not needed:

6.	The SMF sends a request to terminate the multicast distribution [Multicast context/group ID] to MB-SMF.

7.	Based on the information received in step 6, MB-SMF updates the multicast session context identified by the Multicast context/group ID and configures the MB-UPF to no longer distribute multicast data towards UPF.

8.	The MB-SMF acknowledges the request to terminate the multicast distribution to SMF as the response to step 6,

9.	SMF reconfigures UPF to terminate the distribution of multicast data via the unicast PDU session and when steps 6 to 8 were executed also to release the resources for the reception of the multicast data.

The SMF update the UE with the result of the leave handling. In addition if dedicated QoS flow are used for the unicast transfer of the multicast data, the SMF also update the RAN to remove the multicast QoS flow related information, i.e. the mapped unicast QoS flow information, from the associated unicated PDU Session.

10.	The SMF request the AMF to notify the RAN node to release the QoS flows previously used to transport the mulicast data using the Namf_Communication_N1N2Message (N2 SM information) Transfer service. In the N2 SM information, it includes unicast QoS flow information.

11.	The session modification request is sent to the RAN. The N1 SM container (PDU Session Modification Command) is provided to the UE.

12.	The RAN performs the necessary radio resource modification.

13.	The RAN sends the session modification response to AMF.

14.	The AMF transfers the session modification response received in step 13 to the SMF via the Nsmf_PDUSession_UpdateSMContext service

If the UE is receiving multicast via multicast distribution steps 15 to 25 apply:

15.	The SMF request the AMF to notify the RAN node that the UE left the indicated multicast group using the Namf_Communication_N1N2Message (N1SM container (PDU Session Modification Command (PDU Session ID , multicast information ([Multicast Context ID], multicast address,)), N2 SM information) Transfer service.

In the N2 SM information, it includes the multicast flow information (multicast QoS Flow ID and associated QoS information), and multicast service identity UE want to leave.

NOTE:	If mapped unicast QoS flow information, association between the unicast QoS flow and multicast QoS flow, and Unicast information (i.e., QoS rules for Unicast flows) in the N1 SM container are added for multicast distribution, then this information also needs to be deleted at this stage.

16.	The session modification request is sent to the RAN. The request include a multicast service identity and multicast flow information. The N1 SM container (PDU Session Modification Command) is provided to the UE.

The RAN use the multicast service identity to remove the UE from the multicast session context. Also in the UE context the related multicast QoS flow and associated unicast QoS flow information are removed.

17.	The RAN performs the necessary radio resource modification.

If the UE is the last one to leave the indicated multicast service, the RAN release the associated shared downlink tunnel between RAN and MB-UPF.and steps 18b to 22 apply.

18.	RAN node selects the AMF to reach MB-SMF and signals a request for the Multicast user plane distribution release towards that AMF [MB-SMF ID, Multicast context/group ID, downlink tunnel information].

19.	AMF forwards the request towards the MB-SMF

20.	For unicast transport of the multicast distribution session, MB-SMF updates the multicast session context identified by the multicast context identity and request the MB-UPF release the corresponding shard downlink tunnel resource.

21.	SMF sends a multicast distribution session release response to AMF.

22.	AMF forwards multicast distribution session release response to RAN node

23. For the multicast transport of the multicast distribution session, after receive the multicast distribution release response, the RAN sends the leave message (e.g., MLD/IGMP Leave ) to MB-UPF to stop the MBS data transmission towards this RAN node.

24.	The RAN sends the session modification response to AMF.

25.	The AMF transfers the session modification response received in step 24 to the SMF via the Nsmf_PDUSession_UpdateSMContext service.

# * * * * End of changes * * * *