SA WG2 Meeting #140E (e-meeting)	S2-2004823r04

Elbonia, 19 August-1 September 2020	(was S2-200xxxx)

Source:	Qualcomm Incorporated, Ericsson

Title:	KI#4, Interim Conclusions

Document for:	Discussion/Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN/Rel.17

Abstract of the contribution: Analyses the similarities and difference identified in the solutions documented in TR 23.700-07 for Key Issue #4 and proposes interim conclusions.

# Categorisation of solutions of KI#4

Key Issue #4 as it has been modified in SA2#139E contains two distinct components (component 1: UE onboarding and component 2: remote provisioning) and applies to both SNPN and PNI-NPN. There are 17 solutions documented in TR 23.700-07 for Key Issue #4 and can be categorised as follows as applying both these two components for SNPN or only to the remote provisioning component for PNI-NPN. One solution (solution 29) is not categorised since it refers to a separate “standalone” topic i.e. indications provided from onboarding SNPN.

- For SNPN

| Solutions | UE Onboarding using Default UE credentials in Onboarding SNPN (O-SNPN) (NOTE) | UE Onboarding using PLMN credentials and PLMN as ON | UE Onboarding with no credentials for primary authentication | Remote Provisioning (CP) | Remote Provisioning (UP) |
| --- | --- | --- | --- | --- | --- |
| 5 | X |  | X |  | X |
| 6 | X |  |  | X |  |
| 27 | X |  |  | X | X |
| 30 | X |  |  | X |  |
| 31 | X | X |  |  | X |
| 33 |  | X |  |  | X |
| 34 |  | X |  |  | X |
| 35 | X |  |  | X | X |
| 36 | X |  |  |  |  |
| 37 | X |  |  |  |  |
| 39 |  | X |  |  | X |
| 40 | X |  |  |  | X |

For onboarding (component 1), there are three different options that have been proposed:

- UE Onboarding using Default UE credentials (non-3GPP) and O-SNPN as Onboarding Network (ON)

- UE Onboarding using 3GPP PLMN credentials (IMSI, AKA) and PLMN as Onboarding Network (ON)

- UE Onboarding using no primary authentication credentials and O-SNPN as Onboarding Network (ON)

For remote provisioning of SO-SNPN credentials there two categories of solutions:

- Using Control Plane methods for remote provisioning, meaning some modifications in the UE Configuration Update or UE Parameter Update procedure

- Using User Plane protocols after the UE establishes a PDU session as per the UE onboarding procedures listed above

NOTE: The following definitions are assumed as defined in TR 23.700-07:

Default UE credentials: Information that the UE have before the actual onboarding procedure to make it uniquely identifiable and verifiably secure.

Default Credential Server (DCS): The server that can authenticate a UE with default UE credentials or provide means to another entity to do it.

NPN: Non-Public Network as defined in TS 23.501 [4]. The terminology NPN refers to both SNPN and PNI-NPN in this TR unless otherwise stated.

NPN credentials: Information that the UE uses for authentication to access a NPN. NPN credentials may be 3GPP credentials or non-3GPP credentials.

Onboarding Network (ON): The network providing initial registration and/or access to the UE for UE Onboarding.

Provisioning Server: The server that provisions the authenticated/authorized UE with the subscription data and optionally other configuration information.

Subscription Owner (SO): The entity that stores and as result of the UE Onboarding procedures provide the subscription data and optionally other configuration information via the PS to the UE.

Given the UE onboarding and remote provisioning components are rather distinct up to a point is possible to combine the use of UE onboarding option with either control or user plane remote provisioning as explained in the figure below.

Primary authentication using

Default UE credentials in O-SNPN

Primary authentication using

PLMN credentials in PLMN ON

No primary authentication

User plane protocols

Using control plane

procedure e.g.

UPU, UCU

UE Onboarding (component 1)

Remote Provisioning (component 2)

- For PNI-NPN

| Solutions | Onboarding | Provisioning (CP) | Provisioning (UP) |
| --- | --- | --- | --- |
| 7 |  |  | X |
| 27 |  |  |  |
| 28 |  | X | X |
| 31 |  |  | X |
| 32 |  | X |  |
| 33 |  |  | X |
| 38 |  | X |  |

For PNI-NPN, there is no UE onboarding concept, since the UE always has access to the PLMN using its UICC-stored/ PLMN credentials and the purpose is to provision credentials used for secondary authentication (NSSAA or secondary PDU session authentication). It is therefore assumed that UE will get initial connectivity following normal PLMN access procedures and then it can be provisioned with NSSAA or secondary PDU session authentication.

For provisioning of NSSAA or secondary PDU session authentication there are also two approaches:

- Using Control Plane methods for remote provisioning, meaning some modifications in the UE Configuration Update or UE Parameter Update procedure

- Using User Plane protocols after the UE establishes a PDU session as per the UE onboarding procedures listed above

Primary authentication using

PLMN credentials

User plane protocols

Using Control

Plane procedure

e.g. UPU, UCU

UE Onboarding (component 1)



N/A

Remote Provisioning (component 2)

# Analysis of solutions and proposed interim conclusions

- SNPN

UE Onboarding (Component 1 of KI#4)

As there are three different options for onboarding for restricted access the following interim conclusions are proposed:

- UE Onboarding with Default UE credentials in Onboarding SNPN (O-SNPN) as Onboarding Network (ON) used for primary authentication shall be possible

- For UE onboarding with default credentials the distribution of security functions between the O-SNPN and SO-SNPN should be decided by SA3

- Decision on whether primary authentication by O-SNPN is always needed or absence of primary authentication by O-SNPN should be allowed for UE onboarding to be decided by SA3

- Using PLMN credentials and using PLMN as Onboarding Nework (ON) for UE onboarding is already possible and does not require any further standardisation work

Provisioning (Component 2 of KI#4)

- Both options of using CP procedures and using user plane protocols after establishing PDU session with restricted access shall be enabled from the architecture to perform remote provisioning for SO-SNPN credentials

- User plane remote provisioning protocol used for provisioning of SO-SNPN credentials shall be left out of scope of SA2. TBD in SA3 whether out of scope of 3GPP as well

- Control plane procedures for remote provisioning of SO-SNPN credentials shall be based on existing UE Parameters Update procedure

- PNI-NPN

Onboarding (Component 1 of KI#4)

- No UE onboarding (component 1 of KI#4) is needed for the case of PNI-NPN credentials provisioning since the primary authentication for initial access is based on PLMN credentials

Provisioning (Component 2 of KI#4)

- Both options of using CP procedures and using user plane protocols after establishing PDU session shall be enabled from the architecture for PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication

- User plane remote provisioning protocol for provisioning of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication shall be left out of scope of SA2. TBD in SA3 whether out of scope of 3GPP as well

- Control plane procedures for provisioning of of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication shall be based on existing UE Parameters Update procedure

# Conclusion

It is proposed to document the following text in TR 23.700-07.

>>>>START CHANGES<<<<

Primary authentication using

Default UE credentials in O-SNPN

Primary authentication using

PLMN credentials

No primary authentication with O-SNPN

User plane protocols

Using control plane

procedure e.g.

UPU, UCU

Onboarding (component 1)

Provisioning (component 2)

# 8	Conclusions

## 8.X	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4

SNPN UE onboarding (Component 1 of KI#4)

Remote provisioning for SNPN credentials (Component 2 of KI#4)

-

User plane remote provisioning protocol used for provisioning of SO-SNPN credentials .How the UE downloads the NPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2 and uses industry developed mechanisms;

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP based procedure;

-	It shall be possible to pre-configure the Provisioning Server (PS) identity, SO-SNPN identity, neither or both on the UE and it also shall be possible that the O-SNPN provides the PS address, SO-SNPN identity, neither or both to the UE after successful authentication and authorization. The PS address from the O-SNPN shall be integrity protected. The PS and/or SO-SNPN identity provided by the network is prioritized, if configured and overrides any PS and/or SO-SNPN identity stored in the UE. For this purpose, the PS address shall be part of the onboarding configuration data, which are made available to PCF and/or SMF dedicated to onboarding;

-	Configuration of the PS address in the onboarding configuration data can be supported using one of the following methods:

-	Pre-configured.

-	Dynamically by AF via NEF at O-SNPN, for instance using Service specific parameter provisioning procedure as specified in TS 23.502 [6] clause 4.15.6.7,

-	Using new onboarding specific API to be defined.

-	Configuration of PS address from the onboarding configuration data to the UE can be supported using one of the following methods:

-	SMF may deliver the Provisioning Server address(es) as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10);

-	Alternatively, Provisioning Server address(es) may be configured in the UE during Registration Procedure using UE Route Selection Policy (URSP) that may be subject UE capabilities. As part of UE initial registration (based on received UE capability information) AMF indicates to PCF that UE has requested restricted/provisioning registration. The PCF may initiate UE Policy delivery using UE Route Selection Policies (URSP), for instance to trigger UE after successful registration to request establishment of specific type of PDU Session limited to onboarding purposes only;

-	In addition, Provisioning Server address(es) may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area;

-	If upon succesful restricted access PDU session the UE still does not have a PS identity, the device uses a well-known FQDN to perform PS discovery.

NOTE: SA3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.

UE Onboarding for PNI-NPN (Component 1 of KI#4)

Remote provisioning for PNI-NPN credentials (Component 2 of KI#4)

-	At least network initiated remote provisioning of credentials to allow access to PNI-NPN services should be supported in Rel-17;

-	Both options of procedures using Control Plane and using User Plane protocols after establishing PDU session shall be enabled from the architecture for PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication;

-	User plane remote provisioning protocol for provisioning of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication i.e. how the UE download the NPN credential from the PS after PDU session establishment in PNI-NPN is out of scope of SA2;

-	Control plane procedures for remote provisioning of of PNI-NPN credentials used for NSSAA and/or PDU Session secondary authentication shall be based on existing UE Parameters Update procedure;

Editor's note:	For CP based method, whether an extra security layer for protection of credentials between PS and UE is needed is FFS.

- Vertical may verify UE before PNI-NPN credential is provisioned to UE, and how does vertical verify the UE is out of scope of 3GPP.

>>>>END OF CHANGES<<<<