SA WG2 Meeting #140e	S2-2004842

# Aug 19th – Sept 1st, 2020 ; Elbonia        		                					(revision of S2-2004842)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2329 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the mapping from IP addressing information provided to an AF to the user identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA6 LS (S2-2004784 = S6-200947) on IP address to GPSI translation asking - SA6 Question: Does the core network provide an API to translate a UE's IP address (private and public) to its GPSI, and if not, would it be feasible to provide such functionality in Rel-17 in order to address the SA6 requirement for UE IP address translation? - SA6 Question: is it feasible to provide application-specific GPSIs, to ensure that a single GPSI can not be used to track an end user's activity across applications (EASs), to protect end user privacy? |  |  |  |  |  |  |  |  |
| Summary of change: |  | An AF may request (via the NEF) information exposure or parameter provisioning targetting an individual UE, identifying the target UE by providing IP addressing information (IP address and port) of the UE. In this case the 5GC first needs to retrieve a Permanent Identifier of the UE The IP addressing information (IP address and port information) of the UE that is provided by the AF may correspond to an UE IP address as allocated by 5GC or to an IP address that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC needs to first get from the NATF (new 5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE and then a Permanent Identifier of the UE; The CR defines (Figure 4.15.3.2.X-1) how the NEF determines a Permanent Identifier of the UE in this case. Once the procedure described in Figure 4.15.3.2.X-1 has taken place, the NEF Event exposure or the parameter provisioning as defined in R16 may apply. Nnef_EventExposure_Subscribe, Nnef_Location_LocationUpdateNotify and Nnef_ParameterProvision operations are modified accordingly. NATF Services are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for an AF to request Event exposure or parameter provisioning targetting an individual UE, identifying the target UE by providing IP addressing information (IP address and port) of the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.X (new) ; 4.15.6.X (new) ; 5.2.6.2.2; 5.2.6.4; 5.2.6.21.2 ; 5.2.13.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.501 CR2385 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is only provided for technical endorsement. No creation of R17 version of the specification is meant This version removes “Depending on operator policies, the NEF may provide in the answer provided to the AF a GPSI of the UE;” and all related chnages |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.3.2.X	Handling AF requests when the UE is identified via IP addressing information

An AF may via the NEF request Event Exposure (as defined otherwise in other sub-clauses of clause 4.15.3) or parameter provisioning (as defined in clause 4.15.6) targetting an individual UE, identifying the target UE by providing IP addressing information (IP address and port) of the UE.

In this case the 5GC first needs to retrieve a Permanent Identifier of the UE based on:

- the IP addressing information (IP address and port information) of the UE as provided by the AF: this may correspond to an UE IP address as allocated by 5GC or to an IP address (and port) that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC needs to first get from the NATF (5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE,

- the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF using the identity of the AF;

Figure 4.15.3.2.X-1 defines how the NEF determines a Permanent Identifier of the UE in this case. Once the procedure described in Figure 4.15.3.2.X-1 has taken place, the NEF Event exposure as defined in other sub-clauses of clause 4.15.3 or the parameter provisioning as defined in clause 4.15.6 may apply.

Figure 4.15.3.2.X-1: Handling AF requests when the UE is identified via IP addressing information

1.	(when NAT is deployed in the 5GC) The NATF registers to the NRF the public IP address ranges it owns in association with the service where address and port mapping queries may be invoked: the NATF invokes Nnrf_NFRegister or Nnrf_NFManagement_NFUpdate Request (as described in clause 4.17.1 or 4.17.2).

2.	The NRF acknowledges the NATF requests via Nnrf_NFManagement_NFRegister response or Nnrf_NFManagement_NFUpdate response

3.	An application hosted on an EAS receives IP traffic from UE (3a). This traffic may have been subject to NAT. The application may request (3b) an AF e.g. to get or set information about the UE. This request is out of scope of 3GPP.

4	Based on the application request, the AF issues a Nnef request as described in clause 4.15.6 or in other sub-clauses of caluse 4.15.3. This request contains as target UE identifier: UE IP addressing information as seen by the application (source IP y, source Port m in the figure).

5a	If the NEF determines that UE IP addressing information corresponds to a NATed address the NEF may invoke (5a) Nnrf_NFDiscovery_Request (as defined in clause 4.17.4) to discover the NAT binding service corresponding to the UE IP addressing information. The NEF may skip this step if it has cached the result of a previous Nnrf_NFDiscovery_Request.

5b	the NRF sends Nnrf_NFDiscovery_Request Response message including NF Profile of the NATF owning the UE IP address provided in the request. The NATF Profile contains the ranges of (external) IP address “owned” by the NATF;  the NEF may cache the NATF Profile, thus removing the need to further contact the NRF for another external IP address within one of the  ranges indicated in the NATF Profile as long as the caching timer has not elapsed.

6a.	(If the NEF determines that UE IP addressing information corresponds to a NATed address) The NEF invokes the Nnat_lookup_Get to get the 5GC allocated UE IP address (source IP x, source Port k in the figure) from the UE IP addressing information provided by the AF in step 4 (source IP y, source Port m in the figure).

6b.	The NATF answers with the requested addressing information (source IP x, source Port k in the figure).

7a.	The NEF invokes the Nbsf_Management discovery request (IP x, DNN, S-NSSAI) to get the SUPI and GPSI corresponding to the 5GC IP address

7b	The BSF answers (Nbsf_Management discovery response) with the SUPI and GPSI of the UE

8.	The NEF Event exposure as defined in other sub-clauses of clause 4.15.3 or the parameter provisioning as defined in clause 4.15.6 may apply as now the NEF has determined the target UE;.

NEXT CHANGE (2)

### 4.15.6	External Parameter Provisioning

#### 4.15.6.X	Parameter Provisioning when the UE is identified via IP addressing information

Handling of Parameter Provisioning requests targeting an individual UE when the UE is identified via IP addressing information is described in clause 4.15.3.2.X. Once the procedure described in clause 4.15.3.2.X has taken place, the Parameter Provisioning as described in other sub-clauses of 4.15.6 may apply.

NEXT CHANGE (3)

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs (required): (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI, SUPI, External Group Identifier, Internal Group Identifier or, as defined in clause 4.15.3.2.X, IP addressing information (IP address and port) of an UE), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Event Filter, (set of) Application ID(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs (required): When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding event report is included, if available (see clause 4.15.1),

NEXT CHANGE (4)

#### 5.2.6.4	Nnef_ParameterProvision service

##### 5.2.6.4.1	General

This service is for allowing external party to provision of information which can be used for the UE in 5GS.

##### 5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g., Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management).

Inputs (required): AF ID, Transaction Reference ID.

Inputs (optional): GPSI or, as defined in clause 4.15.3.2.X, IP addressing information (IP address and port) of an UE, External Group ID at least one of the Expected UE Behaviour parameters or at least one of the Network Configuration parameters or 5G VN related information, Validity Time or Location Privacy Indication parameters.

Outputs (required): Operation execution result indication.

Outputs (optional): Transaction specific parameters, if available ,

##### 5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer creates a 5G VN group.

Inputs (required): AF ID, Transaction Reference ID.

Inputs (optional): GPSI, External Group ID for 5G VN group creation, External Group ID, 5G VN group related information (e.g., 5G VN group data, 5G VN membership management).

Outputs (required): Operation execution result indication.

Outputs (optional): Transaction specific parameters, if available.

##### 5.2.6.4.4	Nnef_ParameterProvision_Delete service operation

Service operation name: Nnef_ParameterProvision_Delete

Description: The consumer deletes a 5G VN group.

Inputs (required): AF ID, Transaction Reference ID.

Inputs (optional): External Group ID.

Outputs (required): Operation execution result indication.

Outputs (optional): None.

##### 5.2.6.4.5	Nnef_ParameterProvision_Get service operation

Service operation name: Nnef_ParameterProvision_Get

Description: The consumer gets the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters).

Inputs (required): GPSI or, as defined in clause 4.15.3.2.X, IP addressing information (IP address and port) of an UE, AF ID, requested information (e.g., Expected UE Behaviour, Network Configuration parameters).

Inputs (optional): None.

Outputs (required): Requested data, Operation execution result indication

Outputs (optional): None.

NEXT CHANGE (5)

##### 5.2.6.21.2	Nnef_Location_LocationUpdateNotify service operation

Service operation name: Nnef_Location_LocationUpdateNotify

Description: Provides UE location information to the consumer NF.

Inputs (required): Identity of the AF, UE identifier (GPSI or, as defined in clause 4.15.3.2.X, IP addressing information (IP address and port) of an UE), event causing the location estimate (5GC-MO-LR), location estimate, age of location estimate, accuracy indication, LCS QoS class.

Inputs (optional): None.

Outputs (required): Success/Failure indication.

Output (optional): Failure Cause (in the case of failure indication provided)

NEXT CHANGE (6)

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI). This operation may also be used to determine the SUPI/GPSI from the tuple (UE address, DNN, S-NSSAI) e.g. as defined in clause 4.15.3.2.X.

Inputs, Required: UE address (i.e. IP address or MAC address), DNN [Conditional], S-NSSAI [Conditional].

NOTE:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: SUPI, GPSI.

Outputs, Required: PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]), SUPI / GPSI (e.g. as defined in clause 4.15.3.2.X).

Outputs, Optional: None.

NEXT CHANGE (7)

### 5.2.X	NATF Services

#### 5.2.X.1	General

The following table shows the NATF Services and Service Operations:

Table 5.2.X.1-1: NF services provided by the NATF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnat_lookup | get | Request/Response | NEF |

#### 5.2.X.2	Nnat_lookup service

##### 5.2.X.2.1	General

The NATF provides the Nnat_lookup_Get service operation.

##### 5.2.X.2.2	Nnat_lookup_Get service operation

Service Operation name: Nnat_lookup_Get

Description: provide the translation from the UE IP address and port provided in input into the IP address as provided by the 5GC to the UE.

Inputs, Required: UE IP address and port (as detected by an application).

Inputs, Optional: None.

Outputs, Required: Result indication, UE address and port (as handled by the UE).

Outputs, Optional: None.

END OF CHANGES