3GPP TSG-WG SA2 Meeting #140E	S2-2005012r05

Elbonia, Aug 19 – Sep 01, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorperation

Title:	Update of solution #6: NWDAF decompose architecture

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes an update to solution#6.

# 1. Introduction

This contribution proposes an update to solution#6: NWDAF decompose architecture.

# 2. Text Proposal

It is proposed to capture the following update in TR 23.700-91.

* * * * Start of Changes * * * *

## 6.6	Solution #6: NWDAF decompose architecture

### 6.6.1	Description

#### 6.6.1.1	General

This solution addresses Key Issue#1: Logical decomposition of NWDAF and possible interactions between logical functions.

#### 6.6.1.2	Architecture

The NWDAF may be composed by the following functions:

1).	ML model training function: used to train the model;

2).	Analytics function: used to do the data analytics for the consumer and expose the analytics result to the consumer;

Different types of NWDAF maybe deployed in the network, a NWDAF may only support ML model training function, or only support Analytics function, or support both of these two Functions.

#### 6.6.1.3	ML Model Training

The ML model training function is used for initial ML model training and the further update of trained ML model, the initial ML model can be provided by a ML designer or local configuration. The ML designer is a designer environment to create the initial ML model which can be operated either by a 3rd party, or by the operator.

NOTE 1:	The ML designer function is out of SA2 scope, how the ML designer provides the initial ML model to ML model training function is out of SA2 scope.

NWDAF Data Repository is the data storage entities that stores the data collected by NWDAF from other 5G NFs.  NWDAF Data Repository provides training data to the ML model training function for the initial model training.

NOTE 2:	The NWDAF data repository is addressed by the Key issue 11: Increasing efficiency of data collection.

The ML model training function is also used to further update the trained ML model. NWDAF Data Repository provides training data to the ML model training function for the further update of trained ML model.

#### 6.6.1.4	Data analytics

The successful trained ML models are provided to the Analytics function with the related analytics IDs, the Analytics Function can register the supported analytics IDs to NRF. If any analytics request / subscribe is received from the consumer, the Analytics function will collect the data from NWDAF Data Repository or trigger the NWDAF Data Repository to collect data from other entities, performs the data analytics according to the supported ML model and exposure the output to the consumer.

NOTE 3:	The Analytics request / subscribe procedure is initiated from consumer to the Analytics function is already supported in Rel-16. The data collection from NWDAF Data Repository is related to Key issue 11: Increasing efficiency of data collection.

There are two options about how the Analytics function receives the provided trained ML models.

Option 1: ML model provides via ML designer

Figure 6.6.1.4-1 proposed a NWDAF decompose architecture in which the ML designer provides the trained ML model to Analytics Function(s).

Figure 6.6.1.4-1: ML model provides the trained ML model to Analytics Function(s)

Step 1 and 2 in Figure 6.6.1.4-1 refer to the ML model training described in clause 6.6.1.3. After the initial ML model training success, the ML Model Training Function sends the trained model to the ML designer and the ML designer provides the ML model with the corresponding Analytics ID to the Analytics Function. ML designer may publish the trained model to different Analytics function s.

NOTE 4:	How ML designer sends the trained ML model to Analytics functions is to addressed by Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Option 2: ML model provided by ML Model training function

Step 1 and step 2 in Figure 6.6.1.4-2 refer to the initial ML model training described in clause 6.6.1.3 (same as option 1). After ML model training success, the ML Model training function providess the trained ML model and the corresponding Analytics ID to Analytics function. The trained ML model may provide the trained model to different Analytics functions.

NOTE 5:	How ML Model training function sends the trained ML model to Analytics functions is to addressed by Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Figure 6.6.1.4-2: ML Model training function  provides ML model

### 6.6.2	Procedures

#### 6.6.2.1	Initial ML Model Training

Figure 6.6.2.1-1 is the call flow of Initial ML Model Training.

Figure 6.6.2.1-1: Initial ML Model Training call flow

0. The initial ML model maybe received from ML Designer or based on local configuration. This step is out of 3GPP scope.

1. ML Model Training Function sends the Initial ML model training data collection request (Analytics ID) to the NWDAF Data Repository. The Analytics ID identify the Initial ML model.

2. NWDAF Data Repository may trigger the data collection from other entities and provides all the input data that related to the requested Analytics ID to ML Model Training Function.

3. ML Model Training Function performs the initial model training.

Step 1 and step 2 will be reused for further update the trained ML model.

#### 6.6.2.2	Analytics Function receives ML model procedure

For option 1 ML model training function sends the trained ML model to ML designer, and ML designer provides the ML model to different Analytics Functions. The interaction between ML Model training function and ML designer and ML designer to Analytics Function are out of SA2 scope.

For option 2, a standard interface between ML Model Training Function and Analytics Function are supported for ML model publish procedure.

The ML Model Training Function may provide the trained ML model to Analytics Function(s) either by the request received from Analytics Function(s) described in Figure 6.6.2.2-1 or operator configuration in ML Model Training Function described in Figure 6.6.2.2-2.

Figure 6.6.2.2-1 ML Model request procedure

1. 	Operator may configure the supported Analytics ID(s) for the Analytics Function, the Analytics Function send the ML Model Request (Analytics ID) message to ML Traninig Function to collect the trained ML Model.

2. 	ML Model Training Function provides the trained ML model to Analytics Function(s).

NOTE 6:	The parameters in step 1 and 2 and are related to Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Figure 6.6.2.2-1: ML Model Notification procedure

1.  ML Model Training Function provides the trained ML model to Analytics Function(s) based on the operator’s configuration.

3. The Analytics Function(s) send the ACK to ML Model training Function.

NOTE 7:	The parameters in step 1 and are related to Key Issue #19: Trained data model sharing between multiple NWDAF instances.

### 6.6.3	Impacts on services, entities and interfaces

NWDAF

-	NWDAF is decompose to ML Model Training Function and Analytics function

-	for option 2

- an interface between ML Model Training Function and Analytics Function is supported.

- the ML Model Training Function provides ML Model to Analytics Function via the interface between ML Model Training Function and Analytics Function.

* * * * End of Changes * * * *