SA WG2 Meeting #140e	S2-2005065 r01

# Aug 19th – Sept 2nd, 2020 ; Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1, new solution: PC5 DRX configuration for QoS-aware and power-efficient communication

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / Rel-17

Abstract of the contribution: This paper proposes a new solution for Key Issue #1, based on pre-defined applicable DRX patterns provisioned to the V2X layer.

# 1 Introduction

The paper introduces a solution for configuring the PC5 DRX schedule of UEs in a manner that avoids frequent conflicts among the schedules selected and applied by different UEs that need to communicate with each other either via unicast PC5 or via broadcast/groupcast PC5.

Conflicts among different UE PC5 DRX schedules (which can lead to buffering and delays), as well as conflicts between the AS resource pool used for PC5 communications and the applied DRX schedules, are avoided via the use of a pre-defined set of applicable DRX patterns, which can be provisioned to the PC5 UEs either by the CN or by the NG-RAN.

Furthermore, in order to achieve a good tradeoff of QoS awareness and power efficiency, the proposal includes the following criteria for the selection of an applicable DRX pattern:

- Parameters of pattern set

- V2X application(s) requirements

- DRX patterns applied by other UEs

- Uu DRX configurations

Information about the applied DRX patterns can be provided back to the V2X applications for potential adjustments of their communication schedules and configurations.

# 2 Proposal

It is proposed to update TR 23.776-010 as follows:

* * * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |
| --- | --- |
| Solutions | 1 QoS aware NR PC5 power efficiency |
| 1 | ♥ |
| X | ♥ |

* * * * Next Change * * * *

## 6.X	Solution #X: PC5 DRX configuration for QoS-aware and power-efficient communication of Pedestrian UEs

### 6.X.1	Functional Description

In road safety as well as public safety use cases, Pedestrian UEs are often involved in PC5 groupcast or broadcast communication in connectionless manner. One pedestrian UE may need to listen to various PC5 Tx UEs in proximity without knowing about Tx UEs in advance. Thus, possible DRX operation for PC5 Rx UE needs to cope with various and random Tx UEs present in proximity. PC5 Tx UE, on the other hand, may not know or care about presence of any particular Rx UE in proximity. This makes PC5 DRX operation specific to each pair of PC5 Tx UE and Rx UE rather impractical, especially for PC5 groupcast and broadcast.

In connection-oriented PC5 unicast case, DRX operation specific to a pair of Tx UE and Rx UE over PC5 may be agreed beforehand between Rx UE and Tx UE. However, if the PC5 unicast Tx UE or Rx UE has other PC5 unicast/groupcast/broadcast communication services with the same or different peer UEs in addition to the PC5 unicast in question, uncoordinated PC5 DRX configuration for each PC5 unicast communication may lead to mis-aligned PC5 DRX patterns. This makes PC5 DRX overall less effective in terms of power saving.

To coordinate PC5 DRX configurations not only between the peered PC5 Tx UE and Rx UEs for one SL application/service but also among a number of PC5 UEs involved in multiple PC5 communications in proximity, this solution is based on having a common set of PC5 DRX patterns configured in PC5 UEs by the serving network for in-coverage operation or pre-configured for out-of-coverage operation. A PC5 DRX pattern includes information about the ON/OFF periods that shall be used in the (AS-layer) PC5 DRX schedule and each PC5 DRX pattern may be associated with one or more V2X services. Note that there may be DRX patterns that would rather fit combinations of services, therefore probably using longer ON periods than patterns associated with only one of these services. Thus, the (pre-)configured set of PC5 DRX patterns may be corresponding to supported combinations of different use cases, service profiles, status or classes of PC5 UEs. The (pre-)configured set of PC5 DRX patterns can be also in line with resource pool configurations, because the patterns can be known to the AS layer when the resource pools are configured (or the other way round), so that the V2X layer does not need to deal with or understand resource pool configurations. However, the resource pool configuration does not need to be performed in a way that guarantees dedicated radio resources for specific V2X services. In this way, the extensive coordination between relevant PC5 UEs in proximity to determine and agree on the PC5 DRX patterns can be avoided. The relevant PC5 UEs only needs to select one or more PC5 DRX pattern(s) from the limited set of configured DRX patterns and adapt SL transmissions and receptions to the selected pattern(s) of their own and other PC5 UEs in proximity.

Editor's note: 	The question of having a single or two separate DRX schedules for Uu and PC5 is still open also in RAN WG.

### 6.X.2	Procedures

Figure 6.X.2-1 illustrates an example operation of the solution for power efficient PC5 communication for Pedestrian UEs based on pre-defined PC5 DRX patterns.

V2X App(s)

V2X Layer

AS Layer

UE1

V2X App(s)

V2X Layer

AS Layer

UE2

NG-RAN

AF/PCF

1a. Provision pre-configured DRX pattern set

1b. Provision DRX pattern set and Uu DRX configurations (if “served by NR/E-UTRA”)

2. Requirements

3. Select DRX pattern(s) based on available pattern set, app requirements, Uu DRX config and other selected DRX patterns of other UEs (cf. step 7)

4. Request DRX pattern

5. Apply DRX pattern

6. DRX pattern applied or error response

7. Indicate applied DRX pattern

8. Adapt app communication schedule based on received DRX pattern(s) of at least one related UE

9. Adapt SL transmission schedule based on received DRX pattern(s) of at least one related UE

Figure 6.X.2-1: PC5 DRX configuration procedure for QoS-aware and power-efficient communication of Pedestrian UEs

1a. PC5 UEs are (pre-)configured with the PC5 DRX related parameters including a set of applicable PC5 DRX patterns from which PC5 UEs can select. The PC5 DRX configuration may be either provided by the PCF (or by the AF via NEF/PCF) using the procedure specified in TS 23.287 clause 6.2, or pre-configured as specified in TS 24.587 clause 5.2.

1b. If the UE is “served by NR” (as defined in TS 23.287), the PC5 DRX configuration may be alternatively provided at the AS layer, either broadcasted or with dedicated RRC signalling. In this option, the AS layer may indicate the relevant configured PC5 DRX parameters to the V2X layer or apply PC5 DRX configuration on the AS layer (see also the description of step 5).

2. V2X applications provide their requirements to the V2X layer as defined in TS 23.287 clause 5.4.1.1.

3. The V2X layer of UE1 selects at least one PC5 DRX pattern from the (pre-)configured set. The selection takes into account i) the DRX pattern parameters provided in the pattern set, ii) the requirements of all active PC5 applications that the PC5 UE currently has, iii) the Uu DRX configurations received by the NG-RAN (if any), and iv) the selected PC5 DRX patterns that other relevant PC5 UEs have applied and indicated.

4. The V2X layer of UE1 requests the application of (one of) the selected PC5 DRX pattern(s) from the AS layer for control of PC5 transmission and reception according to the selected PC5 DRX pattern.

5. The AS layer of UE1 decides to apply or not the requested PC5 DRX pattern (or one of the requested DRX patterns), i.e., it configures the (AS-layer) PC5 DRX schedule based on the ON/OFF periods indicated in the pattern.

NOTE 1: The selection and request of DRX patterns at the V2X layer is optional and the PC5 DRX schedule (and further related configuration) might be decided on the AS layer only, though ideally based on knowledge about V2X application requirements and traffic patterns. In this case the AS layer may know the set of applicable DRX patterns (as well as all further inputs that it requires) and send indications about the applied DRX schedule to other UEs using mechanisms specified by the RAN WG.

6. The AS layer informs the V2X layer about the success or failure of the application of the requested DRX pattern(s), indicating which pattern was selected if multiple were provided in the request.

7. The V2X layer of UE1 indicates the applied PC5 DRX pattern to other relevant PC5 UEs (e.g. UE2) to allow PC5 UE2 to select its PC5 DRX pattern in a coordinated manner. UE1 may indicate the applied PC5 DRX pattern also to UEs with which it has no established PC5 unicast link.

8. The V2X Apps of UE2 may retrieve from the UE2 V2X layer information about the PC5 DRX patterns applied by one or more related UEs and adjust their communication schedule.

Editor's note: If the information about the applied DRX patterns is “pushed” from the V2X Layer to applications based on a subscription-notification mechanism or if they shall be “pulled” by the V2X apps on demand is FFS and it shall depend also on decisions about the V2X layer interface design.

9. The AS layer of UE2 may retrieve from the UE2 V2X layer information about the PC5 DRX patterns applied by one or more related UEs and adapt the PC5 transmission schedule accordingly.

Editor's note: If the information about the PC5 DRX patterns applied by other UEs is “pushed” from the V2X Layer to the AS layer based on a subscription-notification mechanism or if they shall be “pulled” by the AS layer on demand is FFS and it shall depend also on the scope of related extensions that the RAN WG can agree for the AS layer.

.

### 6.X.3	Impacts on services, entities and interfaces

The solution has the following impact to the existing entities:

-	AF:

-	Needs to perform the provisioning of PC5 DRX related configuration including the set of applicable PC5 DRX patterns (if V2X layer procedure is applied for PC5 DRX parameter configuration).

-	NEF:

-	Needs to expose an API that can receive the AF-provided configurations mentioned above (if V2X layer procedure is applied for PC5 DRX parameter configuration).

-	PCF:

-	Needs to support, handle, and V2X Policies that include the set of applicable PC5 DRX patterns (if V2X layer procedure is applied for PC5 DRX parameter configuration).

-	AMF:

-	Needs to support the (transparent) signalling for PC5 DRX related configuration including the set of PC5 DRX patterns via NAS to the UE (if V2X layer procedure is applied for PC5 DRX parameter configuration).

-	NG-RAN:

-	Needs to support the signalling for PC5 (and Uu) DRX related configuration including the set of PC5 DRX patterns (if AS layer procedure is applied for PC5 DRX parameter configuration).

-	UE:

-	the V2X layer of PC5 UE needs to be able to select a PC5 DRX pattern from a (pre-)configured set based on inputs included in V2X Policies.

-	the AS layer of PC5 UE needs to adapt its PC5 transmissions and receptions according to the selected PC5 DRX pattern

* * * * End of Changes * * * *