SA WG2 Meeting #140E e-meeting 	S2-2005200

Elbonia, Aug 19 – Sep 01, 2020	 (revision of S2-20xxxxx)

Source:	CATT

Title:	KI#3, Sol#6: Service continuity for Layer-3 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes service continuity options for Layer-3 UE-to-Network Relay in sol#6.

# 1	Introduction

One of the issues for KI#3 UE-to-Network Relay is about service continuity.

It can be seen from the definition of “service continuity” in TS 22.261 and TS 23.501 that, service continuity does not require IP address preservation and can be achieved at application layer.

This contribution proposes to add some description about service continuity for sol#6.

# 2	Proposal

It is proposed to add update to sol#6 into TR 23.752.

First change (all text new)

### 6.6.1	Description

This is a solution for key issue #3, UE-to-Network Relay.

The ProSe 5G UE-to-Network Relay entity provides the functionality to support connectivity to the network for Remote UEs (see figure 6.6.1-1). It can be used for both public safety services and commercial services (e.g. interactive service).

A UE is considered to be a Remote UE for a certain ProSe UE-to-Network relay if it has successfully established a PC5 link to this ProSe 5G UE-to-Network Relay. A Remote UE can be located within NG-RAN coverage or outside of NG-RAN coverage.

Figure 6.6.1-1: Architecture model using a ProSe 5G UE-to-Network Relay

The ProSe 5G UE-to-Network Relay shall relay unicast traffic (UL and DL) between the Remote UE and the network. The ProSe UE-to-Network Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic;

-	For IP traffic over PC5 reference point, the ProSe UE-to-Network Relay uses IP type PDU Session towards 5GC.

-	For Ethernet traffic over PC5 reference point, the ProSe UE-to-Network Relay can use Ethernet type PDU Session or IP type PDU Session towards 5GC.

-	For Unstructured traffic over PC5 reference point, the ProSe UE-to-Network Relay can use Unstructured type PDU Session or IP type PDU Session (i.e. IP encapsulation/de-capsulation by UE-to-Network Relay) towards 5GC.

The type of traffic supported over PC5 reference point is indicated by the ProSe UE-to-Network Relay e.g. using the corresponding Relay Service Code. The UE-to-Network Relay determines the PDU Session Type based on, e.g. ProSe policy/parameters, URSP rule, Relay Service Code, etc.

NOTE:	How the UE-to-NW relay determines PDU session type should be evaluated independent from other part of this solution while considering other PDU session parameters, e.g. DNN, SSC mode.

IP type PDU Session and Ethernet type PDU Session can be used to support more than one Remote UEs while Unstructured type PDU Session can be used to support only one Remote UE.

Editor's note:	Support of non-unicast mode communication (i.e. one-to-many communication/broadcast or multicast) between network and UE-to-Network Relay UE and between UE-to-Network Relay and Remote UE(s) depends on the result of FS_5MBS work.

One-to-one Direct Communication is used between Remote UEs and ProSe 5G UE-to-Network Relays for unicast traffic as specified in solutions for Key Issue #2.

The protocol stack for Layer-3 UE-to-Network Relays is shown in Figure 6.6.1-2.

Figure 6.6.1-2: Protocol stack for ProSe 5G UE-to-Network Relay

Hop-by-hop security is supported in the PC5 link and Uu link. If there are requirements beyond hop-by-hop security for protection of Remote UE's traffic, security over PDU layer needs to be applied.

Further security details (integrity and privacy protection for remote UE-Nw communication) will be specified in SA WG3.

According to the definition of service continuity in TS 22.261 [3] and TS 23.501 [6], it can be seen that “service continuity” is different from “session continuity” by definition, and service continuity can be achieved at application layer regardless of IP address preservation:

- For Mission Critical Service in Public Safety, service continuity can be achieved by the application layer mechanism, e.g. as described in Annex B in TS 23.280 [xx].

- For commercial IMS use cases, service continuity can be achieved using mechanisms described in TS 23.237 [yy].

- For commercial use cases with application layer out of 3GPP scope (e.g. non IMS), service continuity can be achieved using similar way, e.g. QUIC.

It is noted that all of the above application layer mechanisms can be reused for Layer-3 UE-to-Network Relay without any enhancements in this study item.

Second change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS); Stage 1".

[3]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[4]	3GPP TS 22.115: "Service aspects; Charging and billing; Stage 1".

[5]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TR 22.842: "Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS); Stage 1".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[9]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[10]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[11]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[12]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3".

[13]	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) charging".

[14]	3GPP TR 23.713: "Study on extended architecture support for proximity-based services".

[15]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[16]	3GPP TS 32.256: "Charging management; 5G connection and mobility domain charging; Stage 2".

[17]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

[18]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[19]	3GPP TS 22.186: "Service requirements for enhanced V2X scenarios".

[20]	3GPP TS 24.334: " Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[21]	3GPP TS 24.333: " Proximity-services (ProSe) Management Objects (MO)".

[22]	3GPP TS 32.290: " Services, operations and procedures of charging using Service Based Interface (SBI)".

[23]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[24]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[25]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[26]	3GPP TR 23.733: "Study on architecture enhancements to Proximity Services (ProSe) UE-to-Network relay".

[27]	3GPP TR 36.746: "Study on further enhancements to LTE Device to Device (D2D), UE to network relays for Internet of Things (IoT) and wearables".

[28]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[xx]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[yy]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

End of changes