3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005349r01

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, #11, Sol #16: Updates to remove ENs

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to complete sol #16 by removing ENs.

1. Discussion

This contribution completes Solution #16: “Roles and inter-NWDAF instance cooperation from lower level NWDAF to upper level NWDAF in hierarchical architecture” in TR 23.700-91 by removing the following Editor’s Notes with providing further information.

- Editor's note:	How an NWDAF in a given level discovers the NWDAF instances in the upper and lower levels is FFS.

This contribution proposes that NWDAF in a given level discovers the NWDAF instances in the other tiers in two different options: static or dynamic binding. In a static binding, for example, NWDAFs in the lower-tier are configured with their owned analytics IDs, and with the analytics Ids of the higher-tier NWDAFs. In a dynamic binding, for example, NWDAFs in lower-tier are configured with their owned analytics IDs and discover the analytics IDs of higher levels NWDAFs via NRF.

- Editor's note:	Details for the mapping of an identification of a subscription to an Analytics IDs and the list of identification of triggered of data to be collected for such Analytics ID generation at other NWDAFs, NFs, AFs, and OAM are FFS.

To increase the efficiency of data collection in hierarchical NWDAF architecture, NWDAF may support a mapping mechanism for a better control and management of Analytics subscription and data to be collected from data sources. For example, the lower level NWDAF may generate an Analytics instance ID for the mapping of the same analytics subscription and/or an Event instance IDs for the mapping of the same data to be collected from the same data source. An Analytics instance ID is a unique ID (e.g., a combination of Analytics ID and Analytics filter information) which can be used to map with one or more the same Analytics subscription. An Event instance ID is a unique ID (e.g., a combination of Event ID and Event filter information) which can be used to map one or more the same Analytics instance IDs. By introducing Analytics instance ID and Event instance ID concept, one Event Instance ID can be associated with multiple Analytics Instance IDs and one Analytics Instance ID can be associated with multiple (unique) Event Instance IDs.

- Editor's note:	Details for the mapping of an identification of a subscription request are FFS.

Please see the same description of the above EN.

Additionally, we change the formatting of the solution to complain with the defined in TR 23.700-91 Clause 6.X.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91.

# * * * * First change * * * *

6.16	Solution #16: Roles and inter-NWDAF instance cooperation from lower level NWDAF to upper level NWDAF in hierarchical architecture

6.16.1	Description

This solution is proposed to address Key Issue #2: Multiple NWDAF instances and KI#11: Increasing efficiency of data collection. The solution describes how to avoid duplication of data collection when NWDAFs at the lowest levels of hierarchy need data for an analytics ID that might be also needed and collected by other NWDAFs in the same hierarchical lower level. The data collection requests or subscriptions refers to the data to be collected from source NFs or the aggregated data to be collected from other NWDAF instances in different hierarchies. This solution is complementary to "Solution #18: Roles and inter-NWDAF instance cooperation from upper level NWDAF to lower level NWDAF in hierarchical architecture".

6.16.2	Procedure

The procedure of inter-NWDAF instance cooperation from lower level NWDAF to middle level NWDAF in hierarchical architecture to coordinate data collection is presented from Figure 6.16.2-1 below. The one or more NWDAF instance in lower level is able to coordinate the triggering of data collection in its associated upper level for efficient data collection. Optionally, the NWDAF instances can be configured with the role of Data Collection Coordination (DCC) functionality.

The discovery of NWDAFs in the different levels of the hierarchy shall be based on NWDAF Serving area and analytics IDs information exposed in NWDAF NF Profile. An NWDAF is configured with the hierarchy information. The hierarchy information defines TAs belonging to each level of the hierarchy, as well as the level of the hierarchy associated with the configured NWDAF. Based on the hierarchy information, an NWDAF in a level of the hierarchy interacts with NRF to discover the NWDAFs associated with upper and lower levels of the hierarchy.

To increase the efficiency of data collection in hierarchical NWDAF architecture, the NWDAF in the different levels of the hierarchy may consider the local mapping mechanism as a solution option. For example, an Analytics instance ID which is a local unique ID (e.g., a combination of Analytics ID and Analytics filter information) can be used for the mapping of the same analytics subscription whereas Event instance ID which is a local unique ID (e.g., a combination of Event ID and Event filter information) can be used for the mapping of the same data to be collected from the same data source. If the NWDAF in one level of the hierarchy receives a new Analytics subscription request to Analytics ID, the NWDAF only creates an Analytics Instance ID, if no existing Analytics Instance ID can support the new Analytics subscription request. For example, a new Analytics subscription request with the same Analytics filter information is already subscribed by one Analytics consumer and an Analytics Instance ID is already created, the new Analytics subscription request can be mapped with the existing Analytics Instance ID. The same concept is applied to the Event instance ID(s) for the subscription/request of data collection. If a new Analytics Instance ID is created by a NWDAF, the NWDAF only creates new Event Instance ID(s) if the existing Event Instance IDs cannot be used.

Figure 6.16.2-1: Procedure for inter-NWDAF instance cooperation from lower level NWDAF to upper level NWDAF in hierarchical architecture

1.	NWDAF instances are configured with the role, e.g. the information of the NWDAF Serving Area of its own tier (e.g, top level, middle level and lower level), as well as the NWDAF serving areas in its associated tiers (e.g. upper level).

2.	An Analytics consumer requests to the NWDAF in a lower level for an Analytics subscription request.

3a.	The requested lower level NWDAF is configured with mechanisms for efficient data collection. To increase the efficiency of data collection in hierarchical NWDAF architecture, the lower level NWDAF generates the mapping of an identification of a subscription to Analytics IDs and the list of identification of triggered of data to be collected for such Analytics ID generation at other NWDAFs, NFs, AFs, and OAM. If the lower level NWDAF receives a new Analytics subscription request to Analytics ID, the lower level NWDAF only creates an Analytics Instance ID, if no existing Analytics Instance ID can support the new Analytics subscription request. If a new Analytics Instance ID is created by NWDAF, the NWDAF only creates new Event Instance ID(s) if the existing Event Instance IDs cannot be used. The lower NWDAF can be able to collect the data from its co-located NF. Based on the association information, the lower level NWDAF determines to ask the middle NWDAFs for any other data it requires.

3b.	If the data to generate the requested Analytics ID is not available, the lower level NWDAF sends the request for data collection to the NWDAF in the middle level, that has DCC functionality.

4.	The middle level NWDAF creates and check the mapping of an identification of a subscription request if data is available, otherwise the middle level NWDAF will request from other lower NWDAFs and/or NFs. The local mapping of the subscription/request of data collection to the actual data from data source can be achieved, e.g., using Event instance ID. For example, if the middle level NWDAF receives a new subscription/request of data collection, the middle level NWDAF only creates an Event Instance ID, if no existing Event Instance ID can support the new data collection subscription/request.

Case A:

5a.	The requested data is already stored and available at the middle level NWDAF and, thus the middle level NWDAF sends the requested data to the lower level NWDAF.

Case B:

5b.	If the requested of data  is not available at the middle NWDAF, i.e., there is no Event instance ID associated with to such requested data, and requires a new data subscription from the data source, e.g. NF. The middle level NWDAF sends the subscription request for the collected data from NF.

5c.	The NF sends the collected data to the middle level NWDAF.

5d.	The middle level NWDAF stores the collected data to be able to reuse if the same data collection is required by other NWDAFs.

5e.	The middle level NWDAF sends the request data to the lower level NWDAF.

NOTE:	For simplicity the procedure in Figure 6.16.2-1 shows only the case when the subscription to analytics ID arrives at the lower level NWDAF and the coordination for data collection reduction happens between lower and middle level NWDAFs. However, the solution principle is the same if the subscription to analytics ID arrives at middle level NWDAF and there is the need of coordination between middle and top level NWDAFs. The case when the subscription to analytics ID arrives at the top level NWDAF is described in Solution #18, Section 6.18.

6.16.3	Impacts on services, entities and interfaces

NWDAF:

-	Inter-coordination of NWDAF instances between lower level and upper level of different hierarchies, mechanisms to support DCC functionalities in hierarchical architecture and storage capabilities at NWDAF instances.

-	Local mapping mechanisms for efficient data collection, e.g., using new parameters of Analytics instance ID and Event instance ID.

-	New NWDAF service to support NWDAF-to-NWDAF interactions for data collection.

# * * * * End of changes * * * *