3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2005526r03

# Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2375 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding handover procedure between EPS and 5GC-TNGF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The handover procedures between EPS and 5GC-TNGF are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a clause to describe the handover procedures between EPS and 5GC-TNGF which includes the handover from EPS to 5GC-TNGF and the handover from 5GC-TNGF to EPS. The handover procedures can refer to the handover procedures between EPS and 5GC-N3IWF with some differences. Added reference to new clauses in general clause 4.11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handover procedures between EPS and 5GC-TNGF are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11, 4.11.X (new), 4.11.X.1 (new), 4.11.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.11.0	General

Clause 4.11 includes the following:

-	Procedures for interworking with EPS based on N26 interface (clause 4.11.1) and interworking without N26 interface (clause 4.11.2);

-	Handover procedures between EPS and 5GC-N3IWF (clause 4.11.3), handover procedures between EPS and 5GC-TNGF (clause 4.11.3a) and handover procedures between EPC/ePDG and 5GS (clause 4.11.4);

-	Impact to 5GC procedure due to interworking with EPC (clause 4.11.5);

-	Interworking for common network exposure (clause 4.11.6).

In clause 4.11, UEs are assumed to support both 5GC NAS and EPC NAS unless explicitly stated otherwise.

# * * * * Second change * * * *

### 4.11.3a	Handover procedures between EPS and 5GC-TNGF

#### 4.11.3a.1	Handover from EPS to 5GC-TNGF

The handover procedure from EPS to 5GC-TNGF is supported as specified in clause 4.11.3.1 for handover procedure from EPS to 5GC-N3IWF with the following differences:

-	The untrusted non-3GPP access is substituted by trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by TNGF.

-	The registration procedure via trusted non-3GPP access as specified in clause 4.12a.2 shall be performed in step 1.

-	The UE requested PDU Session Establishment with Existing PDU Session indication in 5GC via trusted non-3GPP access is performed in step 2 as specified clause 4.12a.5.

#### 4.11.3a.2	Handover from 5GC-TNGF to EPS

The handover procedure from 5GC-TNGF to EPS is supported as specified in clause 4.11.3.2 for handover procedure from 5GC-N3IWF to EPS with the following differences:

-	The untrusted non-3GPP access is substituted by trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by TNGF.

-	The network requested PDU Session Release via TNAP and TNGF shall be initiated as specified in clause 4.12a.7 from steps 3 to 12.

# * * * * End of changes * * * *