SA WG2 Meeting #140-e		S2-2005770r03

Online, Aug 19– Sept 02, 2020      	 	(merge of)

Source:	Sandvine

Title:	KI #7, Sol Z: Update to add the component of improves the QoS Sustainability Analytics ID

Document for:	Discussion, Approval.

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution is related to Key Issues #7, it improves the QoS Sustainability Analytics ID stated in TS-23.288; by adding in the input data the Application-IDs and the related KPIs to the TR 23.700-91.

# Discussion

Key Issue #7 proposes to study and determine which Analytics-ID, from those described in TS 23.288 improves analytic analysis. The addition of data can comprise of either Application ID, related KPIs or both; this data is available at the UPF as described in 23.503 clause 6.2.2.2 and is based on the packet inspection functionality available in the UPF as described in 23.501 clause 6.2.3.

This contribution improves the QoS Sustainability Analytic-IDs stated in TS-23.288; it focuses on improving the accuracy of analytic predictions and statistics reporting. Also, it determines which KPIs should be used.

This contribution is a component or update of the solution proposed in S2-2004974

# 2. QoS Sustainability Analytics

As defined in TS 28.554, QoS flow retain ability is a key performance indicator of how often an end-user abnormally losing a QoS flow during the time of its usage.

As part of the Input data for this Analytics ID, the RAN UE Throughput gross measurement is provided by the OAM. Likely the QoS Sustainability is highly correlated to the total cell throughput. As stated S2-2004974, predicting the throughput with samples of total throughput is not a good practice. For NWDAF to predict the QoS Sustainability with high accuracy, it needs to crunch the throughput split per Application-IDs and the QoS Sustainability samples. This will lead the ML models to provide a highly accurate prediction of the QoS Sustainability.

KPIs to add in the Input data:

The following KPIs are added as input data: Per UE per Application ID, are Round trip time UL/DL, Packets Lost UL/DL, Bytes UL/DL, Bytes Peak UL/DL. Also, it is essential to include the Application IDs that are present in the whole Cell.

These KPIs show how healthy is each Application ID present in the traffic, therefore reflecting the transport resources availability, etc., and will lead the statistic models to predict with high accuracy the QoS Sustainability.  Also, these KPIs will allow the ML models to point out the top ten Application IDs that are influencing most of the degradation of the QoS Sustainability.

# Text Proposal

The following changes are proposed to be applied to TR 23.700-91.

This contribution is an update of the contribution S2-2004974 adding the QoS Sustainability Analytics improvement starting in clause 6. X.9 of that contribution.

*** Start of the change ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Z | 7 |  |  |  |

6. X.9 Improvement of QoS Sustainability Analytics

Editor's note:  This clause describe how the QoS Sustainability Analytics ID improves analysis

*** Next (All new text) ***

As defined in TS 28.554, QoS flow retain ability is a key performance indicator of how often an end-user abnormally losing a QoS flow during the time of its usage.

As part of the Input data for this Analytics ID, the RAN UE Throughput gross measurement is provided by the OAM. Likely the QoS Sustainability is highly correlated to the total cell throughput. As stated S2-2004974, predicting the throughput with samples of total throughput is not a good practice. For NWDAF to predict the QoS Sustainability with high accuracy, it needs to crunch the throughput split per Application-IDs and the QoS Sustainability samples. This will lead the ML models to provide a highly accurate prediction of the QoS Sustainability.

KPIs to add in the Input data:

The following KPIs are added as input data: Per UE per Application ID, are Round trip time UL/DL, Packets Lost UL/DL, Bytes UL/DL, Bytes Peak UL/DL. Also, it is essential to include the Application IDs that are present in the whole Cell.

These KPIs show how healthy is each Application ID present in the traffic, therefore reflecting the transport resources availability, etc., and will lead the statistic models to predict with high accuracy the QoS Sustainability.  Also, these KPIs will allow the ML models to point out the top ten Application IDs that are influencing most of the degradation of the QoS Sustainability.

6. X.9.1 Input data improvement

It is performed one change in the input data part of the QoS Sustainability Analytics ID. The purpose of this change is to include the Application ID and respective KPIs

### The following new information is added to TS 23.288 Table 6.9.2-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| Application IDs | UPF | Per UE, timeslot, per 5QI and per UPF |
| Round Trip Time | UPF | Per Application ID |
| Packets Lost UL/DL | UPF | Per Application ID |
| Bytes UL/DL | UPF | Per Application ID |
| Bytes UL/DL Peak | UPF | Per Application ID |

6. X.9.2 Output analytics improvement

It is performed two main change in the Output analytics part of the QoS Sustainability Analytics ID. The purpose of this change is include in the Output of statistics and prediction a list of top ten Application IDs that are contributing the most in the UPF traffic (throughput). This information serves as criteria for the service consumer NFs in how to act in case of receiving non-desirable outcomes

The following new information is added to TS 23.288 Table 6.9.3-1: "QoS Sustainability" statistics

| Information | Description |
| --- | --- |
| Top ten Application IDs | A list of top ten Application IDs that are contributing the most in the traffic (throughput) |

The following new information is added to TS 23.288 Table 6.9.3-2: "QoS Sustainability" predictions

| Information | Description |
| --- | --- |
| Top ten Application IDs | A list of top ten Application IDs that are contributing the most in the traffic (throughput) |

6. X.9.3 Procedures improvement

They are performed changes in the clause 6.9.4 Procedures Figure 6.9.4-1 and steps descriptions. The purpose of this change is to include the part where the NWDAF receives Data available at the UPF.  It is also added the follow note

NOTE: How NWDAF receives data available at UPFs is not defined in this specs.

### The following new step is added to TS 23.288 Figure 6.9.4-1 and respective description.

Figure 6.9.4-1: "QoS Sustainability" analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period" when requesting QoS Sustainability analytics for monitoring for a path of interest.

- NWDAF receives Data available at the UPF

3.	The NWDAF collects the data specified in clause 6.9.2 from the OAM, following the procedure captured in clause 6.2.3.2.

4.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by consumer in any cell over the requested Analytics target period.

5.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

NOTE: How NWDAF receives data available at UPFs is not defined in this specs.

6. X.9.4 Nnwdaf Services Description

The following new information is added to TS 23.288 Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed, the top ten Application IDs that are contributing the most in the traffic; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. the top ten Application IDs that are contributing the most in the traffic |

6. X.9.5 Impacts on services, Existing Nodes and Functionality

NWDAF:

- To support additional data in the input data as follow

| Information | Source | Description |
| --- | --- | --- |
| Application IDs | UPF | Per UE, timeslot, per 5QI and per UPF |
| Round Trip Time | UPF | Per Application ID |
| Packets Lost UL/DL | UPF | Per Application ID |
| Bytes UL/DL | UPF | Per Application ID |
| Bytes UL/DL Peak | UPF | Per Application ID |

- To support additional data in the output data as follow

Statistics

| Information | Description |
| --- | --- |
| Top ten Application IDs | A list of top ten Application IDs that are contributing the most in the traffic (throughput) |

Predictions

| Information | Description |
| --- | --- |
| Top ten Application IDs | A list of top ten Application IDs that are contributing the most in the traffic (throughput) |

AF:

- The AF acting as service consumer of this Analytics ID in Rel -16 will need to add capacity to receive in the Outcome of this Analytics ID the Top tem Application IDs

*** End of the changes ***