3GPP TSG-SA2 Meeting #140-e	S2-2005803r01

# Online, , 19th Aug 2020 - 2nd Sep 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2398 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCC control for DDD status and availability after DDN failure events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At SA2#137e there was a principle agreement to follow solution B in discussion paper S2-2001809 for PCC control of the DDD status event and corresponding changes were already agreed (see S2-2002347). At SA#138 further changes for TS 23.502 and TS 23.503 were agreed (see S2-2003458 and S2-2003459, respectively): Policy Control Request Triggers were defined for "DDN Failure event Subscribed with Traffic Descriptor" and "DDN Delivery Status event Subscribed with Traffic Descriptor". It is assumed that the PCF provides PCC rules with traffic filters derived from the traffic filers received when those Policy Control Request Triggers are reported. The SMF needs to send evant notifications for traffic matching those traffic filters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | taking into consideration the traffic descriptor for the subscribed DDN status event for PCC rules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | unclear description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.8, 4.15.3.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

4.15.3.2.8	Information flow for downlink data delivery status with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about downlink data delivery notification. The data delivery status notifications relates to high latency communication, see also clauses 4.24.2 and 4.2.3.3.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.3.2.8-1: Information flow for downlink data delivery status with UPF buffering

1.	AF interacts with NEF to subscribe DDD status as described in clause 4.15.3.2.5.

2.	If the UPF is configured to apply extended buffering, step 2 is executed immediately after step 1. Otherwise, step 2 is executed when the SMF is informed that the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation as described in clause 4.2.3, and the SMF then also updates the PDR(s) for flows requiring extended buffering to requests the UPF to buffer downlink packets. If the DDDs status event with traffic descriptor has been received in the SMF in step 1, if extended DL Data buffering in the UPF applies, the SMF provides the Traffic Descriptor in a PDR and requests the UPF to report when there are corresponding buffered downlink packets or discarded packets in the UPF as defined in Clause 4.2.3. If PCC is not used and there is no installed PDR with the exact same traffic descriptor, the SMF copies the installed PDR that would have previously matched the incoming traffic described by the traffic descriptor in the notification subscription, but provides that traffic descriptor, a higher priority, and a buffer notification action within. If PCC is used. the SMF checks if the Policy Control Trigger Request are set to report the reception of a subscription to DDD status and then the SMF requests PCC Rules from the PCF before contacting the UPF; the PCF then provides PCC rule(s) taking into consideration the traffic descriptor for the subscribed DDN status event.

3.	The UPF reports when there is buffered or discarded traffic matching the received PDR to the SMF and the SMF sends the corresponding event report, by means of Nsmf_EventExposure_Notify message. The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

4.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

5.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.

2nd change

4.15.3.2.9	Information flow for Availability after DDN Failure with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about availability after DDN failure.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.3.2.9-1: Information flow for availability after DDN Failure event with UPF buffering

1.	AF interacts with NEF to subscribe availability after DDN failure subscription procedure as described in clause 4.15.3.2.7.

2.	If the SMF is informed the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation, the SMF provides the Traffic Descriptor in the PDR and requests the UPF to report when there are corresponding discarded packets in the UPF as described in clause 5.8.3 in 23.501 [2]. If PCC is not used and there is no installed PDR with the exact same traffic descriptor, the SMF copies the installed PDR that would have previously matched the incoming traffic described by the traffic descriptor in the notification subscription, but provides that traffic descriptor, a higher priority, and a buffer notification action within.

If PCC is used. the SMF checks if the Policy Control Trigger Request are set to report the reception of a subscription to DDN failure notifications and then the SMF requests PCC Rules from the PCF before contacting the UPF; the PCF then provides PCC rule(s) taking into consideration the traffic descriptor for the subscribed DDN failure event.

3-4.	When the UPF determines to discard DL data matching the received PDR, it reports the corresponding event to the SMF and the SMF sends the event reports the DDN Failure status with NEF Correlation ID, by means of Nsmf_PDUSession_SMContextStatusNotify Nsmf_EventExposure_Notify message, to the AMF indicated as notification endpoint. If the UE is not reachable after the AMF received the notification from the SMF, the AMF shall set a Notify-on-available-after-DDN-failure flag corresponding to the Notification Correlation Id and the identifier of the UE if available.

5-6.	[Conditional] The AMF detects the UE is reachable and sends the event report(s) based on the Notify-on-available-after-DDN-failure flag, by means of Namf_EventExposure_Notify message(s), only to the NEF(s) indicated as notification endpoint(s) identified via the corresponding subscription in step 3. In this way, only the AF(s) for which DL traffic transmission failed are notified.

7.	The NEF sends Nnef_EventExposure_Notify message with the "Availability after DDN Failure" event to AF.

End of changes