3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005864

Elbonia, August 19 – September 1, 2020

Source:	Orange

Title:	KI #6, New Sol:  Replacement of the UAV Controller of a UAS

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract of the contribution: This paper proposes a solution for the Replacement of the UAV Controller of a UAS.

# 1 Introduction

The solution gives the possibility to substitute the current UAV controller by another one. Such UAVC substitution may be of particular interest for future drone use cases, for examples:

- The UAV is under the control of an automated controller but some event (ex: anomaly detection, geofencing boundaries crossing) calls for manual control.

- The UAV is under the control of a given UAVC, but law enforcement, or any authorized third-party, wishes to take control of that drone e.g. to modify the drone trajectory.

In the proposed solution, the new UAVC is associated with the same USS.

This solution addresses the requirement "An UAV Controller can be removed from a UAS and replaced with another UAV Controller or a TPAE" (see clause 4.2), as part of Key Issues 3 and 6.

NOTE: Whereas the original version of this pCR was proposing a new solution, the revision of this pCR proposes the same procedure to be added to Solution #5.

# 2 Proposal

It is proposed to update TR 23.754 as follows.

* * * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 | X | X | X | X | X | X | X |
| 3 |  | X |  |  |  |  |  |
| 4 | X | X | X |  |  |  | X |
| 5 | X | X | X |  |  | X | X |
| 6 | X | X |  |  | X |  |  |
| 7 | X | X |  |  | X |  |  |
| 8 | X |  |  |  |  |  |  |
| 9 | X |  |  | X |  |  |  |
| 10 |  | X |  |  |  |  |  |
| 11 |  | X |  |  |  |  |  |
| 12 |  | X |  |  |  |  |  |
| 13 |  |  |  | X |  |  | X |
| 14 |  |  |  | X |  |  |  |
| 15 |  |  |  | X |  |  |  |
| 16 |  |  |  | X | X |  |  |
| 17 |  | X |  |  | X |  |  |
| 18 |  |  |  |  | X |  |  |
| 19 |  |  |  |  | X |  |  |
| 20 |  |  |  |  |  | X |  |
| 21 |  |  |  |  |  | X | X |

* * * * Next Change * * * *

NOTE: This change was not in the original version of this pCR

## 6.5	Solution #5: UAV authentication and authorization by USS/UTM based on NAS supplementary and secondary authentication and authorization procedures

### 6.5.1	Introduction

This solution addresses Key Issue #1, #2, #3, #6 and #7.

This solution is applicable to EPC and 5GS.

This solution enables a:

-	USS UAV Authorization/Authentication (UUAA) to ensure the UAV has successfully registered with USS/UTM and has therefore been authorized for operations by USS/UTM.

NOTE:	The same functionality can be applied to a networked UAV Controller.

-	Authorization of UAV and UAV controller pairing.

-	Flight path authorization/registration for flight operation.

-	Replacement of a UAV Controller by another UAV Controller in a UAS.

* * * * Next Change (all new text) * * * *

## 6.5.3.X	Replacement of the UAV Controller of a UAS

Figures 6.5.3.X-1 and 6.5.3.X-2 depict the procedures to replace the UAV Controller of a UAS, respectively for 5GS and for EPS. These procedures assume that the UAV is already registered and authorized within the 3GPP systems, as well as within the CAA/UTM system, and that this UAV is already communicating with UAV Controller 1, associated with a USS. UAV Controller 1 is replaced by UAV Controller 2, which is associated with the same USS.

Figure 6.5.3.X-1: Procedure for replacing the UAV Controller of a UAS in 5GS

- The UAV is communicating with UAV Controller 1.

- The USS sends a UAV Operation Update to the SMF (via UFES if applicable), containing new authorized UAV and UAV Controller pairing information.

- The SMF configures user plane connectivity for UAV to UAV Controller 2 communication based on information provided by the USS (e.g. policies and/or traffic filters) to enable user plane connectivity between the UAV and the UAV Controller 2 and disable user plane connectivity between the UAV and the UAV Controller 1.

- The UAV communicates with UAV Controller 2.

Figure 6.X.2.2-1: Procedure for replacing the UAV Controller of a UAS in EPS

- The UAV is communicating with UAV Controller 1.

- The USS sends a UAV Operation Update to the P-GW (via UFES if applicable), containing new authorized UAV and UAV Controller pairing information.

- The P-GW configures user plane connectivity for UAV to UAV Controller 2 communication based on information provided by the USS (e.g. policies and/or traffic filters) to enable user plane connectivity between the UAV and the UAV Controller 2 and disable user plane connectivity between the UAV and the UAV Controller 1.

- The UAV communicates with UAV Controller 2.

* * * * End of Changes * * * *