SA WG2 Meeting #S2-141E	S2-2006994

12 – 23 October, 2020, Electronic	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2462 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Bridge Management Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the Bridge Management Information (BMI) description in TS 23.501 Table 5.8.2.11.9-1 has no match in stage 3 specifications. TS 29.244 has three data structures involving signalling of NW-TT port numbers that should be considered here: - Created Bridge Info for TSC IE within PFCP Session Establishment Response - TSC Management Information IE within PFCP Session Modification Request - TSC Management Information IE within PFCP Session Modification Response Created Bridge Info for TSC IE contains multiple NW-TT port numbers and is used when there is no companion PMIC. The presence of multiple NW-TT Port Numbers in this IE has been rendered obsolete with the transfer of NW-TT port numbers into the BMIC (see approved 23.501 CR2451 in S2-2005900) and should also be deleted from TS 29.244 Table 7.5.3.6. On the other hand, Created Bridge Info for TSC IE contains a TSN Bridge ID, which is currently missing from the BMI structure in TS 23.501 Table 5.8.2.11.9-1. In contrast, the TSC Management Information IE has only one NW-TT port number and is used when there is a companion PMIC. It corresponds to the Port Management Information Container structure in TS 23.501 Table 5.8.2.11.10-1, noting though that the NW-TT Port Number is currently missing there. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.8.2.11.9: Assuming that the BMI structure in TS 23.501 Table 5.8.2.11.9-1 matches the Created Bridge Info for TSC IE in TS 29.244, it is proposed to remove NW-TT Port Number from BMI and add a Bridge ID in the BMI. Clause 5.8.2.11.10: It is proposed to add a NW-TT Port Number to the Port Management Information Container structure in TS 23.501 Table 5.8.2.11.10-1 to align it with the TSC Management Info IE in TS 29.244. The structure is renamed into “Port Management Information” (“the word “Container” is dropped out) because it is a superstructure composed of the PMIC proper, plus the corresponding NW-TT port number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned stage 2 and stage 3. The Created Bridge Info for TSC IE in TS 29.244 needs to be corrected according to this CR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.9; 5.8.2.11.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

##### 5.8.2.11.9	Bridge Management Information

The following table describes the Bridge Management Information (BMI) that includes the information required to configure a 5GS logical bridge for TSC PDU Sessions.

Table 5.8.2.11.9-1: Bridge Management Information

| Attribute | Description | Comment |
| --- | --- | --- |
| DS-TT Port Number | Port Number allocated by the NW-TT for the DS-TT for a given TSC PDU Session |  |
| Bridge ID | Bridge identifier of the 5GS TSN bridge |  |

* * * * Next Change * * * *

##### 5.8.2.11.10	Port Management Information

The following table describes the Port Management Information that includes information exchanged transparently via 5GS between TSN AF and NW-TT for TSC PDU Sessions.

Table 5.8.2.11.10-1: Port Management Information

| Attribute | Description | Comment |
| --- | --- | --- |
| Port Management Information Container (PMIC) as in Table 5.28.3.1-1 | Information exchanged transparently between NW-TT and TSN AF via 5GS |  |
| NW-TT Port Number | NW-TT Port Number corresponding to the PMIC |  |

* * * * End of Change * * * *