SA WG2 Meeting SA2-141e	S2-2007087

12 – 23 October, 2020, E-meeting	(revision of S2-2005685r19)

Source:	Intel

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

# 1	Proposal

Based on the discussion in S2-2007085 (refer to Proposals 2-6 and Slide #13) it is proposed to agree an interim conclusion on KI#1 as indicated below. Revision S2-2005685r19 (that could not be agreed during SA2#140E) is used as the basis. Yellow-highlighted text is new.

* * * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

Based on the evaluation in clause 7.1, the following interim concusions are agreed for the baseline functionality:

-	A Paging Cause for both EPS and 5GS indicating at least “voice” service and “SMS”, pending feedback from SA3 and RAN WGs.

Editor’s note: It will be further confirmed with RAN2 whether a value “other service” is needed to allow the UE to discriminate between gNBs supporting the Paging Cause from those that do not.

Editor’s note: It will be determined whether the Paging Cause is applied only for UEs that have indicated MUSIM capability or to all UEs indiscriminately.

-	A Negotiated Short Absence for 5GS, evaluated together with RRC-based coordinated leaving, pending feedback from RAN WGs.

-	A Busy indication pending feedback from RAN WGs. If Busy Indication is pursued for normative work, it is left to CT1 to determine whether a fixed or dynamically signalled Upper bound timer value needs to be used to guard the UE’s sending of Busy Indication.

-	When AMF is aware that UE has left 3GPP access using the procedure for coordinated leaving, and if the UE has a non-3GPP registration with the same AMF, the AMF shall use the NAS Notification procedure for MT service delivery.

Different Paging Cause values based on the Caller ID (as in Sol#9) will not be pursued for normative work in Rel-17.

The optimizations for “on the same network” (as in Sol#24 and Sol#26) are not pursued for normative work in Rel-17.

* * * * End of Change * * * *