3GPP TSG-SA WG2 Meeting #141 	S2-2007129

12 -23 October 2020, Electronic meeting

Source:	Ericsson

Title:	KI#1, Additional Conclusions for Multiple Anchors

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes some additions to the conclusions for KI#1 for multiple anchors.

# 1         Introduction

The evaluation section for KI#1 for Multiple anchors (section 7.1.4) includes a number of scenarios in which the solutions will not work so as a proposal for one of them. The proposal was to capture them in normative phase. This pCR proposes to add them to the Conclusions Clause.

# 2	Proposal

## **************************** Start Change ***************************

### 9.1.1	Conclusions regarding solutions for Key Issue #1 for DNS based solutions for Multiple PDU Sessions

Solution #1 proposing to Enhance the NEF service(s) (AF Requests need to be enhanced) to allow the AF to influence PCF decisions for URSP rules, is recommended for normative phase.

The recommendation to define the URSPs so that Domain Descriptors are used to steer the DNS and the Application traffic into PDU sessions is to be captured in normative phase too, so as the clarifications that are required on how the Domain descriptors in URSP are used on the UE (see clause 7.1).

The solution is not guaranteed to work when devices are doing tethering, when the UE doesn't support URSP rule provisioned from PCF, when the DNS Server at the UE has been configured by the user or when the Application Client generates the DNS Query outside of the DNS libraries in the OS with DoT, DoH or other over the top mechanisms.

To manage an application that ignores DNS settings provided by the network (e.g. when the application generates the DNS queries, or DNS server is configured manually by user), URSP rules can be configured by the operator to not map such application to a PDU session that rely on the DNS Servers being provided by the 5GC.

These limitations and informative guidelines shall be captured to cover scenarios where the OS, user or applications may override the operator-provided URSP or DNS settings. The guidelines should assume no restriction on the OS, user or application.

Editor's note:	It is FFS whether other solution like solution 13 variant on URSP is needed to define how AF can guide PCF to send proper URSP rules.

## ****************************** End Change ******************************