SA WG2 Meeting #140E	S2-2007238

Electronic meeting, Oct 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#6 Update of Solution #21

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper updates solution #21 to detail handover aspects.

# 1	Discussion

The Handover procedures of solution 21 need to be updated to consider a change of the location area while the UE moves and to align with the latest flows of the baseline solution 27.

# 2	Conclusions

Proposal: Agree the proposed changes to Solution #21.

Proposed changes to TR 23.757

#### 6.21.2.3	Enhancement on Handover procedure

NOTE:	For simplification, this figure is based on Solution #27 and only address the Xn based Handover. It can be extended to N2 based Handover and other Handover Solutions if needed.

Figure 6.21.2.3-1: Enhancement on Handover procedure.

Prereouesit:The UE is camping at Source RAN (S-RAN) and receiving multicast data a corresponding to the multicast session ID and location area ID.

1.	Same as in clause 6.27.2.1

2.	Same as in clause 6.27.2.1: S-RAN sends Handover request to T-RAN with including multicast session ID, location area ID and location area.

3-7.	T-RAN determines whether to establish the multicast distribution for multicast session ID and location area ID provided by S-RAN, based on multicast session ID, area session ID and location area.. If a multicast distribution session needs to be established, T-RAN provides multicast session ID and location ID in step 3

8.	T-RAN sends the Handover request Ack to S-RAN, with the accepted multicast session ID and area session ID. When the T-TAN supports multicast but the UE is no longer in the location area indicated in step 2, the T-RAN rejects the handover request with a cause indication that the UE moved out of the location area and the new UE location..

Steps 9 to 20 only apply if the UE moved out of the location area and target gNB supports multicast.

9-10	The S-RAN indicates the new UE location to the SMF via the AMF.

11.	The SMF determines the location area, area session ID and MB-SMF ID for the new UE location. It may query the UDR for that information.

12-13.	The SMF sends the new location area, area session ID and MB-SMF ID to the S-RAN via the AMF.

14-20.	Same as steps 2-7, but with new MB-SMF ID, area session ID and location area.

21-42.	Same as steps 9-29 in clause 6.27.2.1. In steps 29, 35 and 36, the area session ID is indicated in addition to the multicast session ID.

### 6.21.3	Impacts on services, entities and interfaces

Same as in clause 6.3.3. Location area Identifier needs to be stored and signalled in addition.