# 3GPP TSG SA2 Meeting #141E	S2-2007619

# October 12 – 23, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO

Title:	KI#3B: Sol#9: BMCA procedure

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This paper clarifies the BMCA procedure in Solution #9.

# 1		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 6.9	Solution #9: (g)PTP GM support by DS-TT

### 6.9.1	Introduction

Figure 6.9.1-1: DS-TT distributes 5GS time to devices attached to DS-TT using (g)PTP

This solution addresses KI#3B. As illustrated in Figure 6.9.1-1, key idea of this solution is that DS-TT acts as a (g)PTP grand master (GM) for devices attached to DS-TT. The time information used in (g)PTP is based on the time information received from the gNB as defined in TS 38.331 [11].

### 6.9.2	Functional Description

The solution is based on the following principles

-	DS-TT indicates to the network inside a Port Management Information Container (PMIC) whether DS-TT is capable of acting as (g)PTP GM and which version(s) it supports

-	IEEE 802.1AS [6] (i.e. gPTP), and/or

-	PTP over UDP/IPv4 as per IEEE 1588-2008 [13] Annex D, and/or

-	PTP over UDP over IPv6 as per IEEE 1588-2008 [13] Annex E.

-	If DS-TT has indicated that it is capable of acting as (g)PTP GM, then the network may request the DS-TT to generate (g)PTP messages to devices attached to the UE by providing (g)PTP parameters (incl. (g)PTP version, domain number(s), sending rate, etc.) and an instruction to start sending (g)PTP messages inside PMIC to the DS-TT.

NOTE:	DS-TT can be configured to support different (g)PTP profiles, e.g. SMPTE ST 2059-2 [9] or IEC/IEEE 61850-9-3 [10] by providing the (g)PTP parameters defined for those profiles to DS-TT.

### 6.9.3	Procedures

For IEEE TSN integration, i.e. if TSN AF is deployed, TSN AF controls the gPTP functionality in DS-TT using existing Port Management Information Container (PMIC) signaling between DS-TT and TSN AF.

Otherwise (i.e. for IP or Ethernet PDU Sessions not part of an IEEE TSN network), the NEF terminates the PMIC signaling instead of TSN AF. To achieve this, NEF generates PMIC signaling to control the (g)PTP client in DS-TT based on time synchronization API requests from an AF and sends PMIC to DS-TT using existing Rel-16 PCF functionality. Similarly, NEF receives PMIC from PCF using Rel-16 functionality.

It is assumed that a single NEF is handling all time synchronization API requests for a given UE.

Existing Port Management Information Container (PMIC) signalling is reused between DS-TT and the network.

The following figure illustrates an overview of the procedure to activate (g)PTP grandmaster functionality in DS-TT.

Figure 6.9.3-1: Procedure for synchronizing (g)PTP clients attached to DS-TT with the 5G clock

1.	The AF activates the time synchronization service as described in Solution #7. Upon PDU Session establishment, the DS-TT indicates to the NEF (or TSN AF) inside a Port Management Information Container (PMIC) whether DS-TT is capable of acting as (g)PTP GM and which version(s) it supports. NEF receives PMIC from PCF using Rel-16 functionality.

The NEF generates PMIC signaling to control the (g)PTP client in DS-TT based on time synchronization API requests from an AF and sends PMIC to DS-TT using existing Rel-16 PCF functionality. The NEF includes the GM attributes (e.g. grandmaster identity, grandmaster quality) as described in IEEE 802.1AS [6] or IEEE 1588-2008 [13] to the PMIC. The NEF informs the NW-TT in BMIC that the grandmaster is active in DS-TT(s). The NEF informs the NW-TT in BMIC with the GM attributes (default dataset) that are sent by the DS-TT(s) in the time-synchronization event messages if the grandmaster in DS-TT(s) is active. The NEF informs the NW-TT in BMIC also if the grandmaster is deactivated in DS-TT(s).

2.	DS-TT/UE receives the time information from the gNB as defined in TS 38.331 [11].

3.	 DS-TT generates the (g)PTP events based on the time information received from the gNB, and based on the GM attributes received in the PMIC.

BMCA procedure

DS-TT generates the Announce messages as described in IEEE 802.1AS [6] or IEEE 1588-2008 [13]. Upon reception of Announce message from a device behind the DS-TT, if the Announce is better than the current Announce of the grandmaster in DS-TT, the DS-TT forwards the Announce message to the NW-TT for the execution of the BMCA. If the NW-TT determines that due to the result of the BMCA the (g)PTP grandmaster must be deactivated in DS-TT(s), the NW-TT reports the BMCA result to NEF using BMIC signalling.  The details of the BMCA function in NW-TT and NEF (or TSN AF) are described in Solution #1.

### 6.9.4	Impacts on services, entities and interfaces

DS-TT:

-	Optionally send (g)PTP capabilities; receive (g)PTP parameters.

-	Optionally generate (g)PTP messages based on network request.

NEF:

-	Support Rel-16 PMIC signaling via PCF.

*** End of Changes ***