SA WG2 Meeting #141E (e-meeting)	S2-2007713

Elbonia, October 12 – 23, 2020

Source:	vivo Mobile Communications Ltd

Title:	Interim conclusion for Key Issue#2: Clear identification TS 23.287 [5] clause 5.2  is being extended with new data types.

Document for:	Discussion/Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe/Rel.17

Abstract of the contribution: This contribution adds clarity to KI#2 that TS 23.287 [5] clause 5.2 is being extended with new data types

# Background

KI#2 is as follows:

## 5.2 Key Issue #2: Support for NR PC5 ProSe communication ### 5.2.1 General description In TS 22.261 [3] and TS 22.278 [2], use cases and requirements for ProSe services are described, including public safety and interactive services. NR PC5 ProSe communication including unicast and groupcast needs to be supported for the case of public safety and interactive service. Following assumptions apply to this key issue: - For one-to-many direct communication and one-to-one direct communication differences with what is documented in TS 23.287 [5] clause 5.2 will be documented; - For QoS support differences with what is documented in TS 23.287 [5] clause 5.4 will be documented.

It is clear that one aspect of this KI to study was investigate and document differences to procedures documented in TS 23.287 [5] clause 5.2.  These where captured in a number of solutions and conclusion was made in S2-2006555. However the conclusions of the TR are not totally clear, there is no bullet item in KI#2 that talks about “ProSe direct communications” (1st bullet of the conclusions), in fact the KI#2 identifies the different types of direct communications.

Proposal:	Add text to state that TS 24.287 clause 5.2 is baseline with the following new data types.

>>>>Start Changes<<<<

## 8.2	Key Issue #2: Support for NR PC5 ProSe communication

For Key Issue #2 (Support for NR PC5 ProSe communication):

-	Regarding ProSe direct communication, it is concluded to support the mechanism defined in TS 23.287 [5] clause 5.2 with the following the new data unit types (including IPv4, ARP, Ethernet and Unstructured) identified in Solution#5 (clause 6.5) by coordinating with RAN WG.

-	Regarding PC5 QoS support, it is concluded that Solution #20 in clause 6.20 and Solution #21 in clause 6.21 are taken as the baseline for normative work. Whether the new defined PQI values can be supported needs to be confirmed by RAN WGs.