3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009167

Elbonia, November 16 – 20, 2020	(revision of S2-2009015r04)

Source:	Nokia, Nokia Shanghai Bell, CATT

Title:	KI #2: Solution #55 Removal of FFS

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper updates solution #55 and removes FFS added during SA2#141E e-meeting

# Discussion

This paper removes below two FFS by providing updates to corresponding procedure and additional details:

Editor's note:	Whether the AF2 can reuse the SBI information of AF1 to invoke NEF service is FFS.

Editor's note:	The usage of the indication and how it is received are FFS.

This paper also updates some other text, in order to better clarify the use case, and as an editorial changes.

# Proposal

It is proposed to update below in the TR 23.748.

* * * * Start of first change * * * *

## 6.55	Solution #55: Multiple AFs

### 6.55.1	Description

This solution addresses scenarios where Edge Application Server(s) and Application Function are located at the Edge Data Network, thus implying multiple AF instances i.e. one (or more) local AF(s) and also a central AF. Application Function here acts as control plane entity managing one or more EAS and responsible to interact with 5GC. In such deployment scenarios: as part of initial PDU session establishment, a central AF may be involved, however, due to relocation to Edge DNAI and corresponding EAS relocation, a new local AF serving the Edge Applications may be selected.

There are a few additional details that needs to be addressed in such multiple AF deployments, such as: how to relocate AF, how SMF provides to new AF required notification of user plane management events such as PSA relocation, etc.

### 6.55.2	Procedures

Detail procedure involving multiple AF scenarios are provided in the below figure 6.55.2-1.

Figure 6.55.2-1: Procedures to AF Relocation

0.	AF1 is interacting with SMF/5GC e.g. as part of initial PDU Session establishment procedure.

1.	AF1 decides to relocate to local AF i.e. to AF2 which serves local EAS located in the target DNAI. There could be multiple triggers for AF1 to relocate AF e.g. due to decision to relocate to local /another Edge Application Server due to change in DNAI, EAS load balancing, etc.

2. AF1 initiates AF relocation information transfer with AF2, this is to provide to AF2 information such as: application status, UE ID, PDU session Information (e.g. the UE (IP) address, the current DNAI serving the UE, N6 traffic routing information), etc. Any application context transfer details are out of scope of SA2. Subsequently, AF1 may cancel its earlier request by Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

3a:	AF2, based on the information received from AF1, sends via NEF, Nnef_TrafficInfluence_Create request to the UE ID provided by AF1.  It includes AF Transaction ID, UE ID, PDU session information, DNN, S-NSSAI if available and/or received from AF1.

3b. Alternatively, AF1 sends to NEF, Nnef_TrafficInfluence_update request, and includes application status, the target AF information (e.g. AF2 ID, IP address), UE ID.

4:	NEF stores the AF1/AF2 requested information in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID, S-NSSAI and DNN or SUPI).

5:	NEF acknowledges to AF1/AF2 by sending Nnef_TrafficInfluence_update response message.

6.	UDR notifies to PCF with Nudr_DM_Notify of the changed subscription.

Steps 4 to 6 are per R16 specifications

7.	PCF determines that existing PDU Session is impacted by the AF1/AF2 request, and so it updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

8.	Based on the updated PCC rules received from PCF, SMF may decide to reconfigure user plane e.g. change in DNAI, PSA relocation and so on.

9-10.	SMF sends Nsmf_EventExposure_Notify indicating the changes in the user plane management information to AF1/AF2, based on information received in step 7, either directly or via NEF.

11. The AF1 transfers the UP management information to AF2, as receives in step 9-10 (if the target of notification is AF1).

### 6.55.3	Impacts on services, entities and interfaces

SMF:

-	Updates to procedure on AF requests to influence traffic routing.

AF:

-	AF relocation information transfer between AFs e.g. UE ID, PDU session Information, current DNAI serving the UE, etc.

-	determination of the target AF for the received target DNAI.

-	Updates to procedure on AF requests to influence traffic routing.

* * * * End of Change * * * *