SA WG2 Meeting #142E (e-meeting)	S2-2008564

November 16 – November 20, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	KI #1, conclusion update

Document for:	Approval

Agenda Item:	8.10 Study on system enablers for multi-USIM devices

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This paper proposes KI #1 conclusion update.

Background

Proposal

It is proposed to capture the following proposal to TR 23.761.

* * * * Start of 1st Change * * * *

8.1	Conclusions for Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

The following conclusions are agreed for the baseline functionality:

-	Support of Paging Cause(s)

-	Paging Cause(s) are supported in EPS and 5GS with following values:

-	Voice service

-	SMS service

-	Other services

-	NAS signalling

-	The network provides Paging Cause(s) to all UEs indiscriminately.

-	The AMF, MME and RAN determines Paging Causes(s) based on Option 2 in Solution #1.

-	Support of BUSY indication via NAS signalling

-	BUSY indication is supported in EPS and 5GS

-	A UE should send BUSY indication when the UE received paging and decides to keep staying in the other network.

-	Support of Paging Filtering

-	Paging Filtering is supported in EPS and 5GS

-	A UE can update Paging Filtering information via Registration or Service Request procedure

-	The Paging Filtering information is represented by

-	An indication that the UE wants to be paged for All service

-	An indication that the UE wants be paged at all

-	PDN connection / PDU Session ID

-	Paging Cause

-	The UE MMI shall not require input from the user in order for the UE to decide whether to respond to paging.

* * * * End of Change * * * *