3GPP TSG-WG SA2 Meeting #142E e-meeting	S2-2008682

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Samsung

Title:	KI#9, update on conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes to update conclusion on KI #9.

Discussion

It is desirable to not modify existing mechanisms to support interworking b/w NR/5GC and E-UTRAN/EPC. So we propose to have service level based solution only.

Proposal

It is proposed to add the following changes into TR 23.757.

******************************** START OF CHANGES*********************************

8.x	Key Issue #9: Minimizing the interruption of public safety services upon transition between NR/5GC and E-UTRAN/EPC

8.x.1	conclusions

The following agreements apply as baseline for normative work to enable support PS service continuity between 5MBS (NR/5GC) and MBMS(E-UTRAN/EPC):

- Service level-based solution shall be supported.

- BMSC for LTE and MBSF for 5GC are collocated and shared for service level interworking.

- An MBS service and session is identified by the same TMGI between 5MBS and MBMS.

******************************** END OF CHANGES **************************************