3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100837

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	5G core NF service and service operations for UAS

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract: This contribution introduces service operations relevant to UAS.

1. Introduction/Discussion

R17 ID_UAS topic starts normative work, and according to the TS skeleton, clause 4.4.2 is for the description of UAS impact on 5G core service operations.

This contribution proposes contents for this aspect.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.256.

# * * * * First change * * * *

### 4.4.2	Network Function Services

4.4.2.X	NEF Services

4.4.2.X.1	General

In addition to those defined in TS 23.501[2] clause 7.2.8 and TS 23.502[3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.X.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | UAVauthenticate | Request/Response | AMF, SMF |

4.4.2.X.2	Nnef_Authentication service

4.4.2.X.2.1	General

Service Description: This service enables the consumer to authenticate the UE. In case of UAS, the service is used to authenticate the UAV.

4.4.2.X.2.2	Nnef_Authentication_ UAVauthenticate service operation

Service operation name: Nnef_Authentication_ UAVauthenticate

Description: provides the UTM authentication result of the UAV.

Input, Required: CAA-level UAV ID, SUPI.

Input, Optional: USS/UTM address, PEI, UAV IP address (in case the consumer NF is SMF), authentication container provided by UAV.

Output, Required: Success/Failure indication

Output, Optional: none.

# * * * * Second change * * * *

4.4.2.Y	AF Services

4.4.2.Y.1	General

In addition to the AF services defined in TS 23.501[2] clause 7.2.19 and TS 23.502[3] clause 5.2.19, the following table shows the new AF services to support UAS.

Table 4.4.2.Y.1-1: NF Services provided by AF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_Authentication | UAVauthenticate | Request/Response | NEF |

4.4.2.Y.2	Naf_Authentication service

4.4.2.Y.2.1	General

Service Description: This service enables the consumer to authenticate the UE. In case of UAS, the service is used to authenticate the UAV.

4.4.2.Y.2.2	Naf_Authentication_ UAVauthenticate service operation

Service operation name: Naf_Authentication_ UAVauthenticate

Description: provides the UTM authentication result of the UAV.

Input, Required: CAA-level UAV ID, GPSI.

Input, Optional: PEI, UAV IP address, authentication container provided by UAV.

Output, Required: Success/Failure indication

Output, Optional: none.

# * * * * Third change * * * *

4.4.2.a	AMF Services

AMF services related to UAS are defined in TS 23.502 [X] clause 5.2.2.

In addition, when SMF invokes Namf_Communication_N1N2MessageTransfer service operation, it may provide the UUAA result to the UAV.

# * * * * Fourth change * * * *

4.4.2.b	SMF Services

SMF services related to UAS are defined in TS 23.502 [X] clause 5.2.8.

In addition, when AMF invokes Nsmf_PDUSession_CreateSMContext service operation, AMF may provide an indication to indicate “UAV has been authenticated by the UTM”, if the UUAA has taken place during registration procedure by the AMF.

# * * * * Fifth change * * * *

4.4.2.c	UDM Services

UDM services related to UAS are defined in TS 23.502 [X] clause 5.2.3.

In addition, in the response of Nudm_SDM Get service operation, UDM may provide the “UAV UE type” to the NF consumer.

# * * * * Sixth change * * * *

4.4.2.d	LMF Services

LMF services related to UAS are defined in TS 23.273 [Y] clause 8.3.

# * * * * Seventh change * * * *

4.4.2.e	GMLC Services

GMLC services related to UAS are defined in TS 23.273 [Y] clause 8.4.

# * * * * Eighth change * * * *

4.4.2.f	UDR Services

UDR services related to UAS are defined in TS 23.502 [X] clause 5.2.12.

# * * * * Ninth change * * * *

4.4.2.g	PCF Services

PCF services related to UAS are defined in TS 23.502 [X] clause 5.2.5.

# * * * * End of changes * * * *