3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2101066

Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-2100573)

Source:	Huawei, HiSilicon

Title:	KI#1, conclusions updates for AAA case

Document for:	Approval

Agenda Item:	8.2.1

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces updates for conclusions regarding AAA case.

1. Introduction/Discussion

This contribution aims at addressing the following ENs, besides that, it also proposes to apply the conclusion regarding SNPN selection for UEs with SNPN subscription to AAA case.

Editor's note:	Need for and details of using a UE ID other than the SUPI are FFS.

Editor's note:	It is FFS if the AAA server supports providing the subscription information needed for registration and session management procedure.

On>> Need for and details of using a UE ID other than the SUPI are FFS

The user identity used to identify the UE during primary authentication and authorization towards the AAA sever can be an AAA-UE-ID different from the SUPI used within the 5GC SNPN. The AAA-UE-ID is derived from the SUPI/SUIC used within the 5GC SNPN in the case that the SUPI/SUCI is generated by UE based on the AAA-UE-ID or provided by UE explicitly.

Proposal 1: address this EN with the above clarification.

On>> It is FFS if the AAA server supports providing the subscription information needed for registration and session management procedure.

Solution#4, #8 support that AAA-P/AUSF provides the subscription data for mobility management and session management, in this case, the AMF or SMF obtains the subscription data from AAA-P/AUSF using SUPI.

Solution#4, #8 and #44 support the UDM provides the subscription data for mobility management and session management, in this case, the AMF or SMF obtains the subscription data from AAA-P/AUSF using SUPI.

Proposal 2: address these ENs with the above clarification.

Besides that, regarding SNPN selection for UE(s) with credentials owned by separate entity offering AAA Server, conclusions specified for UEs with SNPN subscription should be used as much as possible.

Proposal 3: SNPN selection for UE(s) with credentials owned by separate entity offering AAA Server should follow the conclusion of SNPN selection for UE(s) with UEs with SNPN subscription.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 V1.2.0.

# * * * * First change * * * *

8.1.1	Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server

The scenario where the separate entity owning the credentials has only an AAA Server is supported. It is proposed to adopt the following conclusion principles:

-	The SNPN will host a function (e.g. enhanced AUSF or new NF) supporting primary authentication and authorization of SNPN UEs that use credentials from the AAA Server.

NOTE 1:	Whether to use a new NF or enhanced AUSF will be determined based on feedback from SA WG3.

-	The function (see above) forwards EAP messages to the AAA server that is acting as the EAP Server for the purpose of primary authentication and authorization.

-	The user identity used to identify the UE during primary authentication and authorization towards the AAA sever is the same SUPI as used within the 5GC SNPN.

Editor's note:	Need for and details of using a UE ID other than the SUPI are FFS.

-	The AMF discovers and selects the AUSF as described in TS 23.501 clause 6.3.4 using the Home Network Identifier and Routing Indicator present in the SUCI provided by the UE.

-	The AUSF (e.g. enhanced AUSF or new NF) in SNPN selects the AAA hosted by the external entity. The discovery and selection functionality (within the AUSF) uses the domain name corresponds to the realm part (specified in the NAI format for SUPI in clauses 28.15.2 and 28.16.2)  within Network specific identifier that identifies the external entity to perform discovery and selection of the AAA hosted by the external entity.

-	The UDM supports mobility management and session management subscription information that is needed at the time of Registration and Session Management procedure.

Editor's note:	It is FFS if the AAA server supports providing the subscription information needed for registration and session management procedure.

-	SNPN selection for UE(s) with credentials owned by separate entity offering AAA Server is the same as the conclusion of SNPN selection for UE(s) with SNPN subscription as described in clause 8.1.4.

-	After completing the primary authentication and authorization successfully, the AMF or SMF can retrieve the SNPN UE subscription data (i.e. mobility management and session management subscription) using SUPI from UDM.

NOTE 2:	The interactions or interfaces between the existing/enhanced AUSF (or new NF) and the AAA server can be defined by SA2 during normative phase e.g. based on SA3's feedback.

# * * * * End of changes * * * *