SA WG2 Meeting #143e	S2-2100228

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-2100228)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2477 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SRVCC procedure in case of PDN connections without TI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS on E-RABs that cannot be handed over to 2G/3G or 5G (R3-211273 / S2-2100108) |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 5G SRVCC to 3G it is described in 23.501 § 5.17.2.4 “Mobility between 5GS and GERAN/UTRAN” that “After the 5G SRVCC to UTRAN, all the PDU sessions of the UE are released”; The same behaviour may take place at the indirect mobility 5G 4G (SRVCC) 3G The MME issues a Forward Relocation Request message to the SGSN and only includes those EPS Bearer/PDP contexts that are associated with a TI. No impact is thus expected to the ENB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure of the SRVCC procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.x (new) ; 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE all text is new

#### 4.11.0a.x	Impacts to SRVCC procedures from EPC.

A UE that has moved from 5GS to EPS may later be subject to a SRVCC procedure from EPS as defined in TS 23.216 [xx]. When such a SRVCC procedure takes place, the MME issues a Forward Relocation Request message to the SGSN and includes only those EPS Bearers/PDP context that are associated with a Transaction Identifier (TI, see TS 23.401 [13]) .

After the SRVCC procedure, the UE and the MME locally release any EPS Bearers for which no TI  had been communicated to the UE by the network.

NEXT CHANGE (2)

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[4]	Void.

[5]	Void.

[6]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[7]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[8]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[9]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[10]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[11]	Void.

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	Void.

[15]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[17]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	Void.

[20]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System ".

[21]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[22]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[23]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[24]	3GPP TS 23.203: "Policy and charging control architecture".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[27]	Void.

[28]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[29]	Void.

[30]	Void.

[31]	Void.

[32]	3GPP TS 29.507: "Access and Mobility Policy Control Service; Stage 3".

[33]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[34]	Void.

[35]	3GPP TS 23.251: "Network sharing; Architecture and functional description".

[36]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[37]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[38]	3GPP TS 23.380: "IMS Restoration Procedures".

[39]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[40]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[41]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[42]	3GPP TS 32.290: "Services, operations and procedures of charging using Service Based Interface (SBI)".

[43]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[44]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[45]	3GPP TS 32.255: "5G system; 5G data connectivity domain charging; Stage 2".

[46]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[47]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[48]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012): "IEEE Standard for Information technology - Telecommunications and information exchange between systems Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[49]	IETF RFC 2410: "The NULL Encryption Algorithm and its use with IPsec".

[50]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services; Stage 2".

[51]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[52]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[53]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[54]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[55]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[56]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[57]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[58]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[59]	IETF RFC 6696: "EAP Extensions for the EAP Re-authentication Protocol (ERP)", July 2012.

[60]	IETF RFC 5295: "Specification for the Derivation of Root Keys from an Extended Master Session Key (EMSK)", Aug. 2008.

[61]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[62]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[63]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[64]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[65]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[66]	IEEE Std 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks-Bridges and Bridged Networks".

[67]	Void.

[68]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration".

[69]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[70]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[71]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[72]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[73]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[xx]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

END OF CHANGES