3GPP TSG-WG SA2 Meeting #148E e-Meeting	S2-2108546

# Elbonia,  Nov 15 - Nov 22, 2021													   (revision of S2-2107539)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0022 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local NEF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In MEC, the local NEF is for fast QoS monitoring event. When AF triggers a request for QoS, the NEF will firstly receives the request, and may redirect the request to a local NEF. And some parameters are used in selection of local NEF in NRF, how does NEF to get these parameters are FFS, for example, the local PSA location, UE location and etc. Also, due to the QoS monitoring is the only event for edge, and in order to reduce the interaction between NEF and NRF (to allow NEF to identify which AF requests need to look for local NEFs and which need not), some indication should be added in AF request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Firstly, when AF request QoS establishment to NEF, AF can add an indication, or define a separate edge event for AF(i.e.: QoS monitoring for edge), the NEF can select a local NEF according to the new AF request or the indication. Secondly, give the procedure of how NEF gathers the parameters for local NEF selection, via BSF or UDM to get UE location. At last, the buffer of local NEF with mapping table of UE location, AF location and etc are added, to reduce the long procedure of local NEF discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How does the NEF get the parameters of local NEF selection is unclear, and NEF didn’t know whether a local NEF selection is needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

#### 6.4.2.2	Local NEF Discovery

As specified in TS 23.501 [2] clause 6.2.5.0, the NRF may be used by the AF to discover the L-NEF. To become discoverable, the L-NEF registers with an NRF deployed within the operator's domain where the AF resides.

The AF uses existing procedures as described in TS 23.502 [3], clause 4.17.4 to discover the L-NEF. If the AF only knows the NEF and it initiates a service operation towards this NEF, e.g. a Nnef_AFSessionWithQoS_Update_request procedure, the NEF may re-direct the request to an L-NEF. NEF may use NRF to find a suitable L-NEF and re-direct AF to the L-NEF.

Figure 6.4.2.2-1: NEF redirecting an AF to a more suitable local NEF

0.	The local NEF registers itself into NRF by Nnrf_NFManagement_NFRegister. The parameters for local NEF registration are referred to clause 5.2.7.2.2 in TS 23.502[3].

1.	PDU Session Establishment (as defined in TS 23.502 [3] clause 4.3.2.2.1 or clause 4.3.2.2.2). During the SM Policy Association Establishment procedure (e.g. at step 7 of Figure 4.3.2.2.1-1), when the PCF registers to the BSF (Nbsf_Management_Register), the PCF provides in Nbsf_Management_Register following information:  UE address(es), SUPI, GPSI, DNN, S-NSSAI.

2.	The AF sets up an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3] further providing the parameters, as specified in TS 23.501 [2] clause 6.3.14 and includes a local QoS monitoring subscription or an indication, that the request to target to a local NEF. This AF request is addressed to an initial NEF.

NOTE:	The initial NEF may be a central NEF or a local NEF that may have become unsuitable due to UE mobility

The indication or local QoS monitoring subscription that request to target a local NEF may trigger the initial NEF to look for a (more) suitable NEF/local NEF per the rest of this procedure.

The initial NEF first determines the target (DNN, S-NSSAI): if the target (DNN, S-NSSAI) is not provided in the AF request, the initial NEF determines the target (DNN, S-NSSAI) based on the identity of the requesting AF.

If the initial NEF knows the locality of the AF, the initial NEF issues a Nnrf_NFDiscovery_Request request targeting a NEF and providing as discovery parameters for a more local NE, e.g. Location or DNAI. The initial NEF selects a target NEF and  the procedure then proceeds to step 8. The Initial NEF can get the location of the AF if the AF registers to NRF (a trusted AF) or based on local configuration, e.g. using AF identity

Note:	The NEF can cache results to be used for later L-NEF discoveries.

4.	The initial NEF issues a Nbsf_Management discovery procedure: the initial NEF provides as input the UE address and the target (DNN, S-NSSAI) and gets as a result the SUPI of the UE.

5	The NEF issues a Nudm_UECM_Get service operation to get the SMF ID of the SMF serving the PDU Session providing the SUPI of the UE and the (DNN, S-NSSAI)

6.	The initial NEF initiates Nsmf_EventExposure_Subscribe to the SMF indicated by the UDM in step 4: the initial NEF provides as input the SUPI and the target (DNN, S-NSSAI) and gets  the DNAI; The NEF may use DNAI to discover a UPF in NRF. UPF may not be regustered in NRF.

7.	If the initiatl NEF has not registered the received DNAI in its NF profile, the initial NEF issues a Nnrf_NFDiscovery_Request to discover a NEF/local NEF with the parameter: the target S-NSSAI, DNAI and potentially Location of PSA UPF if known. Else, the procedure ends here,

8.	The NRF answers (Nnrf_NFDiscovery_Request response) with suitable target local NEF(s) NF profile(s).

9.	The initial NEF determines it is not the most suitable NEF to handle the AF request and redirects the AF towards a (more suitable) local NEF.

* * * End of changes* * *