SA WG2 Meeting #S2-149E 	S2- 2200377

14 - 25 February, 2022, Electronic meeting

Source:	China Mobile

Title:	New Key Issue: NSAC enhancement to support multiple NSACFs shared maximum allowed number of UEs or PDU sessions

Document for:	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_Ph3 /Rel-18

Abstract of the contribution: This contribution proposes a new key issue for WT#7

- Discussion

This paper is to propose a new key issue according to the WT#7 of FS_eNS_Ph3.

- Proposal

It is proposed to add the following key issue about WT#7 to TR 23.700-41.

WT#7: Study whether and how to enhance the support of NSAC when more than one NSACF is involved in enforcing a shared maximum allowed number of the UEs or PDU Sessions for a network slice in one PLMN or in roaming, in order to avoid fragmentation of the shared maximum allowed number.

************************ 1st Change, all new text************************

5.X	Key Issue #X: NSAC enhancement to support multiple NSACFs shared maximum allowed number of UEs or PDU sessions

5.X.1	General description

There could be multiple NSACFs associated with one network slice.In R17, the operator needs to split the maximum allowed number of PDU sessions or UEs for the Network Slice and configure these numbers in each NSACF. This has a high requirement for the operators that they should have precise estimations about the distribution of the users.

This key issue studies how to enhance the support of NSAC when more than one NSACF is involved for one network slice. And these NSACFs share the maximum allowed number of the UEs or PDU Sessions in one PLMN or in roaming.

-	How to dynamically assign the maximum allowed numbers of the UEs or PDU Sessions for a network slice to each NSACF;

-	How to expose the maximum allowed numbers of the UEs or PDU Sessions for a certain NSACF;

-	How to support the service continuity when UE moves from one NSACF to another NSACF;

********************************* End Change ****************************