3GPP TSG-SA2 Meeting #149E(e-meeting)	S2-2200468

# Elbonia, Feburary 14 - 25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0080 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS FAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1. MBS XXX ID In SA2#148E, for service continuity for broadcast MBS session in multi-carrier deployment, MBS XXX ID was temporarily used as the introduction of the ID in RAN SIB message. And there is an EN left: Editor's note: The name MBS XXX ID is temporary and FFS. S2-2108394 and S2-2108668 proposes MBS SAI (Service Area Identity or Serving Area Identity), while S2-2108965 proposes BFSID (Broadcast Frequency Selection Identifier). From network perspective, MBS XXX ID needs to be planned and pre-configured in RAN by operators, which is area related. From UE perspective, MBS XXX ID is received in service announcement and/or SIB, which is used for cell/frequency seletion, that is, MBS XXX ID is also frequency related. Also the common understanding is that Service/Serving Area Identity is an identity representing (MBS) service/serving area, but this is not the case for MBS SAI, therefore it is proposed to use MBS FAI (Frequency Area Identity) instead of MBS SAI. Considering the flexibility of planning the MBS FAI, it is proposed to define its length to 3 octets which is also the length of TAC. #2. How MBS FAI can be determined? As described in clause 6.5.4, “MBS XXX ID identifies a preconfigured area”, which means that MBS XXX ID (proposed to be replaced by MBS FAI in this paper) may be determined by the MBS service area. In case the broadcast is to be done on multiple frequencies (e.g. one carrier for public safety and another one for TV), the MBS service area is not sufficient. In this case, the MBS service information needs to be taken into consideration. It is also possible that AF has the MBS FAI information for the MBS broadcast session it is going to create due to business agreement with operator. #3. Which NF determine the MBS FAI? MBS FAI mapping in 5GC is done via OAM. It is possible that either NEF or MB-SMF performs the mapping, however NEF approach would mean mandatory NEF in the deployment which may not be desired. The MB-SMF determining MBS FAI (if not provided by the AF) seems more flexible in deployment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1. Introduce MBS FAI as an abbreviation in clause 3.2 #2. Remove EN and use MBS FAI to replace “MBS XXX ID” and complete the description in clause 6.5.4 #3. Include MBS FAI in broadcast MBS session context in clause 6.9.1 #4. Include MBS FAI as optional input parameter (when AF provides) and optional output parameter (when MB-SMF determines) in Nmbmsf_MBSSession_Create/Update in clause 9.1.3.6/9.1.3.7 and Nnef_MBSSession_Create/Update in clause 9.4.3.2/9.4.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service continuity in multi-carrier deployment may not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.5.4, 6.9.1, 9.1.3.6, 9.1.3.7, 9.4.3.2, 9.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

FEC	Forward Error Correction

CDN	Content Delivery Network

MBMS	Multimedia Broadcast/Multicast Service

MBS	Multicast/Broadcast Service.

MB-SMF	Multicast/Broadcast Session Management Function.

MBSF	Multicast/Broadcast Service Function.

MBSTF	Multicast/Broadcast Service Transport Function.

MBS FAI	Multicast/Broadcast Service Frequency Area Identity

MB-UPF	Multicast/Broadcast User Plane Function

PTM	Point To Multipoint

PTP	Point To Point

SSM	Source Specific IP Multicast address.

LL SSM	Lower Layer SSM

TMGI	Temporary Mobile Group Identity

****************** NEXT CHANGES ***************

### 6.5.4	MBS FAI

The MBS FAI (Frequency Area Identity) is used for broadcast MBS session to guide the frequency selection of the UE. Its length is 3 octets.

MBS FAI identifies a preconfigured area within which the cell(s) announces the MBS FAI and the associating frequency (details see TS 38.300 [9]). MBS FAI and their mapping to frequencies are provided to RAN nodes via OAM.

Based on this configuration, RAN nodes announce in SIBs over the radio interface information about the MBS FAIs and frequencies.

The mapping from MBS service area and optional broadcast MBS session information (e.g., application ID) to MBS FAI is performed in the MB-SMF based on configuration. When a broadcast MBS session is created, AF may provide MBS FAI based on the business agreement. If the AF does not provide MBS FAI, the MB-SMF may determine MBS FAIs based on configuration and sends them to the AF (optionally via NEF).

During MBS Session Start for Broadcast in clause 7.3.1 and MBS Session Update for Broadcast in clause 7.3.3, the MB-SMF may include the MBS FAIs and sends them to the NG-RAN nodes via the AMF, and then the NG-RAN nodes use the MBS FAIs to determine the cells to broadcast MBS session data. Details see TS 38.300 [9] and TS 38.413 [15].

The MBS FAI(s) of a broadcast MBS session are communicated in the service announcement (see clause 6.11) towards the UE. The UE compares those MBS FAI(s) with the MBS FAI(s) in SIBs for frequency selection.

****************** NEXT CHANGES ***************

### 6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | AMF | SMF | MB-SMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive' or 'Configured') | X (note 2) |  | X (note 2) | X |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  |  | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. | X | X | X |  |
| QoS information | QoS information of the MBS session. | X |  | X | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) |  | X (note 1) | X (note 1) |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the multicast session |  | X |  |  |
| AMF | The AMF(s) which are selected for the MBS session | X |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) |  | X (note 1) | X (note 1) |
| SMF | The SMF(s) that manages the associated PDU session. |  |  |  | X |
| UE ID | ID identifying the UE that successfully join the Multicast MBS Session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X |  | X | X |
| PCF | The MB-PCF that provides policy control for the MBS session. |  |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| MBS FAI | MBS FAI is used for broadcast MBS session to guide the frequency selection for the UE | X (note 1) |  | X (note 1) |
| NOTE 1: It is an optional parameter. |  |  |  |  |

****************** NEXT CHANGES ***************

#### 9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session creation. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI request, MBS Service Type (multicast or broadcast), MBS FAI(s).

Input, Optional: DNN, S-NSSAI, MBS service area, MBS activation time, MBS termination time, service description, QoS flow information, Input Transport Address Request, session activity status (active/inactive). For a multicast session, indication that any UE may join. For subscription to notifications event ID(s), Notification Target Address.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBS FAI(s).

****************** NEXT CHANGES ***************

#### 9.1.3.7	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSSession_Update

Description: Update the established multicast session or broadcast session, e.g. QoS update.

Input, Required: MBS Session ID.

Input, Optional: QoS flow information, MBS service area, session activity status (active/inactive), MBS FAI(s).

Output, Required: Result Indication.

Output, Optional: Cause, MBS FAI(s).

****************** NEXT CHANGES ***************

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI request, Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be Cell ID list and/or TAI list, or geographical area information, or civic address information), MBS start time, MBS stop time, service requirement (e.g. Flow Description, QoS), Input Transport Address Request, session activity status (active/inactive), for subscription to notifications notification target address. For a multicast session, indication that any UE may join the multicast session. An Application ID, MBS FAI(s).

Editor's note:	How multiple service requirements with multiple application IDs are handled is FFS.

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info, MBS FAI(s).

****************** NEXT CHANGES ***************

#### 9.4.3.3	Nnef_MBSSession_Update service operation

Service operation name: Nnef_MBSSession_Update

Description: This service is used by the NF service consumer to update the established multicast session or broadcast session, e.g. QoS update. For multicast MBS Session, this service is also used to activate or deactivate the MBS Session.

Input, Required: MBS Session ID.

Input, Optional: Service requirement, MBS service area, session activity status (active/inactive), MBS FAI(s).

Output, Required: Result Indication.

Output, Optional: Cause, MBS FAI(s).

****************** END CHANGES ***************