SA WG2 Meeting #S2-149E	S2-2201135

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0097 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN in the location dependent service procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the join and handover procedure for the location dependent service, there are two EN on It is FFS whether the MB-SMF provides only the service areas associated with the target RAN node or all service areas associated with the MBS session in the shared delivery establishment response message. The SMF get the area session IDs and related MBS service area. When the UE join the multicasr, if the SMF only send the area session id and related service area that UE camp to NG-RAN, then the SMF shall perfrom the filter before sending such information. During the HO, the SMF can not do such filtering when it send the Handover request to target RAN. The SMF behavoir is different in there two scenario. If the source NG-RAN only have the service area of are session id it belong to, then for the Xn HO, if the target RAN is not in the service area, how it work is unclear So this CR proposes: The SMF provides all MBS service area IDs and related MBS service area to NG-RAN in N2 message; During Xn hadnover, the source MBS-capable NG-RAN provides all MBS service area IDs and related MBS service area to target NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SMF provides all MBS service area IDs and related MBS service area to NG-RAN in N2 message. During Xn hadnover, the source MBS-capable NG-RAN provides all MBS service area IDs and related MBS service area to target NG-RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN is not resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.2.1, 7.2.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.4.2	Support of location-dependent multicast service

##### 7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The location dependent multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The location dependent multicast session is created as described in clause 7.2.4.2.2.

-	The UE may have information about the location dependent multicast service including all MBS service areas of the location dependent MBS session.

-	If the UE determines that it is in MBS service area of the location dependent MBS session based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends Join Request to join the multicast group. If the UE determines that it is outside all MBS service area of the location dependent MBS session, the UE does not send the Join Request.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	The SMF check whether the UE is inside MBS service areas of the location dependent MBS session by comparing the User Location Information of the UE with the MBS service areas received from the MB-SMF. If the UE is out of the all MBS service area of the location dependent MBS session, the SMF reject the multicast session join request.

Editor's note:	How the AMF provides the ULI information to the SMF is FFS.

-	If the Join Request from the UE is accepted, the SMF subscribes at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies all MBS service areas of the location-dependent MBS session as Area Of Interest.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the all Area Session IDs and related MBS service area..

-	If the NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN in the shared delivery establishment response.

-	If the NG-RAN node serving the UE does not support MBS and UE is in the MBS service area, the SMF apply individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes (e.g. for a small MBS service area), or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the latter case, the SMF supplies the service area associated with the multicast session where the UE resides as Area Of Interest.

Next CHANGE

##### 7.2.4.2.3	Handover procedure

Editor's note:	The RAN specific behaviour in this clause requires RAN collaboration and confirmation.

Editor's note:	SMF subscribing/unsubscribing to AMF event AOI is repeatedly described and need clean up.

The Handover procedure for the UE is performed as defined in clause 7.2.3.2, 7.2.3.3, and 7.2.3.4 with the following additions:

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Shared MBS traffic delivery before the handover, for the Xn Handover (comparing with the clause 7.2.3.2), the following applies:

-	The Source RAN node includes MBS Session ID, all Area Session IDs and related MBS service area.

NOTE 1:	During the handover procedure the associated QoS flow is established towards a NG RAN node not supporting MBS regardless whether the UE is still in the MBS service area associated with the original area session ID or not

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Shared MBS traffic delivery before the handover, for the N2 Handover (comparing with the clause 7.2.3.2), the following applies:

-	The SMF forwards the RAN container information and may also include MBS session area information (MBS Session ID, Area Session ID and MBS service area) to the Target RAN in Handover request.

NOTE 2:	The SMF cannot determine the UE location and a possible new service area at this stage.

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Individual MBS traffic delivery before the Handover, for the Xn/N2 handover procedure (comparing with the clause 7.2.3.4), the following applies:

NOTE 3:	During the Xn handover procedure, the associated QoS flow is established at Target RAN side regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

-	For the N2 handover procedure, the SMF includes MBS session area information (MBS Session ID, Area Session ID, and MBS service area)  to the Target RAN node in Handover Request message.

-	If the Target RAN node support MBS, it determines whether to establish the resources for multicast distribution for the received MBS Session ID and Area Session ID, based on MBS Session ID, Area Session ID, MBS service area, provided by the source RAN (if source RAN support MBS) or SMF (if source RAN not support MBS) and the location of the UE. If UE is not in the MBS service area provided by the source RAN (if source RAN support MBS) or SMF (if source RAN not support MBS), the Target RAN does not allocate RAN resources for the multicast MBS Session to the UE.

-	If the target RAN node support MBS, when it determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides MBS session area information (Area session ID(s), MBS service area(s)) associated with the MBS session to the NG-RAN in the shared delivery establishment reply.

NOTE 4:	If the target RAN does not support MBS, the associated QoS flow is established at target RAN side during the handover procedure regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

-	If the target RAN supports MBS, but the Source RAN did not support MBS, the SMF configures the UPF to stop sending data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF unsubscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event (for an individual service area).

NOTE 5:	If the UE is still in the MBS session, the subscription for the UE entering or leaving the complete service area does not need to be changed.

-	When the AMF receives the User Location Information from target RAN node via the Path Switch Request message or Handover Notify message, the AMF provide it to the SMF. When the SMF get the User Location Information, the SMF checks the MBS service area of the UE camping cell by comparing the User Location Information received from AMF with the MBS service areas received from the MB-SMF. The SMF uses the determined service area and user location as follows:

-	The SMF updates the area session ID in the locally stored the UE MBS session context with the corresponding area session ID if the area session ID is changed.

-	If the target RAN does not support MBS, the Source RAN supported MBS, and the UE is in a service area of the MBS session, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the latter case the SMF supplies the service area of the multicast session as Area Of Interest. If associated QoS flows are not yet included in the PDU session, the SMF updates the PDU session with associated QoS flows.

-	If the UE has moved to another MBS service area of the MBS session:

-	If the target NG-RAN node support MBS and RAN resource has not been allocated, the SMF provides the MBS session information related to the new Area session ID to NG-RAN. For Xn handover, the SMF uses the Path Switch reply message. For N2 handover, the SMF updates the PDU session after the completion of the handover procedure. Per the received the MBS session information, the 5GC shared delivery is established.

-	If the target NG-RAN node does not support MBS, the SMF updates the UPF to forward the MBS data packet from the tunnel associated with the old Area session ID to the new Area session ID. If the SMF did not configure the UPF to receive the MBS data packet from the tunnel associated with the new Area session ID before, the SMF informs the MB-SMF of the new Area session ID and UPF DL N19mb Tunnel information. MB-SMF configure the MB-UPF to transmit the multicast session data towards UPF using the received downlink tunnel ID. If the SMF subscribed to the "Area Of Interest" event, the SMF also updates the subscription with the new service area as "Area Of Interest".

-	If the UE has moved out of all the MBS service areas of the MBS session:

-	If the target NG-RAN node does not support MBS, the SMF deletes the associated QoS flow from NG-RAN and UPF after the completion of the handover.

-	Per operator's policy (e.g. when a local configured timer expires since the UE left the whole MBS service area), the SMF may remove the UE from the MBS session as defined in clause 7.2.4.2.8.

END OF CHANGES