SA WG2 Meeting #150e	S2-2202753

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution: Support Direct Traffic Steering Policy ID Provisioning to UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for provisioning traffic steering policy ID from the PCF to the UPF directly.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the PCF provisions traffic steering policy ID to the UPF directly with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: Direct traffic steering policy ID provisioning to UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: Direct PFD Provisioning to UPF

6.X.1	Description

Some network information from PCF to UPF is transmitted transparently via SMF, which is not handled or aware by SMF, e.g., the traffic steering policy ID.

6.X.2	Procedures

When the PDU session is set up, the SMF find the appropriate PCF for it, which contains the traffic steering policy IDs related to the mechanism enabling traffic steering to the N6-LAN, DN and/or DNAIs associated with N6 traffic routing requirements. After that, the PCF may find the appropriate UPF and forward the traffic steering policy ID to the UPF. The UPF may support Nupf_ParameterProvision service operation, as defined in TS 23.502 [3] clause 5.2.26.X.

Figure 6.X.2-1: Direct traffic steering policy ID provisioning to UPF

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3]. A UPF is selected for this PDU Session.

1.	The PCF finds the appropriate UPF by using the S-NSSAI and DNN information, as described in clause 6.1.

2.	The PCF sends the Nupf_ParameterProvision_Create request to the UPF including the information of traffic steering policy ID.

4.	The UPF updates the traffic steering policy ID according to the request and acknowledges the PCF request with a Nupf_ParameterProvision_Create response.

**End of change**