3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204541

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #3, Sol #9: Update to add procedures

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	5G_LCS_Ph3 / Rel-18

Abstract: Solution update for local LMF and GML Solution update for local LMF and GMLC selection C selection

1. Introduction/Discussion

This pCR updates the solution #9.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change* * * *

6.9	Solution #9: local LMF and GMLC selection

6.9.1	Introduction

This solution enables dedicated LMF and GMLC selection for the UE positioning in the local network.

6.9.2	Functional Description

Two options are proposed for the AMF to select a local LMF and local GMLC:

-	Based on operator's configuration. An example could be a mapping table of UE identity e.g. MSISDN and LMF/GMLC address is stored at AMF.

-	Fetch from UDM, UE LCS subscription storing its serving local LMF and local GMLC address.

6.9.3	Procedures

6.9.3.1	5GC-MT-LR Procedure for the commercial location service

Is clause 6.1.2 of TS 23.273 [5].

Figure 6.9.3.1-1

Other steps are same as in TS 23.273 [5], except step 10:

10.	AMF, based on local configuration, e.g. mapping table mapping table of UE identity e.g. MSISDN and LMF address, or the UE LCS subscription data where includes a dedicated LMF ID, selects an LMF.

6.9.4	Impacts on services, entities, and interfaces

AMF:

-	configured locally a mapping table of UE identity e.g. MSISDN and LMF/GMLC address is stored at AMF.

-	Fetch from UDM, UE LCS subscription storing its serving local LMF and local GMLC address.

-	Determine the LMF for the location request

# * * * * Fifth change * * * *

# * * * * End of changes * * * *