3GPP TSG SA2 Meeting #152 (e-meeting) 	S2-2205969

17 – 26 August, 2022, Elbonia

# Source:	LG Electronics

# Title:	New WID: Cell based LADN (TEI18_CLADN)

# Document for:	Approval

Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Cell based LADN

Acronym:	TEI18_CLADN

Unique identifier:

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740061 | Stage 2 of 5G System - Phase 1 | Supporting for local area data network |

3	Justification

As defined in 3GPP TS 23.501, LADN (Local Area Data Network) is a feature which supports the access to a DN via a PDU Session only available in a specific service area. Currently, LADN service area is defined as a set of Tracking Areas. However, this limits the usage of LADN services because Tracking Area may be too large compared to service area that operator wants to provide LADN services. Therefore, supporting cell based LADN service area information, i.e. a set of Cells, is considered beneficial and flexible to operate LADN services.

If the AMF provides cell based LADN service area to a legacy UE that does not support cell based LADN service area, the UE cannot understand the LADN service area and will ignore the LADN service area information. In this case, according to current specification, the UE determines that it is out of the LADN service area because the UE considers that it does not have LADN service area information. So the UE will not establish a PDU Session for the LADN. In order to support backward compatibility, the UE should provide its capability of Cell based LADN to the AMF. Based on the capability, the AMF can determine whether to send cell based LADN service area information.

This work item aims to support cell based LADN service area so that smaller LADN service area can be supported in addition to Tracking Area based LADN service area.

4	Objective

This work item specifies the following aspects:

-	enable cell based LADN service area information, i.e. a set of Cells.

-	UE capability indication of Cell based LADN support for backward compatibility.

The work item reuse the existing LADN architecture baseline.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Add description that LADN service area can be a set of Cells as well. Add cell based LADN service area capability in the 5GMM Core Network Capability. | SA#98 (Dec. 2022) |  |
| TS 23.502 | Add description that the UE provides cell based LADN service area capability during the registration procedure. Add description that LADN service area can be a set of Cells as well during the registration procedure or UE Configuration Update procedure. | SA#98 (Dec. 2022) |  |

6	Work item Rapporteur(s)

Myungjune Youn (LG Electronics), m.youn@lge.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

CT1 for stage-3 work.

9	Supporting Individual Members

Supporting IM name

LG Electronics

AT&T

Broadcom

China Mobile

Deutsche Telekom

ETRI

Hansung University

KDDI

KT Corp.

LG Uplus

Oracle

Orange

Qualcomm Incorporated

Rakuten Mobile

SK Telecom

Sony

SyncTechno Inc.

Telecom Italia

Verizon UK LTD

Huawei

HiSilicon

InterDigital Inc.