SA WG2 Meeting #152e	S2-2206246

# August 17th – 26th, 2022; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-89: Updates to Solution #10

Document for: 	Approval

Agenda Item: 	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract of the contribution: This CR proposes updates to Solution #10 to align the solution with the latest corrections in TS 23.501.

# 1 Discussion

There was a correction done in TS 23.501, in SA2#151e as below:

Old text in TS 23.501:

The AMF may report to the PCF the subscribed RFSP Index received from the UDM for further evaluation as described in clause 6.1.2.1 of TS 23.503 [45]. When receiving the authorized RFSP Index from the PCF, the AMF shall replace the subscribed RFSP Index with the authorized RFSP Index.

Updated text in TS 23.501:

The AMF may report to the PCF the subscribed RFSP Index received from the UDM for further evaluation as described in clause 6.1.2.1 of TS 23.503 [45]. When receiving the authorized RFSP Index from the PCF, the AMF shall apply the authorized RFSP Index instead of the subscribed RFSP Index for choosing the RFSP index in use (as described above).

The Solution #10 in TR 23.700-89 is based on the obsolete specification text in TS 23.501 and needs to be updated to reflect the latest specification.

This paper proposes to update Solution #10 to use "RFSP Index in use" instead of "authorized RFSP Index" while notifying to UDM/HSS as the AMF does not store the "Authorized RFSP Index" provided by the PCF.

# 2 Proposal

It is proposed to update 23.700-89 as follows

FIRST CHANGE

## 6.10	Solution #10: Provide RFSP Index via N26 interface when the UE moves from 5GC to EPC

### 6.10.1	Description

This solution is applicable only when N26-based interworking is used.

When N26-based interworking architecture applies, AMF and MME use N26 interface to transfer mobility management message. The parameters, subscribed RFSP index and RFSP index in use, are transferred in:

-	Forward Relocation Request as in 5GS to EPS handover procedures using N26 interface, or

-	Context Response as in 5GS to EPS Idle mode mobility using N26 interface.

| Subscribed RFSP Index | CO | This IE shall be included by the source MME/SGSN/AMF, if received from the HSS or UDM. | RFSP Index | 0 |
| --- | --- | --- | --- | --- |
| RFSP Index in Use | CO | This IE shall be included by the source MME/SGSN/AMF, if it supports the feature. | RFSP Index | 1 |

During UE registration in 5GC, the AMF may report to the PCF the subscribed RFSP Index received from the UDM for further evaluation as described in clause 6.1.2.1 of TS 23.503 [45]. When receiving the "authorized RFSP Index" from the PCF, the AMF shall apply the authorized RFSP Index instead of the subscribed RFSP Index for choosing the RFSP index in use, as specified in clause 5.3.4.3.1 of TS 23.501 [2]. So the "RFSP Index in Use" stored in the UE context in AMF may be different than the "subscribed RFSP Index" received from the UDM as the AMF may decide on the "RFSP Index in Use" based on the "authorized RFSP Index" received from the PCF.

Therefore, this solution proposes that, when UE moves from 5G to 4G, the MME shall compare the "RFSP Index in Use" received from the AMF (which could be the "Authorized RFSP Index" provided by the PCF) with the "RFSP Index" received from the HSS. If they are different

this acts as an indication to the MME that 5GC has dynamically assigned an RFSP Index to the UE and the MME shall, based on operator's policies, ignore the Subscribed RFSP Index received from the HSS e.g. for a configured duration. This policy/duration could be operator determined e.g.:

-	"RFSP Index in Use " is used only for the current registration in the MME and is not used when the UE moves to another MME due to mobility.

-	"RFSP Index in Use " is used only for a configured duration (e.g. T). When the UE moves to another MME due to mobility (e.g. after a period "t"), the source MME then provides the remaining duration (i.e. T-t) for which "RFSP Index in Use " shall be used by the target MME.

-	Or it could be a combination of both of the above.

When the configured duration is reached, the MME may start using the "subscribed RFSP Index" that was received from the HSS.

### 6.10.2	Procedures

There is no additional procedure needed to update the RFSP Index when the UE is served by the EPC. MME uses the 5GC authorized RFSP Index for a configured duration as explained in clause 6.10.1.

### 6.10.3	Impacts on services, entities and interfaces

The solution has the following impacts:

PCF:

-	No impact.

AMF:

-	No impact.

MME:

-	MME shall compare the "RFSP Index in Use " received from the AMF (which could be the "Authorized RFSP Index" provided by the PCF) with the "Subscribed RFSP Index" received from the HSS.

-	If they are different, the

MME shall ignore the "Subscribed RFSP Index" received from the HSS for a configured duration.

-	When the configured duration is reached, the MME may start using the "Subscribed RFSP Index" that was received from the HSS.

N26:

-	No impact.

END OF CHANGES