3GPP TSG-SA2 Meeting #152E	S2-2206650

# August 17 – August 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0543 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling Analytics for NSAG priority |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2, NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NSAG priority, which is introduced in SA2#151e, is determined by AMF and provided to UE. This CR is to enable the AMF to make use of analytics (i.e., NSAG priority statistics) when determining the NSAG priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable AMF to get assistance by NSAG analytics from NWDAF when determining the NSAG priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMF cannot make use of analytics provided by NWDAF in determining the NSAG priority |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

6.7.x	NSAG priority analytics

6.7.x.1	General

The NWDAF provides NSAG priority analytics to a service consumer NF. The analytics results are generated based on the data from other 5GC NFs. The analytics results contain NSAG priority statistics of UE according to UE location. The consumer can request either one-time or continuous reporting.

The service consumer may be a NF (e.g. AMF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "NSAG Priority Analytics".

-	Target of Analytics Reporting: a single UE (SUPI), a group of UEs (an Internal Group ID), or any UE;

-	Analytics Filter Information optionally containing:

-	Area of Interest (AOI) (list of TA or cells);

-	S-NSSAI(s);

-	NSAG ID(s);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

6.7.x.2	Input Data

The NWDAF supporting data analytics on NSAG priority shall be able to collect NSAG priority determined for UE from 5GC.

-	Network data related to NSAG priority from 5GC is defined in the Table 6.7.x.2-1;

Table 6.7.x.2-1: NSAG priority-related information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations | AMF | UE location information |
| NSAG priority | AMF, NSSF | An NSAG priority, which consists of priority value per NSAG ID, provided to the UE |
| Mapping between NSAG to S-NSSAIs | AMF, NSSF | The mapping between NSAG IDs to S-NSSAIs |
| Timestamp | AMF, NSSF | A time stamp when the measurements are taken |

6.7.x.3	Output Analytics

The NWDAF generates NSAG priority statistics. The detailed information provided by the NWDAF is defined in Table 6.7.x.3-1.

Table 6.7.x.3-1: NSAG priority statistics for any UE

| Information | Description |
| --- | --- |
| Area of Interest | List of TAIs or Cell IDs fin the Analytics Filter information |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Slice (1…max) | List of S-NSSAIs in the Analytics Filter information |
| >> average slice priority value | average slice priority value for the S-NSSAI across all users |

Table 6.7.x.3-1: NSAG priority statistics for specific set of UEs

| Information | Description |
| --- | --- |
| Area of Interest | List of TAIs or Cell IDs fin the Analytics Filter information |
| UE IDs (1..SUPImax) | List of UEs in the analytics reporting target |
| >Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > NSAG priority | NSAG priority provided to the UE |

6.7.x.4	Procedures

Figure 6.11.4-1: Procedure for NSAG priority analytics

1.	The consumer NF sends a request to the NWDAF for NSAG priority analytics using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to "NSAG priority analytics", the Target of Analytics Reporting and the Analytics Filter Information are set according to clause 6.7.x.1. The consumer NF can request statistics.

2-3.	The NWDAF subscribes to information related to UE’s NSAG priority from AMF (or NSSF).

4.	The NWDAF derives requested analytics with the collected data. Analytics output parameters are listed in clause 6.7.x.3.

5.	The NWDAF provides the requested analytics to the NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service, depending on the service used at step 1.

6.	If the NF subscribed the analytics at step 1, the NWDAF provides a new analytics when it generated the new output.

* * *End of Changes * * *