3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206723

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI#2: Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: An evaluation and conclusion on KI#2 is proposed.

1. Introduction/Discussion

This proposal proposes an evaluation and conclusion on KI#2 (Support of Satellite Edge Computing via UPF on board).

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change (All new)* * *

7	Overall Evaluation

7.x	Evaluation of solutions for Key Issue #2: Support of Satellite Edge Computing via UPF on board

There are 4 candidate solutions (solutions #4, #5, #6, #10) proposed to solve how to support satellite edge computing services via UPF on-board.

Regarding the selection of a UPF onboard to access satellite edge computing services, both solution#4 and solution#5 propose that the UPF on-board can be selected and inserted by the SMF as UL CL/BP and L-PSA, but solution#4 requires the SMF to select a UPF onboard based on DNAI which is mapped from GEO satellite ID, the satellite ID is determined by AMF based on configuration; solution#5 requires the UPF onboard to report the gNB ID it connects to the SMF via N4 association, then the SMF can select a UPF connecting with the gNB serving the UE after determining the candidate UPFs using existing mechanism.

Regarding Satellite EC service relocation, solution #10 propose to deploy a local DNS on the satellite, which can collect EAS information on this satellite and adjacent satellites. While EAS relocation is required, the UE discovers a new EAS via local DNS. The UE visits EAS via PSA on the satellite. However, it is unclear how to enable the interaction between the SMF and the L-DNS server on the satellite, i.e., how to provide the information about the EAS on neighboring GEO. And whether there are some impacts to IETF spec.

# * * * * Second change * * * *

8	Conclusions

8.x	Conclusions on Support of Satellite Edge Computing via UPF on board

Regarding the selection of a UPF onboard to access satellite edge computing services, it is concluded that the SMF selects the UPF onboard the satellite providing backhaul services to the gNB serving the UE.

# * * * * End of changes * * * *