3GPP SA WG2 Meeting #152E  			   	   S2-2205876

Elbonia, August 17-26, 2022	(was S2-22xxxx)

Source:	Ericsson

Title:	KI#3, New solution: Network Slice or service availability on a per cell level granularity

Document for:	Discussion/Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_PH3

Abstract of the contribution: This contribution provides a new solution for KI#3 based on existing NR slicing work and on Service Area Restrictions.

# 1.	Discussion

UE policies with URSP allows the PCF to generate the Route Selection Validation Criteria in the URSP with a Location Criteria- The Location Criteria can already be specified on a per cell level granularity. It is assumed that URSP with cell level granular Location Criteria will be part of the evaluation of the KI#3.

In case, using URSP on a per cell level granularity (possibly with other enhancements done in Rel-18) is not enough as there is additionally a need to enforce the logic and also to impact e.g. mobility. There are other existing logic to control UE’s access and mobility i.e. Service Area Restrictions.

Service Area Restrictions controls whether the UE can access the network in a certain area, on a per TA level granularity, for normal services. The Service Area Restrictions Allowed Area can be extended to allow a list of cells for a specific Network Slice as to address the KI#3.

# 2 Proposal

It is proposed to include the below solution to the TR.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #15: Service continuity in case of Network Slice instance overload | X |  |  |  |  |  |
| Solution #16: UE assisted slice based VPLMN prioritization for Extended SoR |  | X |  |  |  |  |
| Solution #17: Slice based VPLMN Selection Policy |  | X |  |  |  |  |
| Solution #18: Sending rejected NSSAI to the UDM to assist the UDM to steer the UE to the PLMN supporting rejected NSSAI |  | X |  |  |  |  |
| Solution #19: configuring the UE with network slice aware preferred PLMNs lists |  | X |  |  |  |  |
| Solution #20: VPLMN Selection following existing SoR information |  | X |  |  |  |  |
| Solution #21: Temporary slice based on URSP |  |  | X |  |  |  |
| Solution #22: Enabling graceful slice termination with support of UE policies |  |  | X |  |  |  |
| Solution #23: UE registration for conditional network slices |  |  | X |  | X |  |
| Solution #24: On the handling temporary network slices |  |  | X |  |  |  |
| Solution #25: Handling Rejected S-NSSAIs in some TAs of RA |  |  |  |  | X |  |
| Solution #26: Multiple areas and resource partitioning |  |  | X |  | X |  |
| Solution #27: Exception to the rejected NSSAI handling |  |  |  |  | X |  |
| Solution #28: Support of network slices with TA granularity within a RA |  |  |  |  | X |  |
| Solution #29: On handling S-NSSAIs not supported in certain TAs of a RA during a registration |  |  | X |  | X |  |
| Solution #30: Rejected S-NSSAI with new cause value |  |  |  |  | X |  |
| Solution #31: Enabling Flexible RAs with Slice Service Area |  |  |  |  | X |  |
| Solution #32: Solution for Network Control for UE Slice Use | X |  |  |  |  | X |
| Solution #33: Slice-specific implicit deactivation timers |  |  |  |  |  | X |
| Solution #34: On-demand slices |  |  |  |  |  | X |
| Solution #35: Network Slice usage control by the network |  |  |  |  |  | X |
| Solution #36: UE provided reason for registration to S-NSSAI |  |  |  |  |  | X |
| Solution #37: Actual UE Activity-based Slice Admission Control |  |  |  |  |  | X |
| Solution #38: On configuring the UE with UE behaviour policies |  |  |  |  |  | X |
| Solution #39: Serving PLMN steering UE to preferred slice for selection of PDU session |  |  |  |  |  | X |
| Solution #X: Controlling UE access to the network per Network Slice on a per cell level granularity |  |  | X |  |  |  |

NEXT CHANGE

## 6.X	Solution #2: Controlling UE access to the network per Network Slice on a per cell level granularity

### 6.X.1	Introduction

The solution addresses the “Network Slice Area of Service for services not mapping to existing TAs boundaries” of the Key Issue #3: Network Slice Area of Service for services not mapping to existing TAs boundaries and Temporary network slices.

### 6.X.2	Functional Description

The solution provides two mechanisms to select among for addressing the case when Network Slice Area of Service for services not mapping to existing TAs boundaries.

The two mechanisms are summarized as follows:

1.	Extend Access and mobility related policy information such that Service Area Restrictions can be set per S-NSSAI on a per cell level granularity. UDM can set Service Area Restrictions per subscribed S-NSSAI and also indicate the geographical information (e.g. longitude/latitude, zip code, etc) or (for non-roaming) list of cells, As per current logic the PCF can adjust the Service Area Restrictions, and then AMF sends the Service Area Restrictions to the UE. The UE then applies the Service Area Restrictions possibly on a per network slice and cell granularity.

### 6.X.3	Procedures

#### 6.X.3.1	General

During initial registration the UE indicates that the UE is supporting the extended functionality i.e. URSP, Service Area Restrictions The AMF applies current logic and if the UE requests or subscribes to S-NSSAIs that are applicable for the extended location restriction and the UE does not support the extended functionality, the network decides whether to provide the applicable S-NSSAI in Allowed NSSAI and Configured NSSAI to the UE. The decision can be implemented as operator policy in AMF, or decided by another NF e.g. UDM, NSSF or PCF in which case the UE support needs to be forwarded to the NF.

The initial registration procedure is executed as in 23.502 clause 4.2.2.2.2 with the following differences.

0.

1.	The UE indicates the UE’s capability to support the extended location restriction functionality in the Registration message e.g. in the UE 5GMM Core Network Capability.

2.	No changes to steps 2 to 14a of 23.502 clause 4.2.2.2.2.

3.	If UDM is to be able to decide whether to provide S-NSSAIs applicable for extended location restriction to the UE, then AMF indicates whether the UE and serving network supports the extended location restriction functionality.

4.	 the UDM can provide Service Area Restrictions with per S-NSSAI restrictions. The location information can be as currently, geographical information (e.g. longitude/latitude, zip code, etc) or list of cells,. The UDM can adapt the content based on the knowledge whether the UE and serving network supports the extended location restrictions.

5.	For the Service Area Restrictions option, the AMF provides the subscribed Service Area Restrictions to the PCF and the PCF can adapt the content.

6.	No changes to steps 14c to 20 of 23.502 clause 4.2.2.2.2.

7.	

the AMF provides the extended Service Area Restrictions to the UE.

8.	The UE stores the extended location restrictions and enforces the information.

For the extended Service Area Restrictions option, the UE applies the Allowed or Non-Allowed Area per S-NSSAI and the granularity per list of cells

The NG-RAN enforces the extended location restrictions for  Service Area Restrictions. The AMF enforces the extended location restrictions for the Service Area Restrictions option

10.	In step 22 the UE can additionally report that the extended location restrictions been stored. No additional changes to steps 21b to 25 of 23.502 clause 4.2.2.2.2.

#### 6.X.3.2	Mobility procedures

. the NG-RAN gets the extensions as part of the Mobility Restriction List (that is extended with per S-NSSAI and per cell restrictions) and the Mobility Restrictions for CM-CONNECTED state when in RRC-Connected state are executed by the radio access network and the core network such that UP for the associated S-NSSAIs are not allowed to be activated (as per current procedures).

### 6.x.4	Impacts on services, entities and interfaces

NG-RAN:

-	Support extended location restrictions for the Service Area Restrictions option.

AMF:

-	Support extended location restrictions

PCF:

-	Support extension with a per cell level granularity per network slice for Service Area Restrictions

UE:

-	Support extended location restrictions and indicate the UE’s support for e.g.

-	support Service Area Restrictions on a per S-NSSAI and cell level granularity

UDM:

-	Service Area Restrictions per subscribed S-NSSAI

END OF CHANGES