SA WG2 Meeting #152e	S2-2205919r02

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, AT&T, Deutsche Telekom

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

1 Discussion

2 Proposal

It is proposed to update TR 23.700-18 as follows.

Start of change

8	Conclusions

8.1   Key Issue #1: Traffic Steering Policy and SFC Enhancements

It is recommended to use the following principles and procedures as the basis for the normative work.

-

-

-	Support the N6-LAN traffic steering control and AF-influenced traffic steering control to be applicable to the same traffic simultaneously.

-	5GC may receive from the AF policies associating for one, a group or all UE(s) some traffic (filter) with a SFC Id together to send to the SFC once the corresponding traffic has been identified

-	The PCF determines a policy per SDF/application for the purpose of steering the subscriber's traffic to appropriated N6 service functions deployed by the operator or a 3rd party service provider. The policy is expressed in a Traffic Steering Policy (TSP) ID that may be separate in UL and DL directions.

-

-

-

-	The TSP ID refers to a traffic steering behaviour that is configured in the SMF/UPF.

-	The PCF provides the TSP ID in the PCC rules to SMF.

-	The SMF provisions corresponding PDRs, FARs, QERs etc to support SFC. In particular, the SMF creates a FAR with the Forwarding Policy parameters set to the TSP ID. No impacts to N4 rules

-	The UPF transforms the TSP ID into SFP configuration including the way to indicate over N6 traffic belonging to SFC; there may be different SFP for UL and DL traffic. This mapping is controlled by local configuration.

-	It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

-

-

-

-