SA WG2 Meeting #152-e	S2- S2-2206045r03

Electronic meeting, 17 - 26 August, 2022

Source:	NEC

Title:	KI #6, Sol #38: Update to remove an Editor’s note and update to solution #38

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes to address an Editor’s note and to update solution #38.

Discussion

Proposal

It is proposed to capture the following changes in TR 23.700-08.

* * * * Start of Changes * * * *

6.38	Solution #38: Sequential deregistration by hosting network

6.38.1	Introduction

The solution addresses the KI#6: Support for returning to the home network. The congestion of home network is unavoidable if large number of UEs return to the home network as soon as Localized service is terminated. This solution proposes to control the number of returning UE based on service level agreement between the home network and hosting network.

6.38.2	Functional Description

This solution assumes that

-	There is service level agreement between the home network and hosting network.

-	The hosting network configures the AMF with maximum number of returning UEs for the given units of time for each of home network, e.g. 10 UEs per second for PLMN A. The home network operator can determine the maximum number of returning UEs e.g. considering the network processing capability and notifies it to the hosting network via service level agreement.

-	The AMF of hosting network knows termination time of Localized services.

When the Localized service is terminated, the AMF of hosting network triggers deregistration procedure. The AMF selects UEs from different home networks considering maximum number of returning UEs configured in the AMF. The AMF triggers deregistration for the selected UEs in sequence and ensures that number of returning UEs for each home network does not exceed the configured maximum number of returning UEs for the given units of time.

NOTE 1:	If many UEs trigger deregistration without waiting for network initiated deregistration, the AMF cannot control the number of returning UEs. In order to give some time to the AMF to trigger deregistration in a sequence manner, service termination time provided to the UE can be later than the actual service termination time. The time gap can be determined considering the expected number of returning UEs and maximum number of returning UEs for the given units of time for the home network.

NOTE 2:	How the AMF enforces maximum number of returning UE is not specified by 3GPP specification and it is up to implementation.

6.38.3	Procedures

Figure 6.38.3-1: Overall procedure

1.	The AMF is configured with the maximum number of returning UEs for the given units of time for each of home network.

2.	The UE registers to the hosting network for localized service. The AMF stores the home network of the UE.

When the UE registers to the hosting network, the AMF needs to identify the home network of the UE in order to associate the configuration in step 1 with the UE. The Registration Request from the UE includes valid 5G-GUTI (and optionally NID in case of SNPN assigned case) regardless of whether the 5G-GUTI is assigned by PLMN or SNPN. Based on the received 5G-GUTI (and optionally NID), the AMF in hosting network knows home network of the UE.

NOTE:	In case of roaming, the UE provide 5G-GUTI assigned by roaming network. Then AMF applies the maximum number of returning UEs of the roaming network.

3.	After Localized service is terminated, the AMF triggers deregistration in a sequence manner so that the number of returning UEs does not exceed the configured maximum number of returning UEs. The AMF indicates that Localized service is terminated in the Deregistration Request message. The UE returns to the home network. The UE does not re-register to the hosting network again based on received indication from the AMF.

NOTE:	The AMF can also take into account UE-initiated deregistered UEs when the AMF enforces maximum number of returning UE.

6.38.4	Impacts on services, entities, and interfaces

UE:

-	If the AMF indicates that Localized service is terminated in the Deregistration Request message, the UE finishes deregistration procedure and returns to the home network.

-	 The UE includes valid 5G-GUTI (and optionally NID in case of SNPN assigned case) regardless of whether the 5G-GUTI is assigned by PLMN or SNPN.

AMF:

-	The AMF is configured with maximum number of returning UEs for the given units of time for each of home network. The AMF triggers deregistration in a sequence manner so that number of returning UEs does not exceed configured maximum number of returning UEs.

-	The AMF indicates that Localized service is terminated in the Deregistration Request message.

* * * * End of Changes * * * *