3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206111

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung, Huawei

Title:	Conclusion for FS_GMEC KI#5

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Conclusion for KI#5.

1. Introduction/Discussion

Based on evaluations of solutions for KI#5, it is proposed to update the conclusion for KI#5.

Since sol#6 and sol#17 both assume that the data sent to different groups corresponds to data with different destinations and the UE can determine how to send the data toward a group, their principles or approaches are in line and the scenarios are complementary. Hence, it is proposed to adopt sol#6 and sol#17 when the application on the UE can replicate multiple copies of the data.

Since sol#7 is the only solution to address the case where UE application is not capable to replicate multiple copies of the data, so sol#7 is used for such case.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

8.5	Key Issue #5: Allowing UE to simultaneously send data to different groups with different QoS policy

In case when UE/Application is capable to replicate multiple copies of the data, Solution #6 and Solution #17 shall be the baseline and the following are way forwards.

-	If different groups (IP/Ethernet multicast groups) are associated to the same DNN and S-NSSAI combination used for 5G VN group, then different QoS Flows of single PDU Session may be used to transfer the data copy sent to different groups

-	If different groups (IP/Ethernet multicast groups) are associated to the different DNN and S-NSSAI combinations used for 5G VN group, then different PDU Sessions are used to transfer the data copy sent to different groups

In case when UE/Application is not capable to replicate multiple copies of the data, Solution #7 shall be the baseline and the following are way forwards.

-	UE uses one single PDU Session with a predefined DNN/S-NSSAI (or multiple DNNs/S-NSSAIs but with more impacts on existing NFs) to transfer the data copy sent to different groups.

NOTE:	The impacts to existing R17 network will be reconsidered during normative phase.

-	UE sends one UL copy on the QoS flow with most strict QoS, and UPF is responsible for packet duplication and further distribution to different groups.

# * * * * End of changes * * * *