SA WG2 Meeting #152E	S2-2206158r05

# Aug 17th – 26th, 2022; Elbonia               		       (revision of S2-2206158)

Source: 	Samsung

Title: 	New solution for Key Issue#4. Provisioning of credentials via Hosting network

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This solution proposes the framework of how credentials for accessing localised services can be provisioned to an UE which do not have any prior subscription with the Hosting network, nor any prior relationship with the Localized service provider via the Hosting network.

# 1	Discussion

A particular scenario can be seen in the clause 5.7 of TR 22.844. We need a framework where user who doesn’t have any prior subscription with either the Hosting Network or the localised service provider.

Note that UE may not be able to connect to the LSP server by other means since its Home Network may not be having service in the area where Hosting network is deployed (An event may be hosted in a remote area, far from establishment).

This solution proposes how a User can request “on-demand” connectivity for the localised services. After the UE has subscribed for the services it may be provisioned with the appropriate credentials (which might be temporary in nature). These credential can be credentials for the Hosting network or of a Credential Holder.

Note that UE may manually select the particular Hosting network.

It may also be possible that only one particular 3rd party localised service provider has agreement at a particular time. Then as soon as UE tries to register with the Hosting network, there might be a need to redirect the user to a captive portal so that it can may subscribe for the Localised services ( pay for the services/ receive credential etc.).

The provisioning of default credentials for Localised service as proposed in Sol #22 proposes additional challenges of their provision and how will they be authenticated.

The restricted PDU session access may also be provided to the UEs which cannot be primarily authenticated via the Home network (that is un-authenticated UEs). The collaboration with SA3 will be required.

Also this restricted PDU Session for provisioning is also needed even if the UE can be primarily authenticated via its Home network, since it does not have any subscription/credential to access Localised services. Prior to the provisioning of credentials, UE is registered with the Hosting network only for restricted services. After getting the credentials, UE will have to register again/do authorization to access the localised service.

# 2 Proposal

It is proposed to add the solution to TR 23.700-08 FS_eNPN_Ph2.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  | X | X |  |
| 12 |  |  | X | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| 19 |  | X |  |  |  |  |
| 20 |  | X |  |  |  |  |
| 21 |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |
| 23 |  |  |  | X |  |  |
| 24 |  |  |  | X |  |  |
| 25 |  |  |  | X |  |  |
| 26 |  |  |  | X |  |  |
| 27 |  |  |  | X | X |  |
| 28 |  |  |  | X | X |  |
| 29 |  |  |  | X |  |  |
| 30 |  |  |  | X |  |  |
| 31 |  |  |  | X |  |  |
| 32 |  |  |  | X |  |  |
| 33 |  |  |  | X |  |  |
| 34 |  |  |  | X |  |  |
| 35 |  |  |  |  | X |  |
| 36 |  |  |  |  | X |  |
| 37 |  |  |  |  | X |  |
| 38 |  |  |  |  |  | X |
| 39 |  |  |  |  |  | X |
| X |  |  | X | X |  |  |

* * * Second Change * * * *  ALL text is new

## 6.X	Solution #X: Provisioning of Credentials via Hosting Network PDU Session

### 6.X.1	Introduction

This solution takes ideas from Sol#22.

This solution is mainly focuses on Key Issue#4 regarding how to provision UE with credentials for Localized service. It tries to solve the case of how to access Localized services when UE has neither any prior subscription with Hosting network nor with the Localised service provider (clause 5.7 of TR 22.844). The solution thus proposes how Hosting network can perform “on boarding” of UE for a particular localised service.

It also provides ideas for Key Issue#3, focusing on what kind of information needs to be exchanged between Localized service provider and the Hosting network.

The solution proposes to provision the credential to the UE via a restricted PDU Session. User may choose a particular Localized service based on information displayed on its mobile phone, and based on the chosen service and local configurations, AMF and SMF may create a session for the UE via which UE can connect to a captive portal pay for the services and then be provisioned the required credentials for accessing the Hosting network.

During the service agreement between Localised service provider and the Hosting network, details of captive portal may be provided by the Localised service provider to the Hosting network.

If the Hosting network is able to authenticate UE using its Home Network credentials, even then UE may require new credentials for accessing localised services (because UE’s current subscription does not include Localised services usage).

If the Hosting network is not able to authenticate the UE using its Home network credentials, then provisioning can be RLOS based.

### 6.X.2	Functional Description

Localized service provider may provide captive portal information, whether to create a restricted PDU Sessions for provisioning for the UE and/or DNAI,S-NSSAI information for the restricted PDU Session etc. during service agreement phase.

As per the agreement between Localised service provider PCF may configure captive portal information/redirection information to SMF (SMF may provide it to UPF in form of FAR rules).

User may manually select a particular Hosting Network.

The User may then choose a particular Localised service based on the displayed information. This information may be available to the UE by some other solution (e.g. Sol#26 or Sol#34).

The AMF then decides to generate a PDU session which may be restricted in terms of time and the destination traffic, for the UE based on the chosen localised service. If the UE did not choose any service, based on local configuration UE’s session may be established to a particular portal.

Note: It may be possible that the UE cannot be primarily authenticated. Then decision to generate the PDU Session is based on local policies. SA3 collaboration might be needed.

### 6.X.3	Procedures

Figure 1: Provisioning Localised service credentials via the Hosting network

High level details of the Procedure is described as follows:

- User may manually choose a particular Hosting network which provides a localized service.

EN: It is FFS how UE will register if it cannot be authenticated with the Home Network credentials. To decide whether RLOS-based provisioning of credentials can be used.

- NAS request: UE may include the selected localised service in the request. The details of how this information is available to UEs is assumed to be provided by other solutions.

- AMF decides to create a PDU Session for provisioning purposes.

2a. NAS Downlink Message (AMF to UE) which may include DNN, S-NSSAI, an indication to trigger PDU Session creation.

AMF can send UE an indication or trigger related to “Provisioning for localised service”; which, based on UE implementation, may launch a particular application (e.g. web browser) which will initiate the PDU Session establishment.

For example based on the indication sent by the AMF, UE may display a particular notification to the User informing about provisioning of credentials. User may click the notification and a particular application (UE implementation specific) may launch triggering the PDU Session establishment.

Editor’s Note: Security implications of the AMF triggering the UE to a PDU session establishment need to be assessed by SA3. Potentially, any serving network may trigger the UE to establish a PDU session to any target DNN/S-NSSAI

2b. PDU Session Establishment Request (UE to AMF)

- AMF sends Nsmf_PDUSession_create to the SMF

Editor’s Note: Whether and if yes, which, additional parameters the Nsmf_PDUSession_create request needs to consider is FFS

- SMF creates a PDU session by providing appropriate information to UE and UPF. The session may be time bounded, and filtered for Destination addresses etc.

Editor’s Note: How the SMF matches the Nsmf_PDUSession_create request to the parameters of the restricted PDU session (e.g. destination address) is FFS

- 5a-5b UE connects to the external portal and gets the credentials. Details of this step are out of 3GPP scope.

Note: UE’s PDU Session may be redirected to the captive portal by using PDR/FAR rules installed by PCF.

- UE performs authentication/authorization with the new credentials. This may also involve sending UE a new Initial Registration to the Hosting network.

Editor’s Note: PCF interactions are missing from the procedure flow

### 6.X.4	Impacts on services, entities and interfaces

AMF/SMF capability to create a restricted PDU Session. Support for RLOS

PCF capability update SMF/AMF regarding information of restricted portal, captive portal information etc.

UE capability to initiate PDU Session based on AMF trigger/indication.

EN: Further UE impacts are FFS.

EN: The security impacts shall be evaluated by SA WG3