3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206167r01

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #5 Evaluation and conclusion for switching traffic of an MA PDU Session between two non-3GPP access paths

Document for:	Approval

Agenda Item:	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract: This paper proposes evaluation on solutions for KI #5 about switching traffic of an MA PDU Session between two non-3GPP access paths.

# 1. Introduction/Discussion

In this paper, evaluation part is added for KI #5, with focusing on several evaluation aspects including HO capability indication, HO indication, list of PDU session indication, deregistration timer, ATSSS rules, HO for SA PDU session, HO for same RAT type. It is tried to evaluate that which capability or modification is necessary, while others may be optional.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-53 v0.3.0.

# * * * * First change * * * *

# 7	Evaluation

Editor's note:	This clause provides the evaluation of different solutions. It may contain an evaluation for each Key Issue.

## 7.x	Evaluation for KI #5: Switching traffic of an MA PDU Session between two non-3GPP access paths

There are two categories of solutions for KI#5, called Option 1 and Option 2.

-	Option 1: Switching the data traffic after the Registration procedure (Solution #5.1, #5.3 and #5.4)

-	These solutions propose to perform the Registration procedure over the new non-3GPP access first and after the procedure is finishes, the UE adds a new leg to the existing MA PDU Session by re-using existing access addition procedure. After the access addition is completed, the network performs access release / deregistration over the old non-3GPP access.

The main impact of these solutions is maintaining two simultaneous registrations over two non-3GPP accesses. There may exist undisclosed impacts to the system as the overall solution describes only access switching aspects and other procedures are not investigated in detail.

In solution #5.1, #5.3 there is impact to the UE, AMF, SMF, UPF and UDM. In solution #5.4, there is no clear description that there is impact to the UDM. However, it seems that the UDM needs to maintain two simultaneous registration states so there is impact to the UDM.

-	Option 2: Switching the data traffic during the Registration procedure (Solution #5.2, #5.5 and #5.6)

-	These solutions propose to perform access switching during the Registration procedure over the new non-3GPP access, i.e. there is no need to trigger additional procedure for access switching after the Registration procedure.

In these solutions, there is no need to maintain simultaneous registration over two non-3GPP accesses. This enables to perform access switching by registering over the new non-3GPP access without need to trigger another session related procedure to add additional access to the existing MA PDU Session.

In solution #5.2 and #5.6, there is impact to the UE, AMF, SMF, UPF while in solution #5.5 there is additional impact to the UDM.

Several aspects of Option 1 and Option 2 are considered below.

Path switching (PS) capability indication:

For option 1 & 2:

- UE capability indication - since the traffic switching is initiated by UE, the UE shall be able to support traffic switching. Limiting UE’s handover capability by include indication in its subscription is not necessary since there is no capability indication in subscription for handover within 3GPP accesses.

- Network capability indication - an indication is sent to the UE before switching traffic in order to prevent the UE from initiating the path switching when the AMF/SMF does not support it.

Path switching (PS) indication:

For option 1:

- AMF needs to be informed that this registration via target non-3GPP access is intended for switching the traffic between two non-3GPP access paths. It is used by the AMF to maintain temporary dual-registration via two non-3GPP accesses during the PS procedure. This indication is necessary to allow the AMF to distinguish the scenario where the registration shall be immediately replaced, or temporary registration is maintained for 2 N3GPP accesses.

- SMF needs to be informed that this PDU session establishment request via non-3GPP access is intended for switching the traffic between two non-3GPP access paths. in order to allow the SMF to establish new UP resources on the target access while existing UP resources in source non-3GPP access are preserved.

- Considering that these two indicators are both necessary and they are only applicable for non-3GPP traffic switching, it is preferable to define only 1 indicator (information element) which is used by both AMF and SMF.

For option 2:

- In this option the UE register for the purpose of traffic switching. When the AMF receives the handover indication from UE, it shall update the SM context of UE in order to trigger the user-plane resources establishment. The registration results would also indicate whether the handover between two non-3GPP accesses is success or not. Therefore, from the point of view of the whole procedure, the definition of a new registration type is preferred for option 2.

List of PDU session indication:

For option 1:

- Since UE performs PDU session establishment after the end of registrationprocedure, the UE can indicate the list of PDU session that wants to move in PDU session establishment and therefore it is not necessary to include list of PDU session in Registration Request procedures.

For option 2:

- List of PDU session can used for SMF to distinguish which PDU session the UE is requesting to move, thus it is necessary.

Deregistration timer:

For option 1:

- The AMF needs to determine the maximum time for maintaining the UE registered via two non-3GPP access in order to allow the UE to perform the establishment of PDU session on target non-3GPP access, but to avoid also to keep for a long time the two registrations, for example when the UE does not establish PDU session on target access for some unexpected reason. The usage of a timer is not necessary for SMF.

For option 2:

- It is not required since PS is completed during registration.

ATSSS rules update:

For option 1 & 2:

- When UE completes user-plane resources establishment on target non-3GPP access, two non-3GPP accesses paths are available before that the user-plane resource of source non-3GPP access would be released. If UE has UL traffic to be transmitted, the UE may select either the target or the source non-3GPP access. If source non-3GPP access is selected, the service over non-3GPP access would be interrupted when user-plane resource of source non-3GPP access is released afterward. Enhancements on ATSSS rule with indication of the RAT type (Trusted or Untrusted N3GPP) to be used would solve the issue and improve the service continuity. Thus, updated ATSSS rule with the indication of RAT-type to be used is required.

PS for SA PDU session:

For option 1 & 2:

- All existing solutions can support PS for single access PDU session between Trusted and Untrusted network, i.e., with change of RAT type.

PS for same RAT type:

For option 1 & 2:

- The Trusted and Untrusted N3GPP access mandates the support of MOBIKE (RFC 4555) which enables the mobility of one peer end points (but not of both end point at the same time). For example in TS 23502 for Trusted N3GPP is defined the following:

- “In step 13c, the TNGF provides to UE (a) an "inner" IP address, (b) a NAS_IP_ADDRESS and a TCP port number and (c) a DSCP value. After this step, an IPsec SA is established between the UE and TNGF. This is referred to as the "signalling IPsec SA" and operates in Tunnel mode. Operation in Tunnel mode enables the use of MOBIKE [40] for re-establishing the IPsec SAs when the IP address of the UE changes during mobility events. All IP packets exchanged between the UE and TNGF via the "signalling IPsec SA" shall be marked with the above DSCP value. The UE and the TNAP may map the DSCP value to a QoS level (e.g. to an EDCA Access Class [48]) supported by the underlying non-3GPP Access Network. The mapping of a DSCP value to a QoS level of the non-3GPP Access Network is outside the scope of 3GPP

- Therefore, mobility of UE between different access network without changing of anchor Access node, as shown in figure 1, is currently supported.

- The mobility of UE between the same access network type (RAT-type) with changing of anchor Access node, as shown in figure 2, is not supported by MOBIKE (see RFC 4555) hence the solution selected for support PS for SA PDU session between Trusted and Untrusted N3GPP access needs to be applicable to the scenario of PS for SA PDU session without changing of RAT type.

Figure 1: Mobility of UE between 2 N3GPP access networks without changing of access node

Figure 2: Mobility of UE between 2 N3GPP access networks with changing of AN node (e.g. N3IWF)

The following table lists evaluation aspects of existing proposed solutions.

| Sol | Handover Procedure | Capability Indication | Handover Indicator | List of PDU Session | ATSSS Rules Update | SA PDU Session Support | Deregistration Timer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #5.1 | Separate, option 1 | Not mentioned | IE Indicator for both AMF and SMF | Included in PDU session establishment procedure | Required to indicate target N3GPP access | Yes | Applied for AMF |
| #5.2 | Combined, option 2 | Include both UE and network capability indication | New Registration Type for AMF and IE indicator for SMF | Included in Registration Request procedure | Not mentioned | Yes | Not needed |
| #5.3 | Separate, option 1 | Include only AMF capability indication | New Registration Type for AMF and IE indicator for SMF | Included in PDU session establishment procedure | Not mentioned | Yes | Applied for AMF |
| #5.4 | Separate, option 1 | Include only UE capability indication | IE Indicator for both AMF and SMF | Included in both Registration Request procedure and PDU session establishment procedure | Not mentioned | Yes | Applied for AMF and SMF |
| #5.5 | Combined, option 2 | Include both UE and network capability indication | New Registration Type for AMF and IE indicator for SMF | Included in Registration Request procedure | Not mentioned | Yes | Not needed |
| #5.6 | Combined, option 2 | Include both UE and network capability indication | IE Indicator for both AMF and SMF | Included in Registration Request procedure | Not Mention | Yes | Not needed |

Additional evaluation aspects for the KI#5 solutions are considered below. In particular, the following key aspects are considered:

- New registration type = “non-3GPP path switch” or registration type = “initial” + new IE

- Capability exchange between UE and network

  - 5G Session Management (5GSM) capability exchange

  - 5G Mobility Management (5GMM) capability exchange

- PDU Session Establishment in the new non-3GPP access without requiring UE initiated PDU Session Establishment/Modification signaling

- UDM subscription needed for ATSSS path switching

- AMF update of RAT Type in UDM after user plane resources establishment in the new non-3GPP access

- Timer related to de-registration of new non-3GPP access

- Total Editor Note’s

- Impacts on existing Nodes and Functionality

Tables 7.x-1 & 7.x-2 compares several key aspects where they differ or are align.

Table 7.x-1: Comparison of Solutions to Evaluation Items

| Solution # & Title | New Registration Type or New IE for Path Switch Indication | Capability exchange between UE & network (5GSM or 5GMM layer) | PDU Session Establishment in the new N3GPP access without requiring UE initiated PDU Session Establishment/Modification signaling | UDM subscription needed for ATSSS path switching | AMF update of RAT Type in UDM after User Plane Resources establishment in the new N3GPP Access | Timer for de-registration of new N3GPP Access | Total Editor Note’s |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #5.1: Support traffic switching between two non-3GPP paths | New Registration Type | No | No | No | No | Yes (AMF) | 0 |
| #5.2: Delaying UDM Registration until non-3GPP access switching completes | New Registration Type | Yes (5GSM) | Yes | No | Yes | No | 0 |
| #5.3: Path switching between non-3GPP accesses | Either | Yes (5GMM) | No | No | Yes | Yes (AMF) | 0 |
| #5.4: Non-3GPP access path switching in MA PDU Session | New IE | FFS (Editor’s Note) | No | Yes | ? | Yes (AMF & SMF) (Editor’s Note) | 6 |
| #5.5: Non-3GPP path switch during Registration in new non-3GPP access | Either (Editor’s Note) | Yes (5GSM) (Editor’s Note) | Yes | No | Yes | No | 4 |
| #5.6: Consolidated solution for traffic switching between two non-3GPP access paths | Either | Yes (5GSM) | Yes | No | Yes | No | 1 |

Table 7.x-2: Comparison of Solutions to Impacts on existing Nodes and Functionality

| Solution # & Title | Impacts on Existing Nodes & Functionality |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | UE | AMF | SMF | UPF | UDM |
| #5.1: Support traffic switching between two non-3GPP paths | Yes | Yes | Yes | Yes | Yes |
| #5.2: Delaying UDM Registration until non-3GPP access switching completes | Yes | Yes | Yes | Yes | ? No impact listed, but step 8 shows new functionality |
| #5.3: Path switching between non-3GPP accesses | Yes | Yes | Yes | Yes | Yes |
| #5.4: Non-3GPP access path switching in MA PDU Session | Yes | Yes | Yes | Yes | ? No impact listed, but step 3 shows new functionality |
| #5.5: Non-3GPP path switch during Registration in new non-3GPP access | Yes | Yes | Yes | Yes | Yes |
| #5.6: Consolidated solution for traffic switching between two non-3GPP access paths | Yes | Yes | Yes | Yes | ? No impact listed, but step 11 shows new functionality |

# * * * * End of changes * * * *