SA WG2 Meeting #152E	S2-2206194r01

# May 16th – 20th, 2022; Elbonia

Source: 	Samsung

Title: 	KI#4, new solution for Authentication for localized services and removing Sol#13 EN

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This solution proposes additional UE subscription data in the UDM and authentication procedure for target localised service for the cases when UE is in Home network (for provisioning Hosting network related information) and when UE is in Hosting Network (for granting access to localised services).

# 1	Discussion

This solution aims at solving Key Issue#4, specifically “Mechanisms to authorize UE to access the hosting network”.

The solution proposes addition to UE subscription data in UDM and enhancing AMF capability of initiating Authentication for a target localized service for authentication/authorizing UEs in Hosting Network.

Similar procedure can be used to do authorization for target localized services in Home Network when the UE requests for Hosting network information for a target localized service.

Also it proposes to remove the following Editor’s Note of Sol#13.

Editor's Note: It is FFS how the authorization and authentication for the target localized service are performed in the home network.

# 2 Proposal

It is proposed to add the solution to TR 23.700-08 FS_eNPN_Ph2.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  | X | X |  |
| 12 |  |  | X | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| 19 |  | X |  |  |  |  |
| 20 |  | X |  |  |  |  |
| 21 |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |
| 23 |  |  |  | X |  |  |
| 24 |  |  |  | X |  |  |
| 25 |  |  |  | X |  |  |
| 26 |  |  |  | X |  |  |
| 27 |  |  |  | X | X |  |
| 28 |  |  |  | X | X |  |
| 29 |  |  |  | X |  |  |
| 30 |  |  |  | X |  |  |
| 31 |  |  |  | X |  |  |
| 32 |  |  |  | X |  |  |
| 33 |  |  |  | X |  |  |
| 34 |  |  |  | X |  |  |
| 35 |  |  |  |  | X |  |
| 36 |  |  |  |  | X |  |
| 37 |  |  |  |  | X |  |
| 38 |  |  |  |  |  | X |
| 39 |  |  |  |  |  | X |
| X |  |  |  | X |  |  |

* * * Second Change * * * *  ALL text is new

## 6.X	Solution #X: Authentication for a target localised service

### 6.X.1	Introduction

This solution provides mechanism to authenticate/authorize UE for a particular localised service. The same procedure is applicable in both the Home network and Hosting network.

The UE may need to be authenticated for a target localised service in following cases:

- UE registered in the Serving network/Home network when UE requests for Hosting network information related for a particular Localized service ( Reference : Solution 13, Step 2 of procedure 6.13.3.2-1). Here the authentication is performed to  decide whether UE can be provisioned with Hosting network information fro the target Localised service or not

- In the Hosting Network: Localized service may be provided in the form of enhanced services to the User. UE may be provided specific services (for e.g better QoS etc.) accordingly whether UE can be authenticated by a particular Localized Service provider (maybe a 3rd party).

The mentioned procedure may only apply for Secondary authentication/authorization for a target localised service.

It is assumed that the Home Network/Hosting network are already configured with Localised service related information during the service agreement/relationship establishment between localised service provider and the corresponding network.

Also in this solution we assume that Localised services are provided by some 3rd party, which hosts a AAA server.

### 6.X.2	Functional Description

The following table describes the addition to UE Localised services subscription data in the UDM.

Note: Additional information may need to be included based on requirements for other solutions.

The UE sends a registration request with the Localized service ID to the AMF. In the case of Home network when UE want to get Hosting network selection info, UE can include an “indication” that it requires Hosting network selection information.

AMF then may request the UDM UE subscription data related to how to authorize for the UE provided target localized service. The authentication related information for a Target localized service may also be configured locally to the AMF.

AMF may be notified that UE need not require any authentication for the selected Localized service, i.e. UE’s current subscription does not need authentication/authorization for accessing the particular localized service (in case when UE is in the Hosting network) or for provisioning of Hosting network information for selected Localized service (in case when

It may be provided the information of the AAA server through which UE can proceed to do the authentication. NSSAAF service may be required to authenticate the UE.

Note: SA3 analysis will be required.

EN: It is FFS whether information of DN-AAA server authentication via SMF is necessary or not.

Based on the case, UE will be granted the access to Hosting network for using localized services or will get the requested Hosting network information.

### 6.X.3	Procedures

Figure 6.X.3- 1 Authentication for target localised service

- Registration Request/NAS request from UE to AMF

- :

UE may request the Home network for Hosting network information for the target Localised service or it may request the Hosting network to provide the target localised service.

The request may include the name/identifier for the target localized service.

In case of Home network when UE want to get Hosting network selection info it may also include the additional indication regarding “request for Hosting network information”.

Note: the details of How UE requests for Hosting network information from the Home network and How the Home network provides it, is assumed to be addressed by other solutions (e.g. Sol#12 etc.)

- Nudm_SDM_Get AMF may request the Localised subscription data for the UE for the corresponding localised service

- AMF decides to do the authentication for the target localised service wither based on Subscription data from the UDM or based on the local configuration.

- Authentication procedure: Network may perform the authorization and authentication for the target localised service. Specific steps of Slice specific authentication procedure may be reused, that is AMF may reuse NSSAAF services to authenticate for the target localised service.

EN: details for the procedure needs to be discussed with SA WG3.

- AMF may notify UE about the granting of localized service, or may provide Hosting network selection information.

### 6.X.4	Impacts on services, entities and interfaces

UDM impact: change in UE subscription data

AMF impact: ability to triggering the appropriate Authentication procedure based on Local configuration and subscription data for the localized service.

UE impact: ability to send NAS request to the AMF for accessing to a target localised service or requesting for Hosting network selection information.

EN: Further impacts are FFS.