SA WG2 Meeting #S2-152E	S2-2206207r02

17 - 26 August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0130 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the mobility in RRC inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.247, describes the connected mode mobility procedure for multicast MBS session. But one scenario is missing, i.e. RRC inactive mobility. When the Multicast MBS session is deactivated, the UE may be in RRC inactive state. The UE may resume the connection in another RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause to describe the mobility in RRC inactive state for 5MBS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenarios for handling MBS session(s) in case of RRC inactive mobility are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.3.X	UE Triggered Connection Resume in RRC Inactive for inactive MBS session

If an MBS session is in Inactive state, the UE may be in CM-CONNECTED with RRC Inactive state. The UE may resume the connection in a different NG-RAN node as specified in the TS 23.502 [6] clause 4.8.2 with following enhancement:

-	For an MBS supporting NG-RAN, if the UE context retrieved from the last serving NG-RAN includes MBS session information and

the NG-RAN node has not yet established the 5GC Shared MBS traffic delivery for the MBS session, the NG-RAN establishes the 5GC Shared MBS traffic delivery as specified in clause 7.2.1.4.

-	For an MBS supporting NG-RAN, the NG-RAN sends Path Switch Request indicating the MBS capability as specified in clause 7.2.3.2.

-	For an MBS non-supporting NG-RAN, after the procedure of user plane switch is finished, the SMF removes the associated QoS flow(s) information.

END OF CHANGES