3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206318r01

Elbonia, August 17th – 26th, 2022	(revision of S2-2206318)

Source:	Huawei, HiSilicon, China Mobile

Title:	Service Operations used in IMS Data Channel Architecture

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper gives service operations used in IMS data channel architecture.

1. Introduction/Discussion

Based the IMS data channel architecture described in pCR S2-2205772, this paper lists service operations used in IMS data channel architecture.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.X	Solution #Y: Service operations used in Data Channel architecture

6.X.1	Call Flow Description

6.X.1.1	General

The following clauses illustrate UE initiated bootstrap data channel establishment procedure, UE initiated P2P application data channel establishment procedure and data channel termination procedure to show the services provided by the NFs in IMS data channel architecture.

6.X.1.2	UE initiated bootstrap data channel establishment procedure

Figure 6.X.1.2-1: UE Initiated Bootstrap Data Channel Establishment Procedure

1.  UE A sends a SIP INVITE request with an initial SDP to IMS AS. The initial SDP offer contains media description of bootstrap data channel(s). In this example procedure, SDP offer contains both bootstrap data channel offers for originating side and terminating side.

NOTE: This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS session is setup.

2.  IMS AS validates the Data Channel media description information and/or user subscription data to determine whether the session need to be notified to the DCSF. If the session needs to be notified to the DCSF, the IMS AS selects a DCSF based on local policy.

3.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the call event and the bootstrap data channel(s) media resource information of UE A.

NOTE: The notification without explicit subscription in this step will be confirmed during normative phase.

4.  After receiving the Nimsas_MER_Notify message, the DCSF determines the policy about how to process the bootstrap data channel establishment requirement based on the related parameters in the message and/or operator strategy.

5.  The DCSF sends a Nimsas_DCC_Request message to the IMS AS with the policy of bootstrap data channel establishment, and it includes the indications as follows:

- Terminates the bootstrap data channel which takes the originating network DCMF as target;

- Transfers the media description of bootstrap data channel transparently which takes the terminating network DCMF as target;

- Initiates a bootstrap data channel which takes the terminating UE as target.

The DCSF also includes the media point information, i.e. IP addresses/ports of DCSF for routing of further application requests and applications between DCSF and DCMF for originating side and terminating.

6.  The IMS AS selects a DCMF based on local configuration or discovers and selects a DCMF instance via NRF.

7.  IMS AS sends a Ndcmf_MRM_Create request to the DCMF to apply for DC resource based on the DC media information received from the DCSF. After completing DC resource allocation, the DCMF returns a Ndcmf_MRM_Create response to IMS AS.

8.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the information of bootstrap data channel resource allocated on the DCMF.

9.  The DCSF stores the DC resource information and establishes connections with DCMF for bootstrap DC traffic to originating UE A.

10-11. IMS AS sends the INVITE request with the updated SDP offer adding media information of bootstrap data channel(s) to the originating S-CSCF and then to remote network side and UE B. The SDP offer for bootstrap data channel to UE B may also be included.

12. UE B and terminating network returns an 18X response with SDP answer to bootstrap DC to originating network. If the terminating network decides to establish another bootstrap DC to UE A and UE B, the corresponding data channel media description is also included in SDP answer.

13.  IMS AS sends a Ndcmf_MRM_Update request message to the DCMF for DC resource modification based on the SDP answer received from terminating network.

14.  IMS AS sends a Nimsas_MER_Notify message to the DCSF for the data channel media modification. The media information of bootstrap DC for terminating UE B on DCMF may also be included.

15.  The DCSF stores the DC resource information of bootstrap DC for UE B for routing bootstrap DC traffic.

16-18. The IMS AS modifies SDP answer for bootstrap data channels to UE A in 18X response which may include local bootstrap data channel to UE A and/or remote bootstrap data channel to UE A, and sends 18X response to S-CSCF and P-CSCF. The originating network P-CSCF executes QoS procedure for bootstrap data channel media based on the SDP answer information from the 18X response. Then P-CSCF returns the 18X response to UE A.

19.  The PRACK and UPDATE negotiation between originating and terminating network.

20.  The bootstrap data channels are established between originating DCMF and UE A/UE B.

21.  The bootstrap data channels are established between terminating DCMF and UE A/UE B.

6.X.1.3	UE initiated P2P application data channel establishment procedure

In the following procedure, the IMS session and bootstrap data channel(s) have been established. UE A and UE B have retrieved one data channel application.

Figure 6.X.1.3-1: UE Initiated P2P Application Data Channel Establishment Procedure

1.  UE A sends a SIP re-INVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the established bootstrap data channel information, as well as the application data channel establishment requirement which is indicated by stream ID (dcmap value:greater than or equal to 1000), stream Label, etc.

2.  IMS AS validates the Data Channel media description information and/or user subscription data to determine whether the session need to be notified to the DCSF.

3.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the call event and the application data channel(s) media information of UE A.

4.  After receiving the Nimsas_MER_Notify message, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the DC call request and/or operator strategy.

5.  DCSF sends a Nimsas_DCC_Request message to IMS AS with the policy of application data channel establishment. If the DCMF needs to anchor application data channel, the policy includes the indication as follows:

- Initiates the application data channel which takes UE B as target endpoint via originating DCMF.

If application data channel is not anchored on DCMF, step 6 and 12 are skipped.

6.  If the DCMF needs to anchor application data channel, IMS AS sends Ndcmf_MRM_Create request to the DCMF to apply for application data channel resource allocation based on the DC media information received from the DCSF.

7.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the information of the application data channel media resource allocated on the DCMF.

8.  The DCSF stores the DC resource information.

9-11. IMS AS sends the re-INVITE to originating S-CSCF and then to remote network side and UE B. If the DCMF needs to anchor application data channel, IMS AS includes the updated SDP offer adding media information of the DCMF. UE B and terminating network returns a 200 OK response with SDP answer for application DC to originating network.

12.  If the DCMF needs to anchor application data channel, IMS AS sends a Ndcmf_MRM_Update request message to the DCMF for application data channel media resource modification based on the SDP answer to application DC.

13.  IMS AS sends a Ndcmf_MRM_Update request message to the DCMF for application data channel media resource modification.

14.  The DCSF stores the DC resource information of application DC for UE B for subsequent process.

15-17. The IMS AS modifies SDP answer for application data channels to UE A in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF. The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response. P-CSCF returns the 200 OK response to UE A.

18.  UE A send ACK to the terminating network.

19.  The application data channel between UE A and UE B is established which may be anchored on the DCMF.

6.X.1.4	Data channel termination procedure

Figure 6.X.1.4-1 shows the call flow of an IMS session normal release , in which data channel termination procedure is included.

Figure 6.X.1.4-1: Data Channel Termination Procedure

1.  UE A sends a BYE request to terminate the current session. This BYE is routed to the IMS AS.

2.  IMS AS validates user subscription data to determine whether the session need to be notified to the DCSF.

3.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the call release event.

4.  The DCSF determines the policy that all the data channel resource contained in this session need to be released.

5.  The DCSF sends a Nimsas_DCC_Request to the IMS AS and indicates it to release all the data channel resource.

6.  The IMS AS sends a Ndcmf_MRM_Delete request to the DCMF for DC resource release. The DCMF will release the bootstrap data channel resource used for local UE. If the DCMF has anchored the application data channel(s) used in this session, it need to release the application data channel(s) too.

7.  IMS AS sends a Nimsas_MER_Notify message to the DCSF with the DC resource release event.

8-10. The IMS AS sends the BYE request to the terminating network and UE B. The terminating network also need to release the data channel resource allocated for UE A and UE B. Then UE B returns a 200 OK response to the originating network.

11. After receiving the 200 OK response, the IMS AS sends a Ndcmf_MRM_Delete request to the DCMF. The DCMF will release the bootstrap data channel resource used for remote UE.

12. IMS AS sends a Nimsas_MER_Notify message to the DCSF with the DC resource release event.

13. The IMS AS sends the 200 OK response to UE A through IMS Core.

6.X.2	Service Description

6.X.2.1	General

All the services described below will be registered to the NRF and discovered and selected via the NRF.

6.X.2.2	DCMF Services

6.X.2.2.1	General

Figure 6.X.2.2.1-1 and Figure 6.X.2.2.1-2 show the reference architecture for the DCMF services in SBI representation style and reference point representation style respectively.

Figure 6.X.2.2.1-1: Reference Architecture for the DCMF Services; SBI representation

Figure 6.X.2.2.1-2: Reference Architecture for the DCMF Services; reference point representation

The following table illustrates the DCMF Services and Service Operations.

Table 6.X.2.2.1-1: NF services provided by the DCMF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| NDCMF_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

6.X.2.2.2	Ndcmf_MediaResourceManagement (MRM) service

6.X.2.2.2.1	General

Service description: This service enables the IMS AS to create, update and delete IMS Data Channel related resources required by some specified data channel.

6.X.2.2.2.2	Ndcmf_MRM_Create service operation

Service operation name: Ndcmf_MRM_Create

Description: The IMS AS requests the DCMF to allocate IMS Data Channel related resource required by some specified data channel.

Inputs, Required: Data Channel media description (stream ID, direction, max_retry, max_time, priority), App resource URL allocated by the DCSF, IP address and port of the DCSF.

Inputs, Conditional: If it’s the data channel between the local UE and the DCMF, the remote end point address (IP address and port) is required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication, Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF).

Outputs, Optional: None.

6.X.2.2.2.3	Ndcmf_MRM_Update service operation

Service operation name: Ndcmf_MRM_Update

Description: The IMS AS requests the DCMF to update IMS data channel related resource.

Inputs, Required: Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF), Data channel media information (stream ID, direction, max_retry, max_time, and priority), the remote end point address.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.2.2.2.4	Ndcmf_MRM_Delete service operation

Service operation name: Ndcmf_MRM_Delete

Description: The IMS AS requests the DCMF to delete IMS data channel related resource.

Inputs, Required: Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF).

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.2.3	IMS AS Services

6.X.2.3.1	General

Figure 6.X.2.3.1-1 and Figure 6.X.2.3.1-2 show the reference architecture for the IMS AS services in SBI representation style and reference point representation style respectively.

Figure 6.X.2.3.1-1: Reference Architecture for the IMS AS Services; SBI representation

Figure 6.X.2.3.1-2: Reference Architecture for the IMS AS Services; reference point representation

The following table illustrates the IMS AS Services and Service Operations.

Table 6.X.2.3.1-1: NF services provided by the IMS AS

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_DataChannelControl (DCC) | Request | Request/Response | DCSF |
| Nimsas_MediaEventReport (MER) | Notify | Subscribe/Notify | DCSF |

6.X.2.3.2	Nimsas_DataChannelControl (DCC) service

6.X.2.3.2.1	General

Service description: This service enables the DCSF to indicate the IMS AS to execute operations on data channel streams.

6.X.2.3.2.2	Nimsas_DCC_Request service operation

Service operation name: Nimsas_DCC_Request

Description: The DCSF requests the IMS AS to deal with bootstrap and application data channel(s).

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID, operation type and data channel type (application data channel or bootstrap data channel), App resource URL allocated by the DCSF, IP address and port of the DCSF, and role (the calling party or the called party).

The operation types include:

- Initiate a data channel. For example, the local network initiates a bootstrap data channel between the remote UE and itself.

- Terminate a data channel. For example, the local network terminates the bootstrap data channel between the local UE and itself.

- Reject a data channel. For example, the remote network rejects to establish a bootstrap data channel between the local UE and itself for some reason.

- Transfer the media description of a data channel transparently. For example, the local network transfers the media description of bootstrap data channel(s) between the local UE and the remote network.

- Anchor the data channel media. For example, the local network anchors the application data channel(s) between the local UE and the remote UE to its DCMF based on the service requirement.

- Close the data channel. It means the data channel resource will be released.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.2.3.3	Nimsas_MediaEventReport (MER) service

6.X.2.3.3.1	General

Service description: This service enables the IMS AS to notify the DCSF of IMS Data Channel related media events and session status in a SIP session.

NOTE:	This notification corresponds to an implicit subscription.

6.X.2.3.3.2	Nimsas_MER_Notify service operation

Service operation name: Nimsas_MER_Notify

Description: The IMS AS notify the DCSF of the latest media event and session status related to a session context.

Inputs, Required: Call-ID of the SIP session, GPSI, media event type.

Inputs, Conditional: If the media event type is Data Channel media update, Data Channel media information including dcmap and originator (local or remote) are required. If the session status changes, the latest session status are required. If the media event type is allocated Data Channel resource, Data Channel resource URL allocated by the DCMF and IP address and port of the DCMF are required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *