3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206561

Elbonia, Aug 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	KI#3: Evaluation for redundant traffic steering

Document for:	Approval

Agenda Item:	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract: This paper proposes evaluation on solutions for KI#3.

# 1. Introduction/Discussion

In this paper, evaluation part is proposed for the KI#3.

# 2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53.

# * * * * First change * * * *

## 7.x	Evaluation for KI #3: Support of redundant traffic steering

### 7.x.1	Considerations on Duplication Criteria

In order to support redundant traffic steering, there are solution #3.1, 3.2, 3.3, 3.4, 3.5 and 3.6 proposed to define the Redundant Steering Mode (RSM). If this new steering mode is applied by UE and UPF, the traffic is transmitted via 3GPP and non-3GPP accesses in a redundant way.

Considering redundant transmission is costly, which consumes double resources compared with normal transmission without duplication, it is considered important to define duplication criteria that indicate when RSM should be initiated. In other words, predefined conditions or criteria, as proposed by quite a few solutions, such as solution 3.1, 3.2, 3.3 and 3.4, can be defined as trigger points for initiating redundant transmission. The duplication criteria may not always need as, in some cases, applications may always require redundant transmission without criteria for activation / deactivation.

One of the major benefits of redundant transmission is to reduce packet loss rate, therefore Packet loss rate should be taken as one of the criteria. Except the packet loss rate, the other criteria are also proposed by solution 3.3 and solution 3.4 to trigger the redundant transmission, including Round Trip Time (RTT), jitter, percentage of duplicated traffic and load over an access network.

Another major benefit of redundant transmission is to reduce the RTT. If the RTT via both accesses cannot satisfy the traffic requirement, the redundant transmission can reduce the RTT because the receiver always selects the packet that arrives faster. Note that if the RTT over 3GPP access is smaller than the RTT over non-3GPP access, this does not mean that all packets arrive faster via 3GPP access. Some percentage of packets will arrive faster via non-3GPP and since the receiver always selects the packet that arrives first, this means that the RTT due to redundant transmission will be smaller than the RTT over 3GPP access and than the RTT over non-3GPP access. Therefore, the RTT should be taken as one of the criteria.

As to the percentage of duplicated traffic proposed in these solutions, it is unclear how the percentage is determined and how duplication according to the percentage can satisfy the service requirement, e.g. the PLR.

# * * * * End of changes * * * *