3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206606

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	NTT DOCOMO

Title:	Update of conclusions for KI#2

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC/ Rel-18

Abstract: It is proposed to update the conclusion of KI#2 for a scenario when the UE moves outside or moves back inside the AF requested coverage area.

# 1. Introduction/Discussion

Conclusion for KI#2 does not properly describe the details when the UE moves outside or moves back inside the AF requested coverage area. It is proposed to use similar procedure as being concluded for KI#1; if the AF has requested (g)PTP based time distribution, then TSCTSF temporarily removes the UE/DS-TT from the PTP instance, or adds the UE/DS-TT back to the PTP instance, respectively. Also how the AF is notified for this is harmonized with KI#1.

The current conclusions for KI#2 describe how the TSCTSF is notified for the UE location based on the list of TAs or Cell IDs in the AF requested coverage area. It is unclear how the procedure should work for UEs in Idle state, e.g. whether the AMF pages such UEs to determine the current location. It is proposed to add an EN for this issue. It should be noted that KI#1 requires similar procedure for TSCTSF to be able to determine the impacted UEs under a certain RAN-node for which the 5GS internal clock accuracy has degraded. The impacted UE(s) may be in Idle state.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25.

# * * * * First change * * * *

## 8.2	Key Issue #2: Time synchronization service enhancements

The following bullet points summarize the principles for the way forward:

-	To request a Requested Coverage Area for time synchronization services, AFs within the operator's domain formulates a spatial validity condition using a list of Tracking Area or Cell identities, but AFs outside the operator's domain use a geographical area (e.g. a civic address or shapes) instead, while NEF transforms this information into a list of Tracking Area or Cell identities.

-	The TSCTSF is responsible of activate/deactivate time synchronization services (ASTI or (g)PTP based) considering the spatial validity condition provided by the AF.

-	The TSCTSF discovers the AMF(s) serving the list of TA(s)/Cell(s) that comprise the spatial validity condition using the NRF.

-	The TSCTSF subscribes to UE's presence in Area of Interest at the discovered AMF(s).

-	The TSCTSF determines (based on notifications from the discovered AMF(s)) whether the targeted UE(s) are inside or outside the AF Requested Coverage Area.

Editor's Note: How to solve the Coverage Area on cell-level granularity for UEs in Idle mode is FFS.

-	The TSCTSF activates time synchronization services for UE(s) that are inside the Requested Coverage Area. If the AF has requested (g)PTP based time distribution, then TSCTSF creates the PTP port in DS-TT and adds it to the PTP instance as described in clause 4.15.9.3.2 in TS 23.502 [3].

-	TSCTSF notifies the AF with the indication of 5G access stratum time distribution (enabled, disabled) for the targeted UE(s) for which the AF has requested ASTI-based time distribution that are inside or move inside or outside of the Requested Coverage Area.

-	If the TSCTSF has determined the UE has moved outside of the AF Requested Coverage Area for which the AF has requested (g)PTP based time distribution, then TSCTSF temporarily removes the UE/DS-TT from the PTP instance:

-	If the DS-TT is configured to send Sync, Follow_Up and Announce messages for the related PTP instance, then TSCTSF deactivates the Grandmaster functionality in the DS-TT using PMIC (see also clause K.2.2.4 of TS 23.501 [2]) and removes the DS-TT from the PTP instance (see also clause K.2.2.1 of TS 23.501 [2]).

-	If NW-TT is configured to send Sync, Follow_Up and Announce messages on behalf of the DS-TT, then TSCTSF deactivates the Grandmaster functionality on behalf of the DS-TT in NW-TT using UMIC (see also clause K.2.2.4 of TS 23.501 [2]) and removes the DS-TT from the PTP instance (see also clause K.2.2.1 of TS 23.501 [2]).

-	The TSCTSF informs the AF for the impacted UE by indicating the PTP port state as Inactive for the related DS-TT PTP port.

-	If TSCTSF has determined UE has moved inside of the AF Requested Coverage Area for which the AF has requested (g)PTP based time distribution, then TSCTSF adds the DS-TT PTP port to the PTP instance again and also re-activates the Grandmaster functionality. The TSCTSF informs the AF for the impacted UE by indicating the PTP port state as Active for the related DS-TT PTP port.

# * * * * End of changes * * * *