SA WG2 Meeting #152e				   S2-2206734r01

# 17 - 26 August, 2022, Electronic meeting		       (revision of S2-220xxxx)

Source: 	Charter Communications, Lenovo

Title:	Conclusion for KI#5

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution: This paper provides a conclusion for KI#5

# Introduction/Discussion

Key issue #5 aims to study how the data traffic of an MA PDU Session can be switched between one non-3GPP access path and another non-3GPP access path in the same PLMN.

Solutions 5.2 through 5.6 all provide procedures of switching traffic of a MA PDU Session from untrusted non-3GPP access path to trusted non-3GPP access path, while Solution 5.1 provide procedures of switching traffic of a MA PDU Session from trusted non-3GPP access path to untrusted non-3GPP access path.

For the most part, the current set of solutions are aligned in the procedures required to switch between any two (same or different types) non-3GPP access paths.  This contribution attempts to identify the differences proposed in the current set of solutions, in order to aid in evaluating the benefits in the differences.

According to the evaluation paper S2-2206729, solutions #5.5 and solution #5.6 are very similar and incorporates aspects from solutions #5.2 through #5.4. .

# 2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53 v0.3.0.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause enlists the conclusions of the study, including agreements for work to be done in the normative phase.

## 8.x	Conclusion for KI #5: Switching traffic of an MA PDU Session between two non-3GPP access paths

The solution to be specified in the normative phase shall support the following principles:

1)	The path switching between non-3GPP accesses is performed  after the Registration procedure. Firstly the UE sends a Registration Request via the target non-3GPP access network to add the second non-3GPP access, when the registration is completed the UE performs the existing MA PDU Session by re-using existing access addition procedure. After the access addition is completed, the network performs access release / deregistration over the old non-3GPP access

Editor’s note: It is FFS whether a new registration type (e.g., Registration type = non-3GPP path switching) or a new indication shall be used.

2)	The AMF updates the registration for non-3GPP access in UDM after the data traffic is switched to the new access. AMF to maintain temporary dual-registration via two non-3GPP accesses during the PSHO procedure. This indication is necessary to allow the AMF to distinguish the scenario where the registration shall be immediately replaced, or temporary registration is maintained for 2 N3GPP accesses.

3) 	Path switching between any two non-3GPP accesses shall be supported.

4) The network capability indication of supporting non-3GPP path switching is exchanged between the UE and the SMF during PDU Session Establishment procedure.

  - Editor’s note:  It is FFS whether this capability exchange is also performed at 5G Mobility Management (5GMM) signaling signaling (i.e., Registration procedure ).

5) the UE send the list of PDU session to be moved in new Non-3GPP access to SMF.

6) The Trusted and Untrusted non-3GPP access mandates the support of MOBIKE (RFC 4555) which enables the mobility of one peer end points (but not of both end point at the same time) . Therefore, mobility of UE between different access network without changing of anchor Access node, as shown in figure 8.x-1, is currently supported. The mobility of UE between the same access network type (RAT-type) with changing of anchor Access node is not supported by MOBIKE (see RFC 4555) during normative will considered the extension of solution selected to support HO PS for SA PDU session between within same RAT-type as shown in figure 8.x-2.

Figure 8.x-1: Mobility of UE between 2 N3GPP access networks without changing of access node

Figure 8.x-2: Mobility of UE between 2 N3GPP access networks with changing of AN node (e.g. N3IWF)

# * * * * End of changes * * * *