3GPP TSG-SA WG2 Meeting #152-e	S2-2206762r03

# E-Meeting, 17th – 26th Aug 2022

Title:	[draft]LS Out on NSSRG restriction on pending NSSAI

Response to:	S2-2205406/ C1-224073

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-xxxxxx, S2-xxxxxx

1. Overall Description:

SA2 thanks for CT1 LS on the NSSRG restriction on pending NSSAI. SA2 answers the question as follows:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

SA2 Answer: Yes, the UE should considers the S-NSSAI in Pending NSSAI as an S-NSSAI requested by the UE and therefore the UE considers the S-NSSAIs in the Pending NSSAI in order to apply the NSSRG restrictions.

It was discussed that NSSAA may take a “long” time to complete, but as that would be an abnormal scenario SA2 assume CT1 has the responsibility to take care of such scenario. For example, CT1 could specify AMF logic to check NSSRG compatibility after NSSAA been executed and then let AMF execute UCU/CUC by either adding S-NSSAI from NSSAA in Allowed NSSAI or Rejected NSSAI as required (which may then require UE logic to locally remove the S-NSSAI from Pending NSSAI in the UE).

SA2 agreed the attached CRs.

2. Actions:

To CT1 working group.

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153e		10-14,Oct, 2022	E-Meeting

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting