SA WG2 Meeting #152e	S2-2206770r02

# 17 - 26 August 2022, Electronic meeting         		            (revision of S2-220xxxx)

Source: 	Lenovo, InterDigital

Title: 	Conclusions for KI#3

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

Discussion and Proposal

One of the objectives of ATSSS_ph3 is to study a how to support a new traffic steering mode, called Redundant Steering Mode (RSM). Several solutions have been proposed for this objective and now conclusions should be derived based on these solutions. For deriving those conclusions, it is important to establish a common understand about why RSM is needed and for which use-cases can be beneficially applied. This is discussed below.

What is the purpose of the Redundant Steering Mode (RSM)?

1)	The main purpose of RSM is to duplicate data packets on two (hopefully uncorrelated) access links in order to reduce both RTT and PLR, i.e., to improve both reliability and latency.

2)	This can be beneficial e.g., in scenarios where the PLR between the UE and UPF should be minimized without using retransmissions (e.g., when UDP transport is applied).

3)	In addition, it can be beneficial in scenarios where the RTT between the UE and UPF should be minimized for a given 5QI.

-	Note that minimizing the RTT between the UE and UPF using the Smallest-Delay steering mode is different from minimizing the RTT between the UE and UPF using RSM. The Smallest-Delay cannot reduce the RTT; it only selects the access that has the smallest RTT in a certain period. With RSM, however, we can reduce the effective RTT because, for every transmitted packet, the receiver selects the one that arrives first.

4)	The RSM exhibits statistical benefits which are more evident when the distance between the average RTT/PLR on the two accesses is small. The longer the distance, the smaller the benefits of RSM.

Based on the above, we believe that RSM can be beneficially applied for an SDF:

-	When the SDF should experience reduced RTT/PLR as compared with the RTT/PLR of each access; and

-	When the average RTT/PLR on the two accesses is not very different.

As shown in the following figure, when the two accesses have the same RTT (RTT difference = 0), then the traffic duplication can lead to RTT reduction of 28%. For example, when both accesses have 20ms average RTT and 6ms standard deviation (jitter), the duplication can reduce the RTT to 14.25ms (or 28%). The more the RTT difference, the less the RTT reduction compared to the RTT of the best access, hence, the less the benefits of redundant transmission.

Using the above, we propose the following conclusions for KI#3, which adopt several of the concepts introduced in the associated solutions for KI #3 (solutions #3.1 to #3.6).

Start of Changes

8	Conclusions

Editor's note: This clause enlists the conclusions of the study, including agreements for work to be done in the normative phase.

8.x	Conclusions for KI #3

A Redundant Steering Mode (RSM) shall be defined in the normative phase, which supports the following features:

1)	The PCF decides the SDF(s) for which RSM should be applied based on its own criteria and/or based on information received from AFs.

2)	Duplication criteria may be provided by PCF for an SDF using a non-GBR QoS flow. Duplication criteria shall not be provided for SDFs using GBR QoS flows.

-	The duplication criteria indicate when the UE/UPF shall duplicate the traffic of the SDF on both accesses. The purpose of the duplication criteria is to duplicate traffic on both accesses only when it is not possible to fulfil the RTT/PLR requirements of the SDF using one access only.

3)	When duplication criteria are not provided for an SDF, then the UE/UPF shall always duplicate the traffic of the SDF on both accesses.

4)	When duplication criteria are provided for an SDF, then:

-	When the duplication criteria are fulfilled on both accesses, the UE/UPF shall duplicate the traffic of the SDF on both accesses.

-	When the duplication criteria are fulfilled on one access only, the UE/UPF shall send the traffic of the SDF over the other access.

-	When the duplication criteria are not fulfilled on both accesses, the UE/UPF shall send the traffic of the SDF over the "primary" access. The "primary" access may be selected by PCF or may be selected by UE/UPF based on their own implementation. If the measurements are not available to evaluate the duplication criteria for an access, it is assumed that the duplication criteria are not fulfilled on this access.

NOTE:	For example, the duplication criterion "Max PLR = 0.1%" is fulfilled on one access when the measured PLR on this access exceeds the 0.1% threshold.

5)	The duplication criteria contain threshold values for Packet Loss Rate (PLR) and/or Round-Trip Time (RTT).

Editor’s note: The criteria for initiating traffic duplication (i.e., what thresholds can be used) need further study. For example, if the duplication criterion is "Max RTT = 200ms" and the measured RTT is 250ms and 2s on 3GPP access and non-3GPP access respectively, activating traffic duplication is not expected to have a big impact on the aggregated RTT given that the RTT on the two accesses is significantly different (250ms vs 2s).

6)	The existing structure of ATSSS/N4 rules shall be re-used with enhancements for supporting RSM. An example of such enhancements in an ATSSS rule are shown below.

  - Traffic descriptor

    - Application identity: com.example.app1

    - Protocol: TCP

  - Access Selection descriptor

    - Steering functionality: MP-TCP

    - Steering mode: Redundant

    - Steering Mode Information: Primary access=3GPP

    - Threshold Values: Max PLR = 0.1%

This ATSSS rule indicates that:

-	Redundant traffic steering shall apply to the TCP traffic of application "com.example.app1".

-	If the measured PLR exceeds 0.1% on both accesses (duplication criteria fulfilled on both accesses), then all matched traffic shall be duplicated on both accesses.

-	If the measured PLR does not exceed 0.1% on both accesses, then all matched traffic shall be sent over 3GPP access only (as the primary access). If the measured PLR does not exceed 0.1% on one access only (either 3GPP or non-3GPP), then all matched traffic shall be sent over this access.

7)	The redundant traffic steering shall be applicable to both GBR and non-GBR traffic. For GBR traffic, the SMF shall provide the GBR QoS profile to both accesses.

8)	The AF shall be able (e.g., using the Nnef_AFsessionWithQoS API) to indicate the desired packet delay and/or packet loss rate for a data flow. This information shall be considered by PCF when deciding the steering mode for this data flow and may influence when traffic duplication is activated or deactivated.

Editor’s note: Additional bullets may be added before the completion of this study. It should also be defined how the steering functionality handles the duplicated packets.

End of Changes