3GPP TSG-SA2 Meeting #152-e	S2-2206800r02

# 17 – 26 August, 2022, Elbonia													revision of S2-220xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | nnnn | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Storing UPF ID in UDM enabling routing of traffic via localized IMS media plane functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_eIMS5G2 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Routing of IMS traffic via localized IMS media plane functions is one agreed objective of the TEI18_eIMS5G2 WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Option1: UDM stores UPF identity parameter per UE to enable localized routing. IMS nodes can use UPF identity to select media plane functions close to the UPF. Option 2: Conveying UPF service area to IMS nodes via PCF N5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Routing of IMS traffic via localized IMS media plane functions is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR xxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Begin of changes * * * *

Annex XX (Informative):
Routing of IMS traffic via localized IMS media plane functions

# XX.1	General

The 5GC enables traffic to be routed via localized UPF (close to RAN and UE). The optional enhancements in the present Annex enable a selection of localized IMS media plane functions in proximity to such a localized UPF. An UDM/HSS based solution and a PCC based solution are provided as alternatives.

# XX.2	Option 1: UPF Identity stored in UDM/HSS

During the establishment of a PDU session with a UE for an DNN used by the IMS, the SMF selects an UPF and stores the identity of that UPF (e.g., UPF FQDN) in the UDM for the UE. The UDM stores the UPF identity (e.g., UPF FQDN) in the UDR as part of the SMF registration.

The HSS retrieves the UPF identity related to the IMS PDU session from the UDR and forwards it to the S-CSCF via Cx or Nhss-ims when the S-CSCF registers for the UE. During mobile originating call setup, the S-CSCF includes the UPF identity in the call setup requests. An upstream BGCF may use the UPF identity to select an MGCF in proximity to the UPF based on locally configured data. The MGCF may use the received UPF identity to select an IM-MGW in proximity to the UPF based on locally configured data.

If an IMS-AS handles a MO call setup request from the UE or a MT call setup request towards the UE requires selection of an MRF close to the UPF for that UE, it retrieves the UPF identity via HSS from the UDR, uses this information for the MRFC selection, and includes the UPF identity in the related call setup request. The MRFC may use the received UPF identity to select an MRFP in proximity to the UPF based on locally configured data.

# XX.3	Option 2: Conveying UPF service area to IMS nodes via PCF N5

This solution proposes to convey the UPF service area for the IMS PDU session from SMF to PCF and on to P-CSCF during multimedia session establishment. The PCF either stores the UPF service area as part of the SM policy or fetches the UPF service area from the SMF when requested by P-CSCF. PCF may also subscribe to a new event "UPF service area change" upon establishment of the SM policy association.

When the P-CSCF receives a multimedia session request, it fetches the UPF service area (and possibly network provided location information, see Annex B.8 in 3GPP TS 29.514 [96]) and selects (based on a locally configured mapping from UPF service area) the optimal MGW (e.g. ATGW) for the actual UPF service area.

The P-CSCF forwards the UPF service area retrieved from PCF in the SIP request (e.g. as part of the PANI header). Other IMS functions (e.g. MGCF, IMS-AS) may use the information to select, e.g. optimal IM-MGW, MRF hence ensuring an e2e UPF optimization.

Figure XX.3-1 illustrates how IMS obtains the UPF service area at multimedia session establishment.

Figure XX.3-1: IMS obtains the UPF service area

1.	P-CSCF receives an initial session request.

2.	Based on local policy, P-CSCF requests the current UPF service area (possibly with NPLI) via Npcf_PolicyAuthorization_Create.

3.	Either UPF service area is available at PCF (e.g. SMF included the information based on local policy at Npcf_SMPolicyControl_Create) or PCF requests the information from SMF (Npcf_SMPolicyControl_UpdateNotify).

4.	PCF returns the UPF service area (possibly with NPLI, if requested previously).

5.	Based on current UPF service area, P-CSCF selects an optimal MGW/ATGW, using local configuration.

6.	P-CSCF invokes Npcf_PolicyAuthorization_Update service operation to provide the IP address used for the session on the selected MGW/ATGW.

7.	PCF responds to the Npcf_PolicyAuthorization_Update request.

8.	Before forwarding the SIP request, P-CSCF includes the UPF service area (e.g. in SIP PANI header) so that IMS nodes (e.g. IMS-AS) may use it to assist a media plane function (e.g. MRF).

* * * End of changes * * * *